3GPP TSG-RAN WG4 Meeting #116 	R4-2511292

Bengaluru, India, August 25th – 29th, 2025

Source: 	Huawei, HiSilicon

Title: 	OTA test case reduction

Agenda Item:	7.9.4.2.1

Document for:	Approval

1 Introduction

New WID on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at RAN#103 meeting. The work item concentrates on 2 work areas where the current BS specification may be optimised, i.e. Expected EIRP mask for upper 6GHz and OTA test enhancement. One of major identified RF requirements with the aim to shorten the test time is TX IM.

The considerations on test case reduction for TX IM test reduction was discussed in previous meetings and the latest WF is captured in [6]. In this contribution we provide further discussion and proposals for the objective.

2 Test scope reduction for Tx requirement

At RAN4#115 meeting, for TX test case reduction it is agreed to apply 3xCBW offset for spurious emission and there are still 3 options for OBUE [6].

Issue 1-1-1: test scope reduction for OBUE

- Agreement:

  - Further discuss how to reduce the test simplification for OBUE limits.

    - Option 1: 1xCBW offset

    - Option 2: 3xCBW offset

    - Option 3: 3xCBW offset if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands and 1xCBW and 2xCBW if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE

Issue 1-1-2: test scope reduction for spurious emission

- Agreement:

    - Option 1: 3xCBW offset

2.1	Interferer offsets

6 interferer offsets are tested for each and 12 if AAS supports NC operation, these offsets date back to the UTRA requirements and have been carried through for each subsequent RAT however not they are generating such excessive test time so it is appropriate to see if they are all still necessary or can be reduced.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

At previous meetings, the applicable interference freq offset for ACLR and spurious is agreed, and it is FFS for OBUE.

OBUE

LTE/NR OBUE is generally considered that in the 1st adjacent channel the ALCR requirement dominates and as the emission moves away from the carrier the OBUE is tougher. They have a tougher requirement beyond 10 MHz so considering 5MHz channels the 3x CBW offset. As we are testing at M only, the band filter can be ignored.

The above conclusion applies for most bands when interfering signal is 5 MHz CBW. The channel bandwidth of the interfering signal is defined as the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104. For some bands, e.g. n77, n78 and n79, the minimum BS CBW is 10 MHz. In this case 10 MHz interferer adjacent to carrier is most stringent.

Proposal 1: it is proposed to use 3xCBW offset to check OBUE limits when the smallest CBW of the band is less than or equal to 5 MHz, otherwise 1xCBW offset.

3 Out of band blocking

For RX test case reduction, it is agreed to focus on out of band blocking and focus on the measurement step size for Rx OOBB requirement.

Issue 1-2-1: Out of band blocking

- Agreement on step size of OOBB requirement for further discussion:

    - Option 1: 1/3 of the minimum supported bandwidth

    - Option 2:  Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

    - Option 3: mixed EMC and option 1 approach, FFS on the details.

    - Option 4: To use 2/3 of the test signal configuration channel bandwidths

    - Option 5: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz.

    - Other options are not precluded

RAN4 had similar discussion for FR2 in Rel-15 and finally agreed to reduce the amount of time by increasing the interferer step size. The less than 1/3 BW approach is adopted when define the measurement step for FR2-1.

Table 7.6.4.2.3-1: Interferer signal step size for FR2-1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

For FR1 similar approach can be considered, e.g. option 1. We propose to set the measurement step size according to the supported BS channel bandwidth declared by the manufacture.

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | ≤5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

For option 2 is may be feasible for some BS receiver architectures but it will miss some frequency points which may cause worse blocking cases for some implementations, e.g. superheterodyne receiver.

Proposal 2: It is proposed to set the measurement step size according to the minimum supported BS channel bandwidth declared by the manufacture, as shown in Table 3-1.

4	Summary

Proposal 1: it is proposed to use 3xCBW offset to check OBUE limits when the smallest CBW of the band is less than or equal to 5 MHz, otherwise 1xCBW offset.

Proposal 2: It is proposed to set the measurement step size according to the minimum supported BS channel bandwidth declared by the manufacture, as shown in Table 3-1

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

3	Reference

[1] R4-2419243, Test case reduction for TX IM, Huawei

[2] R4-2419828, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[3] R4-2501854, OTA test case reduction, Huawei

[4] R4-2502375, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[5] R4-2504740, Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP

[6] R4-2508658, Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP