3GPP TSG-RAN WG4 Meeting #116 	R4-2511303

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0299 | rev | - | Current version: | 15.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (AASenh_BS_LTE_UTRA-Core) CR to 37.105: Additional co-location requiremen for BC1 and BC2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The exact frequency offsets for co-location transmitter intermodulation are not clear for AAS test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reffering to 37.141, the frequency offsets are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The exact frequency offsets are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

6.7.2.2	Additional co-location minimum requirement (BC1 and BC2)

The transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in subclause 6.6.6, operating band unwanted emission in subclause 6.6.5 and ACLR in subclause 6.6.3 in the presence of a wanted signal and an interfering signal according to table 6.7.2.2-1 for BS operation in BC2.

The requirement is applicable outside the Base Station RF Bandwidth edges for BC2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For TAB connectors supporting operation in non-contiguous spectrum in BC1 or BC2, the requirement is also applicable inside a sub-block gap with a gap size larger than or equal to two times the interfering signal centre frequency offset. For TAB connectors supporting operation in non-contiguous spectrum in BC1, the requirement is not applicable inside a sub-block gap with a gap size equal to or larger than 5 MHz. The interfering signal offset is defined relative to the sub-block edges.

For TAB connectors supporting operation in multiple operating bands, the requirement applies relative to the Base Station RF Bandwidth edges of a BC2 operating band. The requirement is also applicable for BC1 and BC2 inside an inter Base Station RF Bandwidth gap equal to or larger than two times the interfering signal centre frequency offset. For TAB connectors supporting operation in multiple operating bands, the requirement is not applicable for BC1 band inside an inter Base Station RF Bandwidth gap with a gap size equal to or larger than 5 MHz.

Table 6.7.2.2-1: Interfering signal for
the co-location transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA or NR or UTRA signal |
| Interfering signal type | CW |
| Interfering signal level | Rated total output power per TAB connector in the operating band (PRated,t,TABC) – 30dB |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ± 0.8 MHz ± 2.0 MHz ± 3.2 MHz ± 6.2 MHz |
| NOTE: Interfering signal positions that are partially or completely outside of any downlink operating band of the TAB connector are excluded from the requirement. |  |

<End of changes>