3GPP TSG-RAN WG4 Meeting #116	R4-2511308

# Bangaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.755 | CR | 0003 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.755 on replacing “FC” with “one Rx RF chain”, misspelling, and editorial changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_FR1_DL_Frag_Carrier |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections on replacing “FC” with “one Rx RF chain”, misspelling, and editorial changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on replacing “FC” with “one Rx RF chain”, misspelling, and editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no “FC” mode defined in the TR, therefore, it needs to be replaced with “one Rx RF chain”, otherwise the content is missleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.3. 8, 8.1.2, 8.1.3. 8.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change 1>>

### 7.9.3	Discussion on applicable scenarios when enabling one Rx RF chain mode

The adjusted RF requirements for example bands have also been evaluated by companies. There would be REFSENS degradation due to one Rx RF chain mode on both PCC and SCC in the FDD bands due to self-Tx leakage impact. The network vendors express strong concerns to the degradation on the PCell when one Rx RF chain mode is configured. It was agreed in WF [22] to further discuss on the applicable scenarios. Evaluation results also shows when the channel BW of the CCs plus Wgap are small enough, the REFSENS would not be degraded. If the two CCs are configured as SCCs that may be fallback of higher order CA combination when there’s no scheduled self-band UL transmission, the REFSENS are not degraded also. Sor for TDD bands the situation would be similar that REFSENS of CCs would not be degraded if UE is configured in one Rx RF chain mode. One possible approach to reduce ΔRIBNC is PCC/SCC swapping, the degradation may be largely improved to a few dB form the worst-case configuration. Another use case is when UE is at the location near the gNB, the received level of wanted CCs can be much higher than the REFSENS level while the configured UL transmission power level does not need to be large, PCell REFSENS would also not be degraded due to one Rx RF chain configuration or the degradation can be tolerable.

In summary, to manage the impact of PCell degradation, the following operation scenarios can be considered for intra-band non-contiguous DL CA:

a.) FDD band DL fragmented carriers

i. CBW of both DL fragmented carriers + Wgap are small enough

ii. UE located so near to the gNB that UL Tx power level is not too large

iii. For higher order CA combination, both intra-band DL fragmented carriers are SCC (PCell UL is in another band)

iv. When UE is capable of interference measurement, self-assessment and/or reporting

v. The frequency allocation of PCC and SCC to avoid worst case configuration on DL performance i.e., PCC/SCC assignment

b.) TDD band DL fragmented carriers

# 8	Study of means for a UE to inform the network of appropriate CA configuration it can support

It has been agreed to consider the following aspects for how to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers [22]:

Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

Alternative 1 has no modification of CA configurations in RAN4

Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

Alternative 1a has no modification of CA configurations in RAN4

Alternative 2: use the new notation for CA configurations with single Rx RF Chain

Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

RAN4 also agreed to clarify CA fallback and architecture switching as following aspects:

CA fallback with/without UE architecture switching

Consider the fall back

from high order CA to lower order CA

from CA to single carrier mode

Decided by network

UE architecture switching:

Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

When discussing UE behaviours, it is also agreed to consider triggering condition to enable the “one Rx RF chain” mode as following aspects:

Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

Note: detailed conditions to enable/disable can be further discussed and decided separately

## 8.1	Indication of one Rx RF chain mode for DL fragmented CA

### 8.1.1	Indication of fragmented CA alternative 1: Legacy band combination reporting scheme + Rx sharing capability indication

For Alt.1, it has no modification of CA configurations in RAN4. The UE would report its supported CA configuration with corresponding UE capability indicating whether Rx sharing capability is needed to support the CA configuration in RAN2 signaling. (The granularity of the capability is FFS and could be decided in the WI stage)

For example, if the UE supports CA_n7(2A)-n25(2A) with 1 bit indicating Rx sharing capability and supports CA_n7A-n25(2A) without, UE will report CA_n7(2A)-n25(2A) with the bit set to 1 and CA_n7A-n25(2A) with the bit set to 0. Such UE capability reporting is done when the UE camps on a cell. [38]

Any backwards compatibility issues with a network which does not understand the capability for supporting CA with “one Rx RF chain” would have to be analysed during the WI phase.

### 8.1.2	Indication of fragmented CA alternative 1a: Legacy band combination reporting scheme + Rx sharing capability indication and other related information including the frequency separation between two carriers

In this section, two alternative solutions are proposed based on the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

The first solution is based on the goal of supporting higher order CA configuration. In this approach, the UE is not able to support a specific high order CA configuration including an intra-band non-contiguous component under non-one Rx RF chain mode, but can support this specific high order CA configuration with one Rx RF chain mode in the intra-band component with frequency separation restriction between the two non-contiguous carriers.

An example scenario is provided below.

- one Rx RF chain mode: CA_nXA-nYA-nZA-nPA-nQ(2A)

- non- one Rx RF chain mode: CA_nXA-nYA-nZA-nPA-nQA

In the above scenario, the UE can only support the combination CA_nXA-nYA-nZA-nPA-nQ(2A) under the one Rx RF chain mode for nQ(2A) due to the limitation on the Rx chains, and in order to support the one Rx RF chain mode in band nQ, the UE may have a limitation on the maximum frequency separation restriction between the two non-contiguous DL carriers in band nQ. This restriction should be considered and managed by the network.

In this solution, the UE can still leverage the legacy band combination reporting scheme to report the supported combinations in the one Rx RF chain mode. However, additional signalling is required to indicate the UE's Rx sharing capability, along with other related information such as the frequency separation between the two carriers.

The UE might also report the suggested condition to enable the one Rx RF chain mode such as receive signal strength of the 2CCs and/or the in-gap interference limit between the two non-contiguous intra-band DL carriers.

Then, the second solution is to enable support for more downlink MIMO layers in fragmented CA mode. In this approach, the UE might support one Rx RF chain mode and non- one Rx RF chain mode for a specific intra-band CA NC configuration, and the total maximum MIMO layers supported in the FC mode can be higher than in the non- one Rx RF chain mode.

An example scenario is provided below.

- one Rx RF chain mode: the UE supports up to DL 4x4 MIMO layers in both CCs for CA_nX(2A) in the band X.

- non- one Rx RF chain mode: the UE supports up to DL 4x4 MIMO layers in one CC but can only support up to DL 2x2 MIMO layers in another CC for CA_nX(2A).

- The network is capable of controlling the activation/de-activation of the one Rx RF chain mode.

The scenario on enabling this feature is provided as below.

Figure 8.1.2-1: Possible switching scenario between “one Rx RF chain” and “non- one Rx RF chain” targeting more DL MIMO layer support

In the above scenario, when the 2CC of the intra-band DL CA are already configured, the network will control and might enable the UE switching to the one Rx RF chain mode when the signal strength of both CCs are already strong enough, and the in-gap interference is limited. Higher total DL MIMO layer transmission is possible when switching to the one Rx RF chain mode. And when the network controls the UE switch to the non- one Rx RF chain mode, the 2CCs might still remain configured. Similar to the first solution, the UE may have a limitation on the maximum frequency separation between the two non-contiguous downlink carriers when operating in one Rx RF chain mode. This restriction should be considered and managed by the network.

Then in this solution, the UE will report the same CA configuration (intra-band NR CA configuration or higher order inter-band CA configuration with intra-band CA components.) with different maximum DL MIMO layer support per CC, the configuration with higher total maximum DL MIMO layer support will include an additional indication in the capability signalling in order to indicate it is supported under the “one Rx RF chain mode”. Based on this approach, the UE can still leverage the legacy band combination reporting scheme to report the supported combinations in the one Rx RF chain mode. However, additional signalling is required to indicate the UE's Rx sharing capability, along with other related information such as the frequency separation between the two carriers.

The UE might also report the suggested condition to enable the one Rx RF chain mode such as receive signal strength of the 2CCs and/or the in-gap interference limit between the two non-contiguous intra-band DL carriers.

### 8.1.3	Indication of fragmented CA alternative 2: New notation for CA configurations with single Rx RF Chain

Regarding how to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers, a new notation FnA could be used to represent there are n NC CCs in one band sharing one RX RF chain. For example: When there are 2 NC CCs of band nX receiving in the separate mode (e.g., fully separate mode or partially shared mode), the CA combination type is CA(2A). When the receiving state switches to one Rx RF chain mode, the CA combination type should change to CA(F2A), F represents there are two NC CCs in the one Rx RF chain mode and in the sharing chain.

Similarly, CA_nX(F2A-A) means there are 3 NC CCs and two of them are in the sharing chain (one Rx RF chain mode), CA_nX(F2A-F2A) means there are 4 NC CCs and two of them are in one sharing chain, the other two are in another sharing chain.

Any backwards compatibility issues with a network which does not understand the capability for supporting CA with “one Rx RF chain” would have to be analysed during the WI phase.

## <Omit changes>

## <<End of Change 1>>

## <<Start of Change 2>>

#### 8.1.4.3	Configuration of nXA”+nXA”+nYA+nZA with single Rx chains and with modified MIMO capability

Furthermore, an additional configuration in Figure 8.1.4.3-1 illustrates that reducing the number of supported layers in one of the bands allows for reducing the number of Rx chains. The Rx chain that are freed-up can then be reallocated to support more CCs. In the example below the fragmented carriers with a single Rx chain are configured only in SCells to avoid PCell degradation.

Figure 8.1.4.3-1. Example configuration with PCell (dark grey) and SCell (light grey): nXA” + nXA” + nYA + nZA.

As illustrated by the examples above, all supported band combinations, regardless of the UE implementation, must be explicitly included in the UE capability signaling. The UE cannot inform the NW that it has a spare Rx chain and expects the NW’s action based on this information without corresponding capability reporting. The network can not only use RF requirements to configure, release, activate, or deactivate carriers, and the network’s action is based on corresponding capability reporting.

To facilitate this, introducing a new BW class capability for the fragmented carriers within a band with restricted frequency separations and MRTD (to ensure collocated scenarios) could indicate the support for such fragmented carriers with a single Rx chain.

The example frequency separation class table is shown below. The capabilities of the corresponding band entry (i.e. feature set) would then be conditioned on a maximum frequency separation of the carriers.

Table 8.1.4-1: NR intra-band non-contiguous DL CA frequency separation classes with single Rx chain mode

| NR NC DL CA frequency separation class | Maximum allowed frequency separation | Maximum allowed number of CCs |
| --- | --- | --- |
| I | 100 MHz | 2 |
| II | 200MHz | 2 |
| III | 200 MHz | 4 |

Another example is to use the currently undefined NR CA bandwidth class (BCS) ‘F’ for indicating a CA configuration with fragmented carrier operation.

Table 8.1.4-2: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the table above, where the newly added parts are highlighted with bold font, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two fragments are shown in Fig 8.1.4.3-2.

Figure  8.1.4.3-2: Example of CA_n25(2F)

## <<End of Change 2>>