3GPP TSG-RAN WG4 Meeting #116									                         R4- 2511314

Bangaluru, India, 25th – 29th Aug. 2025

Agenda item:	7.21.2.2

Source:	Ericsson

Title:	Simulation and discussion on SBFD UE-UE CLI

Document for:	Discussion

# 1 Introduction

In the Rel-18 study, for the scenario of Urban Hotspot->Urban Hotspot (denoted as Scenarios2 in the below context), DL throughput degradation was observed at the cell edge due to the inter-UE CLI for different grid shifts with different BS Tx power. As suggested in [1], in this paper, a detailed analysis for the scenario with intra-operator adjacent-channel inter-cell will be provided using Scenario 2 as reference.

# 2 Discussion

In this contribution we follow the RAN4 coexistence evaluation methodology applied to the 4 coexistence cases identified for study during Release 18. The study focuses on Case 1 where the aggressor is UL of the SBFD network, and the victim is the DL of the NR TDD network. The scope is to identify the impact of the UE-UE CLI (Cross Link Interference) generated by the UL of SBFD to the DL of NR TDD. Once identified this impact, the study aims at reducing the maximum UE transmission power from the baseline of 23 dBm, for the SBFD-aware UEs, in order to reduce the impact of the UE-UE CLI.

Coexistence Case 1 is characterized by a “baseline ACIR”, already identified in Release 18 TR 38.858 and calculated below and based on a “flat” assumption for SBFD UE ACLR and TDD UE ACS. Based on offline discussions, a stepwise model for ACLR and ACS is considered, based on TR 36.942, to take into account the frequency separation that may exist between the two channels if SBFD is configured e.g. D-U-D. As a result, different resulting ACIR are considered in the study presented in this contribution, as they have been suggested in the offline discussions with other companies.

Table 2-1: FR1 Baseline ACIR and additional tested ACIR

| Coexistence Case | ACIR of interest | ACIR | ACLR | ACS |
| --- | --- | --- | --- | --- |
| 1 | ACIR_UE_UE (baseline) | 28.2 dB | ACLR UE SBFD = 30 dB | ACS UE TDD = 33 dB |
|  | ACIR_UE_UE | 28.4 dB | ACLR UE SBFD = 30 dB | ACS UE TDD = 33.6 dB |
|  | ACIR_UE_UE | 32.6 | ACLR UE SBFD = 43 dB | ACS UE TDD = 33 dB |
|  | ACIR_UE_UE | 33.1 | ACLR UE SBFD = 43 dB | ACS UE TDD = 33.6 dB |

The simulation scope is focused on scenario 2 defined in TR38.858 during Release 18, focusing on the Urban Hotspot scenario where 80% of users are clustered in indoor hotzones and where the minimum distance between UEs is 1 m. Notice that for the results presented in this paper, the 80% of users are clustered in indoor hotzones that are randomly located in the coverage scenario. The number of clusters is equal to the number of cells, so that on average the deployment covers one hotzone per cell. The two networks share the hotzones, so that in each hotzone, users from different networks can be located close to each other. It is also worth mentioning that this hotzones deployment is slightly different from the one which hasrecently been discussed offline, and which allocates exactly one hotzone per cell. In this case, when the Gridshift increases, the overlapping zone of the two networks reduces and the aggressor UE gets closer to the aggressor Base Station, reducing the UE aggressor transmission power and consequently the UE-UE CLI (cross link interference), thus making Gridshift 100% a best-case scenario.

In Scenario2, already during Release 18 it was identified the existence of a UE-UE CLI coexistence issue generating degradation above 5% on the 5%tile throughput of DL TDD. The degradation of the mean user throughput was instead considered as acceptable in the conclusions from the Study Item.

The study presented in this contribution considers two Gridshift (GS) values, the baseline 100% considered in Release 18 and GS 5%. Other values of Gridshift can be considered too, but we focused on these two as a priority to observe the trends of the extreme cases.

Other important simulation assumptions are:

- TRP=49 dBm,

- the antenna configuration for SBFD is 2, i.e. the same antenna gain is maintained compared to the TDD one (i.e. SBFD config 2 (baseline): (Mg,Ng,M,N,P) = (1,1,8,8,2)),

- and in our results, we include the blocking model agreed during Release 18 for the UL receiver.

The Annex reports the details on the simulation assumptions, based on TR38.858.

## 2.1 Gridshift 100%

### 2.1.1 Scenario 2 (Urban Hotspot), ACIR= 28.2

Figure 2.1.1-1 shows the Mean and 5%tile user throughput of the DL TDD for different Adjacent Channel Interference (ACI) conditions. Comparing the blue and green bars, it can be observed that the 5%tile DL throughput degradation when SBFD coexists with TDD, compared to the case that TDD coexists with another TDD network, which reveals the impact of the UE-UE CLI, is around 31.4%, for the baseline case that the UE maximum transmission power is 23 dBm.

The mean user throughput degradation for the same comparison is around 5%, so that no significant degradation is observed for the mean user throughout.

In the plots we can also observe the benefit of reducing the maximum UE transmission power down to a value of 15 dBm. This reduction helps reduce the degradation up to 13%, so that the issue is still not solved, even if the maximum UE power is reduced by 8 dB.

Figure 2.1.1-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 28.2 dB and GS 100%).

- For Scenario 2/Case 1, with GS 100%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network, imposes a 31.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected.

- For Scenario 2/Case 1, with GS 100%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 13%.

### 2.1.2 Scenario 2 (Urban Hotspot), ACIR= 28.4

This case is very similar to previous results shown in Section 2.1.1, since the ACIR is not significantly increased when increasing the ACS to 33.6 dB. Observations are the same as discussed in previous section.

Figure 2.1.2-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 28.4 dB and GS 100%).

### 2.1.3 Scenario 2 (Urban Hotspot), ACIR= 32.6

When the ACLR is increased to 43 dB, the degradation is decreased.

Figure 2.1.3-1 shows the Mean and 5%tile user throughput of the DL TDD for different Adjacent Channel Interference (ACI) conditions. Comparing the blue and green bars, it can be observed that the 5%tile DL throughput degradation when SBFD coexists with TDD, compared to the case that TDD coexists with another TDD network, which reveals the impact of the UE-UE CLI, is around 19.6%, for the baseline case that the UE maximum transmission power is 23 dBm. This degradation is significantly lower than the one shown in sections 2.1.1 and 2.1.2.

The mean user throughput degradation for the same comparison is below 5%, so that no significant degradation is observed for the mean user throughout.

In the plots we can also observe the benefit of reducing the maximum UE transmission power up to a value of 15 dBm. This reduction helps reduce the degradation up to 9%, so that the issue is still not solved, even if the maximum UE power is reduced by 8 dBm, and the ACLR of SBFD is 43 dB in this case.

Figure 2.1.3-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 32.6 dB and GS 100%).

### 2.1.4 Scenario 2 (Urban Hotspot), ACIR= 33.1 dB

This case is very similar to previous results shown in Section 2.1.3, since the ACIR is not significantly increased when increasing the ACS to 33.6 dB. Observations are the same as discussed in previous section. The mean user throughput does not show degradation, while the 5%tile user throughput shows a degradation of 19.1%, which can be reduced up to 9% if the UE maximum transmission power of SBFD-aware UEs is reduced by 8 dBm. This results in 40% mean user throughput degradation of UL SBFD compared to an UL TDD deployed in the same scenario.

Figure 2.1.4-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 33.1 dB and GS 100%).

- For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant at approximately 19%, depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by up to 8 dB further mitigates the degradation to around 9%.

## 2.2 Gridshift 5%

### 2.2.1 Scenario 2 (Urban Hotspot), ACIR= 28.2

Figure 2.2.1-1 shows the Mean and 5%tile user throughput of the DL TDD for different Adjacent Channel Interference (ACI) conditions. Comparing the blue and green bars, it can be observed that the 5%tile DL throughput degradation when SBFD coexists with TDD, compared to the case that TDD coexists with another TDD network, which reveals the impact of the UE-UE CLI, is around 21.4%, for the baseline case that the UE maximum transmission power is 23 dBm.

This observed degradation is lower than the one we observed in Section 2.1.1 for the same baseline ACIR (31.4%).  The reason is that GS 100% is a worst-case scenario for the DL, so that the DL is weaker than the case GS is 5%. This makes the TDD DL when GS is 5% less sensitive to the additional interference generated by UE-to-UE CLI than the case GS 100%.

The mean user throughput degradation for the same comparison is around 5%, so that no significant degradation is observed for the mean user throughout.

In the plots we can also observe the benefit of reducing the maximum UE transmission power down to a value of 15 dBm. This reduction helps reduce the degradation up to 10%, so that the issue is still not solved, even if the maximum UE power is reduced by 8 dB.

Figure 2.2.1-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 28.2 dB and GS 5%).

- For Scenario 2/Case 1, with GS 5%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network imposes a 21.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected.

- For Scenario 2/Case 1, with GS 5%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 10%.

- For Scenario 2/Case 1, with GS 5%, TDD network’s DL 5%tile throughput experiences less degradation compared to GS 100% due to UE-to-UE CLI the impact. The reason is that GS 100% represents a worst-case scenario for DL, so that the DL of TDD results weaker and more sensitive to the UE-to-UE CLI when GS is 100%.

### 2.2.2 Scenario 2 (Urban Hotspot), ACIR= 28.4

This case is very similar to previous results shown in Section 2.1.1, since the ACIR is not significantly increased when increasing the ACS to 33.6 dB. Observations are the same as discussed in previous section.

Figure 2.2.2-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 28.4 dB and GS 5%).

### 2.2.3 Scenario 2 (Urban Hotspot), ACIR= 32.6 dB

When the ACLR is increased to 43 dB, the degradation is decreased.

Figure 2.2.3-1 shows the Mean and 5%tile user throughput of the DL TDD for different Adjacent Channel Interference (ACI) conditions. Comparing the blue and green bars, it can be observed that the 5%tile DL throughput degradation when SBFD coexists with TDD, compared to the case that TDD coexists with another TDD network, which reveals the impact of the UE-UE CLI, is around 15.6%, for the baseline case that the UE maximum transmission power is 23 dBm. This degradation is significantly lower than the one shown in sections 2.2.1 and 2.2.2.

The mean user throughput degradation for the same comparison is below 5%, so that no significant degradation is observed for the mean user throughout.

In the plots we can also observe the benefit of reducing the maximum UE transmission power down to a value of 15 dBm. This reduction helps reduce the degradation up to 7%, so that the issue is still not solved, even if the maximum UE power is reduced by 8 dB, and the ACLR of SBFD is 43 dB in this case.

Figure 2.2.3-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 32.6 dB and GS 5%).

### 2.2.4 Scenario 2 (Urban Hotspot), ACIR= 33.1 dB

This case is very similar to previous results shown in Section 2.2.3, since the ACIR is not significantly increased when increasing the ACS to 33.6 dB. Observations are the same as discussed in previous section. The mean user throughput does not show degradation, while the 5%tile user throughput shows a degradation of 13.1%, which can be reduced up to 7% if the UE maximum transmission power of SBFD-aware UEs is reduced by 8 dB.

Figure 2.2.4-1 DL TDD Mean User Throughput and 5%tile user throughput (ACIR 33.1 dB and GS 5%).

- For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant ranging from 13.1% to 15.6% depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by 8 dB further mitigates the degradation to approximately 7%.

## 2.3 Simulation result overview

The simulation results are formatted in agreed template.

# 3 Conclusion

In the previous sections, we made the following observations:

Observation 1	For Scenario 2/Case 1, with GS 100%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network, imposes a 31.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected.

Observation 2	For Scenario 2/Case 1, with GS 100%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 13%.

Observation 3	For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant at approximately 19%, depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by up to 8 dB further mitigates the degradation to around 9%.

Observation 4	For Scenario 2/Case 1, with GS 5%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network imposes a 21.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected.

Observation 5	For Scenario 2/Case 1, with GS 5%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 10%.

Observation 6	For Scenario 2/Case 1, with GS 5%, TDD network’s DL 5%tile throughput experiences less degradation compared to GS 100% due to UE-to-UE CLI the impact. The reason is that GS 100% represents a worst-case scenario for DL, so that the DL of TDD results weaker and more sensitive to the UE-to-UE CLI when GS is 100%.

Observation 7	For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant ranging from 13.1% to 15.6% depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by 8 dB further mitigates the degradation to approximately 7%.

# 4 References

- R4-2508708, Way Forward on UE-UE CLI problem statement and potential solutions, Samsung.

# 5 Annex

In Table 5-1 lists parameters considered for simulation campaign relevant for urban hotspot scenarios, FR1.

Table 5-1: Urban Macro/Urban Hotspot scenario FR1

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | Static TDD (DDDDU), SBFD (XXXXX) |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 100% SBFD slots |
|  | Channel bandwidth | 100 MHz for STDD 80:20 MHz (DU) for SBFD |
|  | Available resource blocks | 273 for STDD 218:55 (DU) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 1 active users in UL or DL per cell at a time |
|  | Channel model | gNB-UE: UMa TR 38.803/38.828 gNB-gNB: UMa TR 38.803/38.828 UE-UE: UMi TR 38.803/38.828 (for d>10 m), FSPL (for d<10m) |
|  | UE to BS min 2D distance | 35 m |
|  | Grid-shift | 100% |
| BS | (Mg, Ng, M, N, P) | (1,1,8,8,2) (same antenna gain) |
|  | Sub-array configuration | 1x1 |
|  | gNB Tx Power | 49 dBm (same antenna gain) |
|  | (dv, dh) | (0.5λ, 0.8λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element | TR 38.803 |
|  | Subarray electrical down-tilt | N/A |
|  | Mechanical down-tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Link level model | As per TR 38.803 |
|  | BS height | 25 m |
|  | Noise figure/blocking model | Sub-band filters. As per TR 38.858 |
|  | Panel HW assumptions | Same antenna gain (antenna config 2) |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm and reduced with 2dBm step up to 15 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 15 dB |
|  | Noise figure | 9 dB |
|  | Link level model | As per TR 38.803 |
|  | UE distribution outdoor/indoor | 20: 80 for hotzone distribution |
|  | Clusters | Circular zones with radius 25 m, 1 cluster per cell. Minimum distance between UEs is 1 m |