3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2511320

# Bengaluru, India, 25th – 29th August, 2025

Title: 	MPR and antenna isolation evaluation for 4Tx PC1

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.6.3.2.1	Tx requirements	[NR_IoT_NTN_req_test_enh-Core]

Document for:	Approval

# 1	Background

In RAN#115, a way forward [2] captured the open topics for conclusion of the work item in August. It was observed that PC1 HPUE NR-NTN still needed significant work. In this contribution, we provide a limited set of MPR evaluations for 4Tx PC1 based on 2Tx measurements. Based on our findings, we propose a way forward on HPUE for R19 and future work.

# Discussion

## 2.1 Way forward input on 4Tx PC1

The way forward [2] lists the open items for NR-NTN HPUE MPR and A-MPR. The part related to 4Tx PC1 is copied below in italics.

Issue 1-6: PC1 ACLR: Agreement:PC1 ACLR = 34dBc.

Issue 1-8 ~ Issue 1-9 PC1 MPR related: Agreement:

- Interested companies are encouraged to work on two tables with tentative values for PC1 MPR, one for 1Tx and one for 4Tx

  - Further check these tentative values, especially for the edge RB results by considering the working assumption of L_CRB as [8]RB.

  - With the understanding the [4Tx] architecture for PC1 is not agreed yet.

    - If 4Tx would be agreed, it can be further discussed if we can merge two tables then.

Table 1. PC1 MPR [with 1Tx]

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | [Note 1] | ≤ [0.5] | ≤ [0] |
|  | QPSK | [Note 1] | ≤ [1] | ≤ [0] |
|  | 16 QAM | [Note 1] | ≤ [2] | ≤ [1] |
|  | 64 QAM | [Note 1] | ≤ [2.5] | ≤ [2.5] |
| CP-OFDM | QPSK | [Note 1] | ≤ [3] | ≤ [1.5] |
|  | 16 QAM | [Note 1] | ≤ [3] | ≤ [2] |
|  | 64 QAM | [Note 1] | ≤ [3.5] | ≤ [3.5] |
| NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB. |  |  |  |  |

[Table 2. PC1 MPR with 4Tx]

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.5] | ≤ [2.0] | ≤ [0.5] |
|  | QPSK | ≤ [10.0] | ≤ [2.5] | ≤ [0.5] |
|  | 16 QAM | ≤ [10.0] | ≤ [3.5] | ≤ [1.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [4.0] | ≤ [3.5] |
| CP-OFDM | QPSK | ≤ [10.0] | ≤ [4.5] | ≤ [2.0] |
|  | 16 QAM | ≤ [10.0] | ≤ [4.5] | ≤ [2.5] |
|  | 64 QAM | ≤ [10.0] | ≤ [5.0] | ≤ [5.0] |

As observed, except for the 12-tone and 1-tone 15KHz SCS edge and center allocations, most of the allocations have not been evaluated. In this contribution, we provide the MPR and A-MPR evaluations for all the applicable allocations.

## 2.2 Emulating 4Tx performance with 2Tx measurements

We have performed and pioneered 2Tx measurements in 3GPP for years now and we consider our test bench, and data are solid and more accurate than simulations. However, it is not likely that we will develop a 3Tx or 4Tx bench.

Nevertheless, it is feasible to emulate >2Tx in a 2Tx measurement by adjusting the coupling between the two PAs to be representative of more PAs coupling together.

For example, for 3Tx with a 15dB coupling, it is equivalent to measure 2Tx but coupling twice the power into each PA, which is equivalent to reduce the isolation by 3dB. Assuming 4dB post-PA losses and variable antenna isolation the Table below gives the equivalent 2Tx, 3Tx and 4Tx PA to PA coupling.

Table 1: 2Tx PA to PA isolation to emulate different 3Tx and 4Tx antenna isolations

| post-PA losses | Antenna Isolation | 2Tx PA to PA isolation |  |  |
| --- | --- | --- | --- | --- |
|  |  | 2Tx | 3Tx | 4Tx |
| 4.0 | 10.0 | 18.0 | 15.0 | 13.2 |
| 4.0 | 15.0 | 23.0 | 20.0 | 18.2 |
| 4.0 | 20.0 | 28.0 | 25.0 | 23.2 |
| 4.0 | 25.0 | 33.0 | 30.0 | 28.2 |

The values in the last column were used in our 2Tx measurement setup that can adjust the coupling automatically to perform the measurements disclosed in the next chapter.

## 2.3 2Tx measurement results emulating 4Tx

Table 2 and Table 3 provides the RAW measured data for SEM (high/low) and ACLR (high/low) margin CP-OFDM and DFT-s-OFDM respectively with:

- ACLR target of 34dB agreed for PC1 NTN

- SEM target+3 dB to emulate a 4Tx sum based on 2Tx measurement

- 5/10/20/50MHz CBW with 15kHz SCS and 100MHz with 30kHz SCS

- Different equivalent 4Tx antenna isolation levels of 10/15/20/25dB emulated with a 2xPC2 2Tx measurement modelling the equivalent coupling of 3 paths into one.

- QPSK Inner, outer/full and outer/edge allocations

- MPR level where at least one isolation scenario meets requirements (when possible within the measured range) and the 1dB less MPR showing fails

- NR PC2 PAs are calibrated with 31dB ACLR at MPR1 for 20MHz QPSK DFT-s-OFDM 100RB0 with 4dB post PA losses.

Table 2: ACLR and SEM margins for CP-OFDM

Observations on CP-OFDM:

- As expected, the performance improves with higher antenna isolation

- Inner allocations (maximum LCRB) at 1dB MPR versus 31dBm pass at all antenna isolation.

  - ACLR is marginal for 100MHz and 10dB antenna isolation

- Inner allocations (maximum LCRB) at 0dB MPR versus 31dBm fail ACLR and/or SEM at all antenna isolation.

  - Except for 5MHz and antenna isolation >10dB which passes

- 1RB0 Edge allocation at 2dB MPR versus 31dBm pass at 20 and 25dB antenna isolation.

  - ACLR is marginal for 100MHz and 20dB antenna isolation

  - All antenna isolation pass for CBW up to 50MHz

- 1RB0 Edge allocation at 1dB MPR versus 31dBm fail SEM low at all antenna isolation.

  - Except for 5MHz all antenna isolations which passes

- 4RB0 Edge allocation at 3dB MPR versus 31dBm pass at all antenna isolations.

  - SEM is marginal for 20MHz and 10/15dB antenna isolation

- 4RB0 Edge allocation at 2dB MPR versus 31dBm fail SEM low at all antenna isolation.

  - Except for 50 and 100MHz all antenna isolations and 5MHz >10dB isolation which passes

- Full allocation at 3dB MPR versus 31dBm fail ACLR low and high, would probably need 4dB MPR.

Table 3: ACLR and SEM margins for DFT-s-OFDM

Observations on DFT-s-OFDM:

- As expected, the performance improves with higher antenna isolation

- Inner allocations (maximum LCRB) at 0dB MPR versus 31dBm pass at all antenna isolation.

  - ACLR is marginal for 50 and 100MHz and 10/15dB antenna isolation

- Inner allocations (maximum LCRB) at 1dB boosting versus 31dBm fail ACLR and/or SEM at all antenna isolation for 20/50/100MHz. Although there is 4x26dBm, 32dBm cannot be reached.

- 1RB0 Edge allocation at 1dB MPR versus 31dBm is marginal at 25dB antenna isolation for 100MHz.

  - All antenna isolation pass for CBW up to 50MHz

- 1RB0 Edge allocation at 0dB MPR versus 31dBm fail SEM low at all antenna isolation.

  - Except for 5MHz all antenna isolations which passes

- 4RB0 Edge allocation at 2dB MPR versus 31dBm pass at all antenna isolations.

  - SEM is marginal for 20MHz and 10/15dB antenna isolation

- 4RB0 Edge allocation at 1dB MPR versus 31dBm fail SEM low at all antenna isolation for 10 and 20MHz.

- Full allocation at 1dB MPR versus 31dBm fail ACLR and SEM, would probably need 2dB MPR, potentially 3dB at 100MHz.

## 2.4 Preliminary conclusions on MPR

Although our evaluation is limited to only a few allocations, it provides some good understanding on the boundaries for a feasible 4Tx PC1 implementation based on 4 NR PC2 PAs.

We can draw the following conclusions:

- 32dBm cannot be reached with 4x26dBm PAs even with 34dB ACLR.

- 31dBm 0dB PC1 MPR can be reached with the worst case QPSK DFT-s-OFDM inner thanks to a 34dB ACLR requirement. 37dB ACLR requirement would need at least an extra dB MPR.

- Edge/outer allocations require up to 2/3dB MPR up to 4RB0 for DFT-s-OFDM/CPOFDM, further evaluation will be needed to check beyond 4RB edge allocations

- Full/outer will require 2.5/4dB MPR for DFT-s-OFDM/CP-OFDM

- To achieve reasonable performance, antenna isolation of at least 20dB between each of the 4 paths will be needed especially for large CBW.

Comparing in Table 4 the Table in way forward [2] and our results, we see major differences for the 4Tx MPR table especially for the edge allocations. Inner allocation MPR is on the higher side in [2] and outer MPR is well aligned.

However, it is unclear how this table in [2] was derived (measurements/simulations, 1Tx/2Tx?) and which antenna isolation assumptions were used. The difference may be due to different waveform filtering for Edge allocations.

Proposal on 4Tx PC1 results: Differences in results in Table 4 are further discussed for future work to converge on:

- Architecture assumptions like antenna coupling and PA calibration.

- Waveform filtering for edge allocations.

- Modeling of 4Tx coupling, simulation, measurement.

Table 4: 4Tx PC1 MPR comparison for Table in way forward and this contribution

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | QPSK [2] | ≤ [10.0] | ≤ [2.5] | ≤ [0.5] |
|  | QPSK [this contribution] | ≤ [2] | ≤ [2-3] | ≤ [0] |
| CP-OFDM | QPSK [2] | ≤ [10.0] | ≤ [4.5] | ≤ [2.0] |
|  | QPSK [this contribution] | ≤ [3] | ≤ [4-5] | ≤ [1] |

## 2.5 Way Forward

We recognize that our input is far too preliminary to target 4Tx PC1 in Release 19. This is dues to the late agreement on ACLR requirements and still no agreement on antenna isolation assumptions. This did not allow us to engage into an exhaustive set of evaluations and we also needed significant work on emulating a 4Tx performance of varying isolation with a 2Tx measurement.

However, we could perform enough measurements for the corner waveforms to confirm that PC1 4Tx implementation is promising in both TN and NTN applications and are compatible with FWA/CPE, large form factor UEs, and smartphones at high frequencies where antenna coupling can be optimized.

Separately, in [1] we have shown that inner 3rd allocations have a strong potential for boosting whatever the power class as OOB emissions is not subject to IMD3 issues.

Proposal on 4Tx PC1 in R19 and R20 5GA/6G HPUE:

- Unless other companies can provide more data or the WI is extended, we do not expect to extend the WI for the introduction on 4Tx PC1 in NTN R19. Additionally, we are not convinced that commercial 1Tx PC1 PAs will be easily available for large volume/low-cost UEs soon.

- However, we strongly suggest including 4Tx PC1 in the Release 20 5GA HPUE WI and 6G HPUE studies with the following aspects:

  - Target 34dB ACLR for 4Tx PC1 based on 4x26dBm PC2 PAs

  - Target at least 20dB antenna isolation between all 4 paths, <20dB is FFS

  - Target for all UEs, FDD/TDD and both TN and NTN applications

- Separately, we strongly suggest that the Release 20 5GA HPUE WI and 6G HPUE studies include the evaluation of further optional boosting for PC3/2/1.5/1 power classes and based on 1/2/3/4Tx implementations with existing PC3 (23dBm/30dB ACLR MPR1) or PC2 26dBm/31dB ACLR MPR1) PAs with:

  - Inner 3rd allocations which do not generate IMD3 OOB

  - Potential improvement of impairment assumptions.

# Conclusion

In this contribution, we evaluated the emissions for 4Tx PC1 implementation using 4x26dBm PC2 PAs by emulating the equivalent coupling in a 2Tx measurement for different antenna isolation scenarios. THIs allowed us the following conclusions and proposals.

We can draw the following conclusions:

- 32dBm cannot be reached with 4x26dBm PAs even with 34dB ACLR.

- 31dBm 0dB PC1 MPR can be reached with the worst case QPSK DFT-s-OFDM inner thanks to a 34dB ACLR requirement. 37dB ACLR requirement would need at least an extra dB MPR.

- Edge/outer allocations require up to 2/3dB MPR up to 4RB0 for DFT-s-OFDM/CPOFDM, further evaluation will be needed to check beyond 4RB edge allocations

- Full/outer will require 2.5/4dB MPR for DFT-s-OFDM/CP-OFDM

- To achieve reasonable performance, antenna isolation of at least 20dB between each of the 4 paths will be needed especially for large CBW.

Proposal on 4Tx PC1 results: Differences in results in Table 1 are further discussed for future work to converge on:

- Architecture assumptions like antenna coupling and PA calibration.

- Waveform filtering for edge allocations.

- Modeling of 4Tx coupling, simulation, measurement.

Proposal on 4Tx PC1 in R19 and R20 5GA/6G HPUE:

- Unless other companies can provide more data or the WI is extended, we do not expect to extend the WI for the introduction on 4Tx PC1 in NTN R19. Additionally, we are not convinced that commercial 1Tx PC1 PAs will be easily available for large volume/low-cost UEs soon.

- However, we strongly suggest including 4Tx PC1 in the Release 20 5GA HPUE WI and 6G HPUE studies with the following aspects:

  - Target 34dB ACLR for 4Tx PC1 based on 4x26dBm PC2 PAs

  - Target at least 20dB antenna isolation between all 4 paths, <20dB is FFS

  - Target for all UEs, FDD/TDD and both TN and NTN applications

- Separately, we strongly suggest that the Release 20 5GA HPUE WI and 6G HPUE studies include the evaluation of further optional boosting for PC3/2/1.5/1 power classes and based on 1/2/3/4Tx implementations with existing PC3 (23dBm/30dB ACLR MPR1) or PC2 26dBm/31dB ACLR MPR1) PAs with:

  - Inner 3rd allocations which do not generate IMD3 OOB

  - Potential improvement of impairment assumptions.

# References

[1] R4-2507775 On power boosting options for PC2 NR-NTN, Skyworks Solutions Inc., RAN4#115

[2] R4-2508008 WF on NR_IoT_NTN_HPUE_part1, Samsung, RAN4#115