3GPP TSG-RAN4 Meeting #116	R4-2511321

# Bengaluru, IN, 25th – 29th Aug, 2025

Title:	n1 NS_05 PC3 AMPR region adjustment for 15MHz channel bandwidth

Source:	Qualcomm Incorporated

Agenda item:	4.2.1

Release:	Rel-19

Document for:	Approval

# 1.	Background

n1 NS_05 requirements are relaxed since the PHS service is closed. However, the DECT service in the same range 1884.5MHz - 1915.7MHz must still be protected in Japan. The updated Japanese regulations were initially captured and the PHS protection requirement was modified to protect DECT [1], and PC3 A-MPR definitions were agreed in latest WF [3] for NS_05 requirement in the previous meeting. In this paper we verify the further suggestion to remove Region C for 15MHz channel bandwidth with 1932.5 ≤ FC < 1938.7 case to finalize PC3 A-MPR regions for n1 NS_05.

# 2. 	Discussion

## 2.1	Background and Motivation

In [3], the A-MPR regions for 15MHz channel bandwidth with 1932.5 ≤ FC < 1938.7 were agreed as shown below in Table 1.

| Channel bandwidth [MHz] | Carrier centre frequency FC [MHz] | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 15 | 1927.5 ≤ FC < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 |
| 15 | 1932.5 ≤ FC < 1938.7 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | ≤ 2.16MHz/12/scs | ≤ 1.62MHz/12/scs | ≤ 2.16MHz/12/scs | > 1.62MHz/12/scs ≤ 2.16MHz/12/scs |

Table 1: A-MPR regions for NS_05 region split (Power Class 3, Channel BW 15MHz)

Recent efforts and investigation for further harmonization of the A-MPR regions across various company submissions led to the realization that Region C does not require A-MPR for Power Class 3, since the IMD3 peak does not reach the mask and the current PC3 MPR is sufficient for that region.

Hence it was proposed to check the feasibility to remove Region C A-MPR region for 15MHz channel bandwidth with 1932.5 ≤ Fc < 1938.7 since there is no A-MPR need observed for that region. The value of upper end for RBstart for this particular case (≤ 2.16MHz/12/scs) was adopted from the initial region harmonization efforts from [1], where it was deemed required.

We use the observations and conclusions from recent investigations by other companies to base our motivation for this paper and in the following sections, we show how we verify the proposal to remove Region C.

## 2.2 	Measurement and simulation verifications

According to the proposed changes, we first verify the simulated/measured values from our initial submission in [2].

The simulated/measured values in our initial submission in [2] for 15MHz channel bandwidth with 1932.5 ≤ Fc < 1938.7 is as shown in Figure 1 below.

Figure 1. PC3 NS_05 simulation/measurement result from initial submission in [2]

The agreed A-MPR regions from the WF harmonized table in [3] from the previous RAN4 meeting are shown in Figure 2 below.

Figure 2. Agreed PC3 NS_05 A-MPR regions for with 1932.5 ≤ Fc < 1938.7 in WF [3]

## 2.3 	Proposed changes

Proposal 1: Remove Region C A-MPR definition for 15MHz Channel bandwidth 1932.5 ≤ Fc < 1938.7

After comparing Figure 1 and Figure 2, it is evident that indeed Region C does not require A-MPR and can be removed from the n1 NS_05 A-MPR region definitions, since the current MPR is sufficient to meet the NS_05 requirement.

Figure 3 below shows the graphical representation for A-MPR regions for 15MHz channel bandwidth with 1932.5 ≤ Fc < 1938.7 after removal of Region C.

Figure 3. PC3 NS_05 A-MPR regions for 15MHz 1932.5 ≤ Fc < 1938.7 after Region C removal

The corresponding revised region definitions for 15MHz channel Bandwidth is shown below in Table 2.

| Channel bandwidth [MHz] | Carrier centre frequency FC [MHz] | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBstart |
| 15 | 1927.5 ≤ FC < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs; > 0 |
| 15 | 1932.5 ≤ FC < 1938.7 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | ≤ 2.16MHz/12/scs | ≤ 1.62MHz/12/scs |  |  |

Table 2: Revised A-MPR region definition table for Power Class 3 Channel BW 15MHz NS_05

# Conclusion

This paper verifies the efforts and investigation for further harmonization of the A-MPR regions across various company submissions by removing Region C for 15MHz channel bandwidth 1932.5 ≤ Fc < 1938.7 since it was observed that the IMD3 peak does not reach the mask and current PC3 MPR is sufficient to meet the NS_05 requirements.

Proposal 1: Remove Region C A-MPR definition for 15MHz Channel bandwidth 1932.5 ≤ Fc < 1938.7

# Reference

[1] R4-2420481, “WF on PHS protection update”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[2] R4-2500488, “n1 NS_05 and NS_05U PC3 A-MPR re-evaluations”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17th – 21st February, 2025

[3] R4-2508023, “WF on PHS protection update”, KDDI, OPPO, Ericsson, 3GPP TSG-RAN WG4 Meeting #115, Malta, MT, 19-23rd May, 2025