3GPP TSG-RAN WG4 Meeting # 116	R4-2511322

# Bengaluru, India, 25th – 29th August, 2025

Title: 	Discussion on introduction of FR2 PC8

Source: 	Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC

Agenda item:	5.29.1

Document for:	Discussion

- Background

Due to Ericsson’s continue objection but without any better proposal on how to change the current FR2 industry very depressed situation for almost ten years, WF [5~6] was still noted finally, although just several options are listed. Considering the current depressed situation of FR2, to promote the FR2 industry prosperous development, we strongly think that it is beneficial to diversify the FR2 UE types. In this contribution, we continue share our consideration and proposal on the necessity and benefit of introduction of FR2 PC8 for the FR2 industry development.

- Discussion

Comparison between FR2 PC3 and FR2 PC8

As stated in [1], the detailed comparisons between FR2 PC3 and FR2 PC8 are summarized as below:

Table 1: Comparison between FR2 PC3 and FR2 PC8

| FR2 Power Class | Advantage | Disadvantage |
| --- | --- | --- |
| PC3 | Higher peak EIRP = 22.4 dBm | - Higher cost and lower usage rate - Exceeds the business value of the use case - Extreme higher power consumption - 3~4 times of FR1 that will result in overheating - Extreme complex beam management procedure - 2 panels with 1x4 each - Large delay and higher power consumption - Bad user experience and/or discouragement FR2 industry development |
| PC8 | - Low cost - Encourage more vendors to involve FR2 industry - Lower power consumption - Half compared to that of FR2 PC3 - Low complex beam management - One panel with 1x2 - 1/10 of PC3 beam management - Good performance under hotspot area | Lower peak EIRP = 16.4 dBm |

After more than 10 years development of FR2 in 5G ear, it is still in very depressed situation. With introduction of 2.6GHz and 3.5GHz key 5G spectrum and more and more LTE low band refarming to NR, almost all operators are focusing on 5G deployment with those 5G FR1 bands. Only very limited experimental FR2 Network for some operators but with lower priority. The FR2 deployment scenario should target for hotspot areas to improve the network capacity with advantage of larger channel bandwidth of rich FR2 spectrum compared to FR1, such as stadium, scenic spot area with crowded guests and etc., Under such hotspot area, good signal quality can be guaranteed by well designed network deployment. Considering this is the first generation for mmWave industry development, the amount of available usable terminal devices is the key factor for FR2 industry development. With those advantages for FR2 PC8 listed in Table 1, we can naturally draw the conclusion that FR2 PC8 can well meet the real application service request.

Observation 1: Under hotspot area, FR2 PC8 has many advantages than FR2 PC3 to ensure good user experience and facilitate the FR2 industry development.

Comparison between different UE RF architecture

During the previous meeting discussion, some company argued that some FR2 UE already implemented simplified RF architecture with one panel with 1x4 antenna array instead of two panels with 1x4 antenna array each to reduce UE cost, but still support the peak EIRP = 22.4 dBm. Here we would like to emphasize the advantages of FR2 UE with two panels with 1x2 antenna array each compared to that FR2 UE with one panel with 1x4 antenna array.

Table 2: Comparison between different UE RF architectures

| UE RF architecture | Advantage | Disadvantage |
| --- | --- | --- |
| 1 panel with 1x4 antenna array | Higher peak EIRP = 22.4 dBm | - Serious hand blocking issue - Higher power consumption - Worse spherical coverage |
| 2 panels with 1x2 antenna array each | - Better avoid the hand blocking issue - Lower power consumption - Better spherical coverage | Lower peak EIRP = 16.4 dBm |

From the above comparisons, we can know that the only disadvantage of FR2 PC8 is the lower peak EIRP = 16.4 dBm. However peak EIRP is only one specific direction at one time instance during the beam sweeping, spherical coverage is more important to ensure the overall user experience in the whole network.

Observation 2: UE supports RF architecture with 2 panels with 1x2 antenna array each has better performance than UE with 1 panel with 1x4 antenna array.

Network deployment scenario FR1-FR2 DC

The scenario mentioned [7] is under FR1-FR2 NR-DC deployment scenario, UE needs to keep 2 uplinks all the time, 2 uplinks split UE power and limit the UL coverage. So the coverage issue analysed by some company is not due to the lower UE transmit power, the network deployment scheme also impacts the real UL coverage.

In practice DC is a “switch”, not an aggregator [5]. If it is real “switch”, the total capacity will be impacted.

Observation 3: The network deployment FR1-FR2 DC has serious impact on the UL coverage due to the 2 uplinks splitting UE transmit power.

Views on FR2 for 6GR

From the 6G workshop, we collected some operators’ views on FR2 in the upcoming 6GR:

Table 3: Some operator’s view on FR2 for 6G

| Company | Contribution | View |
| --- | --- | --- |
| Deutsche Telekom's | 6GWS-250011 | P4: Spectrum for 6G - > 24GHz: “FR2” and (lower) THz: Lower priority for DT, but 6G assumed to work up to 71 GHz as NR does - {7-24GHz} “FR3”: No potential for Europe currently seen |
| KT Corp. | 6GWS-250021 | P2: 6G spectrum: Upper mid-band（7.125-8.4GHz）to be prioritized for 6G RAN specification developments |
| NTT DoCoMo | 6GWS-250052 | P3: Spectrum aspects - FR2: Despite the huge hard work in NR standard/field, FR2 operation still need improvements in real fields - View: Both FR2 and FR3 should be equally considered for 6G specification work, to achieve real improvements in many regions P8: 6G performance gain – FR2 - FR2 in NR: Expected to improve throughput/capacity, while not successfully deployed - Difficulties for commercialization: Significant propagation loss, LOS-dominant radio - FR2 in 6G: Still very important for throughput/capacity improvement - Availability of 6G new bands (e.g., FR3) is not clear in some regions (e.g., Japan) - Key points for 6G FR2: How to overcome the difficulties? - Direction#1: Usage of more beams and TRPs, to overcome propagation loss and increase LOS probability - Direction#2: Optimize TRP selection, esp. around cell-edge area - Evaluation results show that in FR2, both directions are beneficial (Direction#1=dense (2x TRP), Direction#2=cell free (HO margin=0dB) - For 5%-tile: - Direction#1 with 2x TRPs achieves 78% gain - Direction#2 achieves 20% gain - Direction#1 (with 2x TRPs) + Direction#2 achieves 87% gain P14: FR1: coverage band. Target design: further improved throughput/capacity, utilizing fragmented small spectrum FR3: coverage and capacity band. Target design: better performance than FR1, easier deployment than FR2 FR2: Capacity band. Target design: more practical deployment |
| Verizon | 6GWS-250075 | P11: FR2 optimization for robustness and coverage: Seamless multi-carrier operation cross FR2, FR1 and FR3 P13: FR2 specific: - Advanced antenna system with beam management and PA enhancement - Enhanced multi-frequency operation with FR1 and FR3 |
| Orange | 6GWS-250130 | P4: - mmWave (26GHz): still uncertain; business case not better in 6G vs 5G |
| Vodafone | 6GWS-250153 | P2: Key expectations - Works well in the upper 6GHz band - No inter-RAT DC, CA of upper 6GHz with mid&low band is essential for network performance P5: Consider energy consumption for all 6G components and features |
| Rakuten Mobile, Inc | 6GWS-250236 | P5: Enhanced User Experience in 6G RAN Multi-Band Carrier Aggregation → Optimized resource use across FR1, FR2, FR3. |

From the summary above, we can get the following observations.

Observation 4:

- Some operators set low priority for FR2 considering the lessons learned from 5G FR2 network deployment.

- FR2 is mainly used for throughput/capacity improvement in the hotspots area.

- Good user experience is the ultimate target for all feature introduction

- Energy saving in both UE and BS sides should be considered from Day 1 of 6G

- CA across FR1, FR2 and FR3 should be considered to ensure optimized resource use

- Improved network deployment: such as with more beams or TRPs, cell-free and etc., to ensure no cell-edge performance degradation

Max UL duty cycle for FR2 transmission

As per FCC and ICNIRP regulation request, there are restriction on power exposures:

5.1.4. LOCAL TRANSMITTED POWER DENSITY (>6 GHz – 300 GHz) As described in Section 4.3.3.1.2, the guidelines take a transmitted power density of 200 W m-2 , averaged over 6 minutes and either 4 cm 2 (>6 to 30 GHz) or 1 cm 2 (>30 to 300 GHz) surface area of the body, within the >6 to 300 GHz range, as the local exposure corresponding to the operational adverse health effect threshold for both the Head and Torso, and Limb regions (5 and 2 °C local temperature elevation in Type-1 and Type-2 tissue respectively). As per the local SAR restrictions, a reduction factor of 2 was applied to this threshold for occupational exposure to account for scientific uncertainty, as well as differences in both thermal baselines and thermoregulation ability across the population. This results in a basic restriction for occupational exposure of 100 W m-2. As the general public cannot be expected to be aware of these exposures and thus to mitigate risk, a reduction factor of 10 was applied, which reduces the general public basic restriction to 20 W m-2.

Table 4: Analysis on allowable maximum output power and minimum duty

cycle assuming FCC [10] and ICNIRP [9] exposure limits [9]

| PC3 |  |
| --- | --- |
| Max allowed TRP (dBm) by 3GPP | 23 (200 mW) |
| Max allowed Power Density | 10 W/m2 or 1 mW/cm2 |
| Total allowed exposure is averaged over 4 cm2 (Note: 4cm2 is draft ICNRIP averaging area at f= 28 GHz) | 4 mW/cm2 = 6 dBm (conservative assumption all power is transferred into body) |
| % of exposure | 100* |
| Back off required for compliance | 23 – 6 = 17 dB |
| FCC Averaging interval (sec) | 4 |
| Maximum Duty cycle | 10^(-17dB/10) = 2 % |
| NOTE *: Here we have assumed human tissue is exposed to all of the radiated power. |  |

From the above regulation and analysis, we can know that tradeoff between max UL duty cycle and max TRP transmission needs to be considered:

- The back off required for compliance regulation is 17 dB with 2% max UL duty cycle is required to transmit 23 dBm max TRP and with EIRP = 22.4dBm as defined in the specification [9];

- The back off required for compliance regulation is 11 dB with 8% max UL duty cycle is required to transmit 17 dBm with EIRP = 16.4dBm;

- For TDD pattern DDDSU with S=10:2:2 used in the current FR2 network, i.e. about 23% duty cycle for UL transmission, the back off acquired is about 6 dB;

- For TDD pattern DSUUU with S=10:2:2, about 63% duty cycle for UL transmission, the back off acquired is about 2 dB.

Table 4: Allowable maximum output power and minimum duty

| TDD pattern | DDDSU (23% UL duty cycle) | 2% UL duty cycle | 8% UL duty cycle | DSUUU (63% UL duty cycle) |
| --- | --- | --- | --- | --- |
| Back off acquired (dB) | 6 | 17 | 11 | 2 |
| Max TRP to meet the exposure limits (dBm) | 12 | 23 | 17 | 8 |
| Peak EIRP (dBm) | 11.4 | 22.4 | 16.4 | 7.4 |

From the above Table 4, we can know that shorter UL duty cycle can only be allowed for higher max output power transmission to meet FCC and ICNIRP EMF compliance requirements. It means lower transmission chance for UL and will result in lower throughput ultimately. The lower throughput will completely against the motivation to improve capacity for deployment of mmWave network.

Observation 5: Only lower transmission chance with lower UL throughput can be adopted for higher output power transmission to meet FCC and ICNIRP EMF compliance requirements.

- Conclusion

As per our comprehensive analysis above, our observations and proposals are summarized as following:

Observation 1: Under hotspot area, FR2 PC8 has many advantages than FR2 PC3 to ensure good user experience and facilitate the FR2 industry development.

Observation 2: UE supports RF architecture with 2 panels with 1x2 antenna array each has better performance than UE with 1 panel with 1x4 antenna array.

Observation 3: The network deployment FR1-FR2 DC has serious impact on the UL coverage due to the 2 uplinks splitting UE transmit power.

Observation 4:

- Some operators set low priority for FR2 considering the lessons learned from 5G FR2 network deployment.

- FR2 is mainly used for throughput/capacity improvement in the hotspots area.

- Good user experience is the ultimate target for all feature introduction

- Energy saving in both UE and BS sides should be considered from Day 1 of 6G

- CA across FR1, FR2 and FR3 should be considered to ensure optimized resource use

- Improved network deployment: such as with more beams or TRPs, cell-free and etc., to ensure no cell-edge performance degradation

Observation 5: Only lower transmission chance with lower UL throughput can be adopted for higher output power transmission to meet FCC and ICNIRP EMF compliance requirements.

Proposal 1: Introduce FR2 power class 8 with the access control scheme and with the min peak EIRP = 16.4 dBm and EIRP spherical coverage@50%-ile CDF = 5.5 dBm

- intraFreqReselectionFR2PC8

- cellBarredFR2PC8

- Reference

[1] R4-2507884 Discussion on introduction of FR2 PC8, RAN4#115, Huawei, HiSilicon

[2] R4-2505232 WF on FR2 PC8, RAN4#114bis, Huawei, HiSilicon

[3] R4-2508036 WF on FR2 PC8, RAN4#115, Huawei, HiSilicon

[4] R4-2504322 Discussion on introduction of FR2 PC8, RAN4#114bis, Huawei, HiSilicon

[5] 6GWS-250084 Overall vision & priorities for RAN in 6G, Ericsson

[6] R4-2503175 On the proposed FR2 PC8 (new power class) UE, RAN4#114bis, Qualcomm

[7] R4-2501569 PC8 performance and FR2 handheld UE antenna arrangements, RAN4#114, Ericsson

[8] R4-1900253 Discussion on FR2 UE MPE remaining issues, RAN4#90, OPPO

[9] R4-1900440 P-MPR and maxULDutycycle limit parameters, RAN4#90, Qualcomm

[10] R4-1814719 Update on RF EMF regulations of relevance for handheld devices operating in the FR2 bands, RAN4#89, Ericsson, Sony