3GPP TSG-RAN WG4 Meeting #116	R4-2511327

Bengaluru, India, 25 – 29th August 2025

Source:	BT

Title:	Simulation Results for SCM Alignment

Agenda item:	7.12.3

Document for:	Discussion

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN4#112, RAN4#112b, RAN4#113, RAN4#114, RAN4#114b, and RAN4#115 approved way forward agreements [2-5,7,8] respectively. This contribution provides simulation results updated from [9], based on agreements in [5,7].

Simulation Results

Simulation setup

Simulation parameters are provided in Annex A, where the agreements from [5] are integrated in the table. Throughout the simulation results presented in this contribution we use a truncated version of the TR 38.901 CDL-C channel model based on [7], and the default AAV configurations from [5]. Perfect channel estimation is used in the simulation results provided in this contribution. In our current simulation method, a constant noise power level is utilized while sweeping across different transmit powers, to achieve the target SNR at the receiver ports. The long-term SNR is calculated using the formula

SU PDSCH and PMI results for alignment

Results in this section provide the minimum SNR values required for the given percentage of maximum throughput, at 30% and 70% for PDSCH, and 70% and 90% for PMI. CDL option A4 is based on Annex A of [7].

Table 1. SU PDSCH Results with Truncated TR 38.901 CDL-C Channel Model

| Channel Model |  | UE Speed/ Doppler | PMI | Test point - % of Rel TP | SNR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Codeword 1 | Codeword 2 |
| 4x4 w/ 4 layers |  |  |  |  |  |  |
| CDL Option A4 |  | 3 kph | Type 1 Random | 30% | 6.0 |  |
|  |  |  | Type 1 Random | 70% | 14.3 |  |
| 8x8 w/ 8 layers |  |  |  |  |  |  |
| CDL Option A4 |  | 3 kph | Type 1 Random | 30% | 6.4 | 10.1 |
|  |  |  | Type 1 Random | 70% | 16.9 | 23.7 |

Table 2. SU PMI Results with Truncated CDL-C Channel Model

| Channel Model |  | UE Speed/ Doppler | PMI | Metric (% of Max TP) | SNR |
| --- | --- | --- | --- | --- | --- |
| 8x4 w/ 4 layers |  |  |  |  |  |
| CDL Option A4 |  | 3 kph | Type I Follow | 70% | 11.0 |
|  |  |  |  | 90% | 13.1 |
|  |  |  | eType II Follow | 70% | 14.3 |
|  |  |  |  | 90% | 17.1 |

Conclusion

This contribution provides SNR data points for the CDL channel model alignment cases in [7].

Full numerical results will be provided to the results collection table R4-2509413 during RAN4#116.

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2419782, Way Forward for [113][320] NR_SCM, RAN4#113, Nov 2024.

- R4-2502378, Way Forward [114][322] NR_SCM, RAN4#114, Feb 2025.

- R4-2501744, Simulation Results for SCM Comparison and Alignment, BT Plc, RAN4#114, Feb 2025.

- R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114-bis, April 2025.

- R4-2508776, Way Forward for [115][321] NR_SCM, RAN4#115, May 2025.

- R4-2507824, Simulation Results for SCM Comparison and Alignment, BT Plc, RAN4#115, May 2025.

Annex A: Simulation parameters

| Parameter |  |  | Unit | 8TX |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | FDD |
| TDD DL-UL configurations |  |  |  | N/A |
| MCS |  |  |  | 13 |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8,(4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8,(5) |
|  | CSI-RS interval and offset |  | slot | 10, 0 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | Perfect |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |