3GPP TSG-RAN WG1 Meeting #116	R4-2511328

Bengaluru, India, 25th – 29th August, 2025

Source:	BT, Ericsson

Title:	Channel Properties for Comparison in SCM

Agenda item:	7.12.3

Document for:	Discussion

Introduction

In this contribution, we report on new measures performed by BT and Ericsson on a large set of cells in BT’s live NR network in the UK. The purpose is to provide factual evidence from field deployments for the discussion on channel properties as part of SCM comparison.

This new campaign adds to previously reported properties of MIMO channels from typical deployments [1-7]. This contribution is based on a very large dataset consisting of hundreds of cells and thousands of UEs from an operational network, which gives the observations a strong statistical significance.

In this contribution we focus on the following aspects identified in [8], as well as in singular value analysis:

- Power Angular/Phase Distribution in Tx and Rx Directions (including comments on stability and diversity)

- Time Coherence as channel correlation between symbols

Discussion

Overview of live network measurements

The measurement data consists of radio channels estimated from uplink transmissions between UEs and gNBs in a 5G NR network in the UK. Two carrier frequencies were used, 3.56 GHz and 3.7 GHz, both with 40 MHz bandwidth. The channel estimates were based on uplink transmissions from many thousands of different UEs during regular network operation. Therefore the locations or identities of these UEs are unknown but the statistics from these measurements represent a very good sample of the channel conditions that are experienced in the real world.

The full dataset comprises tens of thousands UEs across 352 cells, classified further into UMa, SMa, and RMa deployment types based on the deployment context. Figure 1 below illustrates respective operating environments.

Regular operation of the network resulted in rapid change in the channel characteristics for certain UEs. Selection of datasets exhibiting a relatively stationary behavior was conducted for singular value analysis in Sec 2.4.

| Urban neighborhood in one of the UMa cells (206 cells) | Suburban neighborhood (128 cells) | Rural area (18 cells) |
| --- | --- | --- |

Figure 1. Birds-eye view of measurement scenarios

Power Angular/Phase Distribution in Tx and Rx Directions

Figure 2A illustrates the relative strengths of receive beams for an SRS transmission port for a specific UE. In the illustrated case, the base station detects the strongest SRS reception over beam with index 6 in the lower row. This is a typical example of a clear spatial preference experienced by the receiving side.

Figure 2B illustrates the evolution of spatial preference across measurement time (several seconds). The figure demonstrates an example of a long-term stable spatial preference across most of the measurement duration. Occasional switching of beams is assumed to be due to the inherent mobility in the operating environment.

| A | B |
| --- | --- |
| Figure 2. Urban UE: Spatial Behavior - stability of the direction of arrival |  |

Measurements reconfirm the existence of long-term stable spatial receive directions experienced by typical operational MIMO systems.

Temporal Coherence and Properties

Figure 3 represents the cumulative distribution function (CDF) of temporal coherence time, Tcoherence, for user equipment (UE) operating in urban, suburban, and rural deployment scenarios.  Tcoherence characterizes the time scale over which the channel’s complex impulse response remains highly correlated and is inversely related to the Doppler spread. These results are relevant for setting the temporal evolution parameters in the SCM. The results were obtained from measured channel realizations following the procedure explained in [7] and illustrate the statistical variability of the coherence time across different propagation environments.

In the urban case, the CDF indicates a narrow distribution of short coherence times, with over 85% of channel realizations below 0.05 s. This reflects high Doppler spreads arising from rich multipath and higher UE mobility, typical of dense urban layouts. The steep curve shape indicates low variability - fast temporal variation is consistently observed across realizations. For SCM parameterization, this implies short time correlation intervals with limited spread, ensuring realistic modelling of CSI ageing.

The rural and suburban environments exhibit more widely distributed coherence times, with the CDF showing a smoother slope and over 90% of realizations below 0.8 s. This broader spread represents moderate Doppler conditions caused by a mix of LoS and NLoS propagation and a lower scatter density than in urban scenarios. In SCM terms, suburban models should incorporate slower average temporal decorrelation than urban while preserving variability to capture changes in UE speed and environmental transitions.

Figure 3. Temporal Coherence per UE Distribution

Figure 4 shows the average temporal correlation as a function of lag for urban, suburban, and rural deployment scenarios. The results are averaged over 1200 UEs in urban, 75 UEs in suburban, and 300 UEs in rural deployments.

In the urban case, the mean correlation decays rapidly, with Tcoherence≈0.02 S, consistent with the short and narrowly distributed coherence times observed in the CDF analysis. The steep initial drop and narrow variability band indicate that fast temporal variation is consistently observed across UEs, reflecting high Doppler spreads in dense scattering environments.

The suburban and rural scenarios both exhibit slower average temporal decorrelation and greater UE-to-UE variability compared to urban. In suburban, the correlation remains above the 0.7 threshold for the whole lag range, while rural reaches Tcoherence≈0.3 S. Both show broader distribution tails in the CDF results, indicating the presence of channels with extended coherence times under low Doppler conditions, as well as occasional faster decorrelation events caused by changes in UE speed or propagation geometry. The larger variability bands for these scenarios capture the diversity of LoS/NLoS conditions and mobility patterns typical outside urban environments.

These findings highlight the importance of modelling both mean behavior and statistical spread in the SCM

Channel experiences a shorter coherence time in urban conditions compared to suburban and rural. This translates in associated Doppler spread expected for these three operating environment classes.

Figure 4. Temporal Properties

Singular Value Profiles

The singular value analysis of the MIMO channels between UEs and base stations confirms previously observed patterns in the distributions of resultant singular values. Figure 5 demonstrates distribution of four singular values for a sample 32x4 MIMO channel. For singular values of the MIMO channel span several decibel, confirming the expectation of generally differing strengths of eigenmodes provided by MIMO channel between receivers and transmitters in MIMO systems.

Figure 5. Typical distribution of singular values for Specific UE over the observation time

Singular values of MIMO channel in operational network generally exhibit a span of several dB. Within that span, eigenmodes are spread with respect to specific operating conditions.

Conclusion

This contribution provided power angular distribution in Tx and Rx directions, time coherence and singular value analysis based on measurements from an operational network. Observations are summarised as follows.

- Measurements reconfirm the existence of long-term stable spatial receive directions experienced by typical operational MIMO systems.

Channel experiences a shorter coherence time in urban conditions compared to suburban and rural. This translates in associated Doppler spread expected for these three operating environment classes.

Singular values of MIMO channel in operational network generally exhibit a span of several dB. Within that span, eigenmodes are spread with respect to specific operating conditions.

References

- R4-2402277 “Discussion on 8Rx general demodulation aspects and spatial channel models”, Nokia, Nokia Shanghai Bell, BT, Feb 2024

- R4-2411557 “Scenarios and Requirements for Spatial Channel Modelling”, BT, Aug 2024

- R4-2415673 “Further observations from deployment and MU-MIMO use cases for SCM evaluation”, BT, Oct 2024

- R4-2419338 “Further observations on the properties of MIMO channels in typical deployments”, BT, Ericsson, Nov 2024

- R4-2501745 “Measurement results for MIMO channels in band n78”, BT, Ericsson, Feb 2025

- R4-2504431 “PMI statistics for measured MIMO channels in band n78”, BT plc, Ericsson, April 2025

- R4-2507427 “Temporal and Frequency Coherence for MIMO channels in band n78”, BT, Ericsson, May 2025

- R4-2508776, “Way Forward for [115][321] NR_SCM”, RAN4#115, May 2025.