3GPP TSG-RAN WG4 Meeting #116	R4-2511330

India, Aug. 2025

Source	:	Qualcomm Incorporated

Title	:	System level considerations for UE RF LPWUS requirements

Agenda item	:	7.24.3.1

Document for	:	Approval

# 1.	Introduction

In this contribution we present high-level proposals that are common to all LP-WUS UE RF requirements.

# 2. 	Discussion

## 2.1	Applicability of LP-WUS requirements

Note that the LR or LPWUR is just a notional concept and is not defined as a physical entity either in RAN1 or RAN2 (see Annex3). It is an abstract logical entity based on some defined functionality. Moreover, for both idle and connected modes, RAN1 has declined to mandate LPWUR operation – it is entirely up to UE when to use LPWUR and when to fall back on the MR. This flexibility is transparent to the network. See Annex 1.

Furthermore, RAN4 have agreed that ‘LP-WUS detection’ is determined based on ‘UE response to network after LP-WUS detection’ [4]:

Issue 4-1-1: How to verify LP-WUR Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - The ultimate test method including the potential test mode will be decided by RAN5

This agreement implies too that LR/MR handover implementation details are UE’s business, and only the UE’s network behavior is standardized.

Observation 1: The LR is an abstract construct and whether the LP-WUS is indeed decoded by the UE in any monitoring occasion is transparent to the network.

Consequently, it is not meaningful to specify LP-WUS requirements as reflection of the LR’s capability, instead these requirements can only apply at a UE level (like all legacy UE RF requirements).

Proposal 1: LP-WUS requirements only apply at a UE level, not at the ‘LR level’.

Also,

Proposal 2: The feature is referred to as ‘LP-WUS’ in the standard.

## 2.2	LP-WUS reference signal side conditions

The LP-WUS feature has implicit dependence on other DL signals for frequency and time tracking. These are LPSS (for type 1 UEs) and SSB (for type 2 UEs). Since these signals are crucial for proper operation of the UE (for this feature), side conditions must also define periodicity of these transmissions, as well as their power levels. RAN1 has made the following agreement pertaining to power [RAN1 chairman notes from RAN1#121]:

While there is some flexibility afforded by RAN1, a conservative assumption is for the gNB to transmit all signals with the same PSD. Accordingly:

Proposal 3: For the sake of setting RAN4 (FR1 and FR2) UE RF core requirements, it is proposed that

- LP-WUS, LPSS and SSB all share the same EPRE.

- LPSS shares the same M and same length as LP-WUS

- LPSS shares the same length in OFDM symbols as LP-WUS

- LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition

- SSB is transmitted at 20 ms periodicity

This proposal is captured in CR form for FR1 [5]  and FR2 [6] respectively.

## 2.3	Capabilities

### 2.3.1	RAN1 FG 62

RAN1 has already defined separate capabilities for idle vs connected mode support and ability to demodulate OFDM content in LP-WUS. See partial excerpt below from current state of RAN1 UE feature list [1]

| Features | Index | Feature group |
| --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

These capabilities already provide considerable flexibility to tailor implementation to target scenarios for the UE. For example, an inexpensive IoT type device that is mostly IDLE and very cost sensitive may only support 62-1. On the other hand, a handheld UE that is mostly in CONNECTED state may choose 62-2 or 62-2a. An extremely cost sensitive device may choose 62-1a, etc. If RAN4 now decides to add another dimension to these, there would be 8 variants for which to set requirements, which makes the requirement framework unnecessarily complicated, and complicates handling of UEs in a real deployment.

There is also the RAN4 agreement that explicitly associate type 1 UEs requirements with demodulation of OOK content, type 2 with OFDM content [2]:

- For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted - 1 RB guard for ASCS and 0.5dB de-sens - For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted - 0 RB guard for ASCS and 0 dB de-sens - For Type 1 and Type2 UE are not limited to specific LP-WUS waveform.

It is therefore proposed that:

Proposal 4: RAN4 LP-WUS type 1 UEs are those that support FG 62-1 and 62-2, type 2 UEs are those that support of FG 62-1a and 62-2a. No further RAN4 capabilities are required for UE RF.

### 2.3.2	Loss of flexibility?

The capabilities convey that a UE that indicates support for 62-1 and 62-2 decodes the OOK content of the LP-WUS. For RAN4 compliance, and in the real world, a type 1 UE remains free to decode the OFDM content in the LP-WUS signal, as long as it meets the performance expectation for that type of UE.

Similarly, a UE that indicates support for 62-1a and 62-2a is expected to decode the OFDM content of the LP-WUS. Here too, the UE is free to derive the LP-WUS indication from the OOK content, provided performance expectations are met for that type of UE.

In summary:

Observation 1: There is no loss of implementation flexibility with re-using RAN1 FG 62 for RAN4 UE RF requirements.

## 2.4	Proposal to amend the NF assumption

The NF assumption drives many LP-WUS requirements, as do the ‘same environment for LR and MR’ assumptions. The latter manifests as a one-to-one relationship between respective MR requirements and LR requirements. This means that the LP-WUS requirements would naturally follow the same frequency breaks and general structure as the MR requirements, at least as a starting point. Now, RAN4 have already agreed on a band-agnostic NF+IM assumption for the LR [3,4]. To ensure simplicity in requirement derivation it would then help to adopt the same NF degradation across bands in the MR for the LR also:

Proposal 5: LR NF trend across bands is assumed to the same as MR NF trend across bands. i.e. noise figure difference between MR and LR is constant across bands.

# 3.	Conclusions

Observation 1: The LR is an abstract construct and whether the LP-WUS is indeed decoded by the UE in any monitoring occasion is transparent to the network.

Proposal 1: LP-WUS requirements only apply at a UE level, not at the ‘LR level’.

Proposal 2: The feature is referred to as ‘LP-WUS’ in the standard.

Proposal 3: For the sake of setting RAN4 (FR1 and FR2) UE RF core requirements, it is proposed that

- LP-WUS, LPSS and SSB all share the same EPRE.

- LPSS shares the same M and same length as LP-WUS

- LPSS shares the same length in OFDM symbols as LP-WUS

- LPSS is transmitted at lowest periodicity (160 ms), or equivalently at highest rate of repetition

- SSB is transmitted at 20 ms periodicity

Proposal 4: RAN4 LP-WUS type 1 UEs are those that support FG 62-1 and 62-2, type 2 UEs are those that support of FG 62-1a and 62-2a. No further RAN4 capabilities are required for UE RF.

Observation 1: There is no loss of implementation flexibility with re-using RAN1 FG 62 for RAN4 UE RF requirements.

Proposal 5: LR NF trend across bands is assumed to the same as MR NF trend across bands. i.e. noise figure difference between MR and LR is constant across bands.

# 4.	References

[1]	R1-2501397, ‘Initial RAN1 UE features list for Rel-19 NR after RAN1 #120’

[2]	R4-2505104, ‘WF for [114bis][131] NR_LPWUS_UERF’, vivo RAN4#114-Bis

[3] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[4]	R4-2507936, ‘WF for [115][136] NR_LPWUS_UERF’, vivo RAN4#115

[5]	R4-2511341, ‘dCR to 38.101-1, LPSS and SSB side conditions’, Qualcomm, RAN4#116

[6]	R4-2511343, ‘dCR to 38.101-2, LPWUS feature CR’, Qualcomm, RAN4#116

# 5.	Annex 1, RAN1 rule for monitoring LP-WUS

In general, RAN1 has left it up to the UE on when it should rely on the LR for triggering PDCCH/paging monitoring, and when it needs to fall back on the MR.

For connected mode:

There is no consensus in RAN1 whether to specify UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS in RRC CONNECTED mod

It means no specific condition is specified in RAN1 on how / whether a UE falls back from WUS monitoring e.g. in case the UE is out of WUS coverage. For connected mode, there is no entry/exit condition, no out-of-coverage report, etc. This practice is aligned with the legacy DCP feature (Rel-16 WUS).

Similarly for IDLE mode:

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

Here too, the interesting detail is no mandate on the UE to use the LR  (i.e no ‘shall’).

Observation: RAN1 does not mandate the UE to use LPWUR in connected or idle/inactive mode, it is up to the UE whether it uses the LR or MR