3GPP TSG-RAN WG4 Meeting #116	R4-2511334

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	TP for TR 38.774: FR2 sensitivity for LP-WUS

Agenda Item:	7.28.3.3

Document for:	Approval

# Background

This contribution provides the TP for TR 38.774 on sensitivity requirements for LP-WUS.

# References

[1] R4-2503830, TR 38.774 V0.3.0

# Text Proposal

<Start of Text Poposal>

### 6.3.5	FR2 simulations

120 kHz is the relevant SCS for FR2. For 120 kHz SCS, RAN1 have already agreed that M = {1} and L = {1,2}. SNR curves for 1% FAR are presented for N = 5 (number of information bits) to the right.

The SNR curves differ based on expected frequency and time error, which in turn is related to frequency with which these parameters are tracked by the UE. In connected mode, the error window is expected to be smaller and consequently the threshold SNR is lower.

The key take away is that the SNR threshold is in the -9 dB range, much lower than in FR1, primarily due to reduction in M from 4 to 1. This value, however, is well beyond the coverage needs of MSG3.

Now, FR2 networks differ fundamentally from legacy FR1 networks, because of beam forming. Moreover, cell coverages are smaller, which means there is less opportunity to exercise the full 32 UE/UE subgroup facility allowed by RAN1 design. While OFDM-based receivers tolerate short LP-WUS, the signal duration is still tied to the number of information bits. Longer signal durations are not favorable for the network. Considering the lack of need to support as many as 32 UE subgroups, most deployment are likely to configure only 2 or 3 information bits.  It is therefore suggested that for FR2, RAN4 adopt 3 information bits as the side condition for LP-WUS requirements. The SNR comparison between 3 and 5 bits is presented in the figure to the right, and it confirms that there is no appreciable change in threshold performance.

Consequently, the FR2 LPWUS parameter set is adopted as:

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 120kHz |
| Chip rate |  | M=1 (1 chip in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | L = 2 |
| WUS duration for OOK |  | N/A (*) |
| WUS duration for OFDM |  | 6 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 3 |

While M = 1 affords better sensitivity for FR2 compared to FR1 (M = 4), for the sake of uniformity, the FR1 threshold value of -4.5 dB is adopted for FR2 also.

<End of Text Poposal>

<Start of Text Poposal>

### 7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas.

In RAN4, there had been no consensus on what IM includes, and no agreement was achieved on individual values of NF and IM, so a joint number was agreed. There are two sets of NF + IM values agreed, and these will be used for REFSENS calculations. The values are,

-	Set 1: 18 dB,

-	Set 2: 13.5 dB.

The above mentioned numbers are for FR1. For FR2, the NF + IM value is agreed as 12.0 dB for set 2.

The NF assumption drives many LP-WUS requirements, as do the ‘same environment for LR and MR’ assumptions. The latter manifests as a one-to-one relationship between respective MR requirements and LR requirements. This means that the LP-WUS requirements would naturally follow the same frequency breaks and general structure as the MR requirements, at least as a starting point. While, RAN4 have already agreed on a band-agnostic NF+IM assumption for the LR, for the sake of spec simplicity, it would help to adopt the same NF degradation across bands in the MR for the LR also.

### 7.1.4		REFSENS requirements

#### 7.1.4.1	General

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR.

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively. Further, there are two set of values for NFLR + IMLR, i.e. 18.0dB and 13.5dB for Type 1 and Type 2 UEs, respectively. There is only single receiver so there is no diversity gain included in the REFSENS.

In the REFSENS test, only the RBs allocated for LP-WUS are populated with the LP-WUS and the rest of the RBs for a given channel bandwidth are left empty. This configuration, however, does not correspond to the real-life case where NR signals in other RBs are generally present and thus can create a coverage shortfall. Separately, the ASCS requirement allows a maximum degradation of 0.5 dB and the side conditions incorporate populated adjacent RBs.

Given the small value of the ASCS degradation factor (0.5 dB), it was agreed not to include this factor in the REFSENS.

For FR1, a SNR value of -4.5 dB for both Type 1 and Type 2 UEs was selected by RAN4 to be used for the REFSENS equation.

For FR2, the same general principle is retained while recognizing that the basic expression for REFSENS also contains aspects of antenna gain and dual pol. combining. These details are documented separately.

#### 7.1.4.2	FR2 sensitivity

##### 7.1.4.2.1	Topology

For FR2 it is impractical to have a standalone LR module, and heavy front-end reuse with the MR is anticipated for the LPWUS feature. Legacy FR2 connected mode assumptions feature a 4 dual-pol. element array (‘module’). In keeping with the low-power goal of the feature, RAN4 have agreed that LPWUS reception would assume single (dual pol.) element reception. Moreover, RAN4 have agreed that there is no combining received signals across polarizations, instead there is merely section of the Rx chain with the stronger signal. While implementation details are open, this assumption allows placing a bound on combining gain of -3 dB, in contrast to 0 dB for the same parameter in legacy PDSCH reception.

##### 7.1.4.2.2	Antenna gain

Single element beam

Multi-element refined beam examples

Uniform array with repeated element

Superposition of all refined beams

For a uniform array of repeated elements, it is well understood that the shape described by the envelope of all possible refined beams is congruent to the shape of the beam of each element. See figure to the right.

Specifically, for a uniform array, the refined beam envelope in any direction has 10log(Nelem) dB more gain compared to that of the prototype element. This is justification to maintain the same gain drop and spherical coverage fraction for LPWUS reception as for legacy MR operation.

Due to coverage having identical shape whether as an array or as a single element, sensitivity in any direction (including REFSENS in legacy beam peak direction) for LPWUS can be calculated formulaically from the legacy MR sensitivity requirements, agreements in RAN4 and basic principles:

- NF+IM is 12 dB (1dB degraded from low FR2 band conn. mode PDSCH reception)

- Diversity gain is -3 dB (3dB degraded from conn. mode PDSCH reception)

- Antenna gain assumption is based on single element rather a 4-element array (6 dB degraded from conn. mode PDSCH reception)

- The SNR proposal for FR2 LPWUS is -4.5 dB (3.5 dB improvement from MCS4)

- BW adjustment (degraded by 10log(NRB/11))

The net degradation of LPWUS sensitivity from connected mode PDSCH requirement is therefore 6.5 + 10log(NRB/11) dB, i.e. (1+3 + 6 -3.5 +10log(NRB/11)) dB.

.

<End of Text Poposal>