3GPP TSG-RAN WG4 Meeting #116	R4-2511335

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	TP for TR 38.774: Reference signal side conditions for LP-WUS

Agenda Item:	7.28.3.3

Document for:	Approval

# Background

This contribution provides the TP for TR 38.774 on max. power limit and ASCS for LP-WUS.

# References

[1] R4-2503830, TR 38.774 V0.3.0

# Text Proposal

<Start of Text Poposal>

### 7.1.1	General

#### 7.1.1.1	FR1

The minimum requirements will be determined assuming there is only one receiver. A 1% MDR criterion will be used for verification of all core RF requirements. Following LP-WUS parameters will be used

Table 7.1.1-1. Common reference channel parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | Bits | 16 |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| Overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 |
| WUS duration for OOK |  | 8 OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 5 |

In the specification, tables will be defined corresponding to 15 KHz and 30 KHz SCS.

#### 7.1.1.2	FR2

The FR2 requirements are target towards enable exclusively type 2 receivers. Accordingly, the following LP-WUS parameters will be used for the requirement.

Table A.3M.1-1. Common reference channel parameters for 60 KHz SCS.

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | KHz | 60 |
| Chip rate | chip per OFDM symbol | M=1 |
| Overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 2 |
| LP-WUS duration for [type 2 LR] | OFDM symbols | 6 |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 3 |

Table A.3M.1-2. Common reference channel parameters for 120 KHz SCS.

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | KHz | 120 |
| Chip rate | chip per OFDM symbol | M=1 |
| Overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 2 |
| LP-WUS duration for [type 2 LR] | OFDM symbols | 6 |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 3 |

#### 7.1.1.3	Common to FR1 and FR2

The LPWUS feature has implicit dependence on other DL signals for frequency and time tracking. These are LPSS (for type 1 UEs) and SSB (for type 2 UEs). Since these signals are crucial for proper operation of the UE (for this feature), side conditions must also define periodicity of these transmissions, as well as their power levels. Accordingly, the following parameters are chosen for the LPWUS requirement.

Table A.3M.1-3. Relative power of SSB, LPSS and LP-WUS

| Parameter | Unit | Value |
| --- | --- | --- |
| Ratio of LP-WUS EPRE with respect to SSB EPRE | dB | 0 |
| Ratio of LP-WUS EPRE with respect to LPSS EPRE | dB | 0 |

Table A.3M.1-4. Parameters for LPSS (FR1 only).

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LPSS bandwidth | RB | 11 |
| Subcarrier spacing | KHz | (same as LP-WUS) |
| RM coding | Bits | 32 |
| Chip rate | chip per OFDM symbol | M=4 |
| LPSS duration | OFDM symbols | 8 |
| Manchester coding for OOK |  | none |

<End of Text Poposal>