3GPP TSG-RAN4 Meeting #116	R4-2511339

# Bengaluru, IN, Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR for 38.101-1: LPWUS max. input level + ASCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Max input limit and ASCS for new feature (LP-WUS) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add max. input limit, ASCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete LP-WUS requirment for Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 7.4M, 7.11M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* start change */

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aerial UE: A UE supporting UAS (Uncrewed Aircraft Systems) as indicated by the capability aerialUE-Capability-r18 [15] and that has an aerial subscription as described in TS 23.256 [17]. The UE is considered to have access to UAS services after the UE ha performed a successful authentication and authorization with the USS as described in TS 23.256 [17].

Aggregated Allocation Bandwidth: Total bandwidth of all allocated RBs in a transmission occasion. Can be calculated for two aggregated CCs as LCRB, 1* 12* SCS1 + LCRB,2 * 12 * SCS2 .

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

ATG UE: The terminals or user equipments which are mounted in aircraft and support ATG feature (i.e. UE capability airToGroundNetwork-r18) as defined in clause 4.2.2 from TS38.306[15].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Concurrent operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

eRedCap UE: The UE with enhanced reduced capabilities as defined in clause 4.2.22.1 from TS38.306 [15].

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Inter-band concurrent operation: Operation of NR Uu carrier and NR Sidelink carrier in different operating bands.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Intra-band SL CA UE: UE that supports NR SL CA operation in a single band

LP-WUS power boost: difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)

NR SL CA: Aggregation of two or more NR Sidelink component carriers in order to support wider transmission bandwidths

NR SL inter-band concurrent operating Band：Band combinations of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL-U UE: UE that supports NR Sidelink operation in unlicensed bands (e.g. n46, n96, n102).

Railway Mobile Radio: railway operations encompassing GSM-R and its successor(s), including the Future Railway Mobile Communication System (FRMCS); in the context of this specification the Railway Mobile Radio is limited to NR operation in band n100, or n101.

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [15].

Sub-band:  For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands.  Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

Two Rx antenna port XR UE: A non-(e)RedCap XR UE that is equipped with only two Rx antenna ports in frequency band(s) where 4 Rx antenna ports are required. The UE is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

/* end change */

/* start change */

## 7.4M	Maximum input level for LP-WUS

Maximum input level is defined as the maximum mean power received at the UE antenna connector, at which the MDR criterion shall be met with the side conditions in clause [7.1M] and with parameters specified in Table 7.4M-1, and in Tables 7.4M-2a and 7.4M-2b respectively. For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M.

Table 7.4M-1: Maximum input level, Case 1

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in LP-WUS1 transmission BW config | dBm | -25 | -25 + 10log10(BWChannel /20) | -20 |
| Power of each non-LP-WUS DL RB | mW | 0 |  |  |
| ASCS Guard RB2 | PRB | 0 |  |  |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 2: See section 5.3M |  |  |  |  |

Table 7.4M-2a: Maximum input level for a [Type1 UE], Case 2

| RX parameter | Units | Channel BW (MHz) |  |  |
| --- | --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Total power including LP-WUS carrier1 and non-LPWUS RBs | dBm | -25 | -25 + 10log10(BWChannel /20) | -20 |
| ASCS Guard RB2 | PRB | 2 |  |  |
| LP-WUS power boost | dB | 0 |  |  |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 2: See section 5.3M NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS. |  |  |  |  |

Table 7.4M-2b: Maximum input level for a [Type2 UE], Case 2

| RX parameter | Units | Channel BW (MHz) |  |  |
| --- | --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Total power including LP-WUS carrier1 and non-LPWUS RBs | dBm | -25 | -25 + 10log10(BWChannel /20) | -20 |
| ASCS Guard RB2 | PRB | 0 |  |  |
| LP-WUS power boost | dB | 0 |  |  |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 2: See section 5.3M NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS. |  |  |  |  |

/* end change */

/* start change */

## 7.11M	Adjacent subcarrier selectivity for LP-WUS

For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M. Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS carrier at its configured RBs, in the presence of fully populated non-LP-WUS DL RBs. The MDR criterion in clause [7.1M] shall be met with the side conditions in [7.1M] and the parameter sets specified in Tables 7.11M-1 and 7.11M-2 respectively. The requirement applies for all channel BW included in sub-clause 5.3M.

Table 7.11M-1: ASCS side conditions for a [type1 UE]

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS1 + 0.5 |
| ASCS Guard RB2 | PRB | 2 |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: REFSENSLP-WUS is the value specified in clause [7.3M.2] NOTE 2: See section 5.3M NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS |  |  |

Table 7.11M-2: ASCS side conditions for a [type2 UE]

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS1 |
| ASCS Guard RB2 | PRB | 0 |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: REFSENSLP-WUS is the value specified in clause [7.3M.2] NOTE 2: See section 5.3M NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS |  |  |

/* end change */