3GPP TSG-RAN4 Meeting #116	R4-2511340

# Bengaluru, IN, Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to 38.101-1, LPWUS spurious resp., IM, spurious emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce spurious response, intermodulation and spurious emissions requirements for LP-WUS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify spurious response, intermodulation and spurious emissions, based on legacy requirement (because LR and MR must withstand the same intereferers). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete LP-WUS specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7M, 7.8M, 7.9M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change >>

## 7.7K	(Reserved)

## 7.7L	(Reserved)

## 7.7M	Spurious response for LP-WUS

The UE’s LP-WUS spurious response is a measure of the UE’s ability to receive an LP-WUS signal at its configured RBs inside the configured channel in the presence of an unwanted CW interfering signal at any frequency other than for which a response is obtained, i.e. for which the out-of-band blocking limit as specified in clause 7.6M.3 is not met.

The MDR criterion with the side conditions specified in 7.1M shall be met with parameters for LP-WUS as specified in Table 7.7M-1 and Table 7.7M-2, and for the interference as specified in Table 7.7M-3.

For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M. For this requirement, the power in any non-LP-WUS DL RB is 0 mW.

Table 7.7M-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS transmission BW config 2 | dBm | REFSENSLP-WUS + 6 dB | REFSENSLP-WUS + 7 dB | REFSENSLP-WUS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 2: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |

Table 7.7M-2: Spurious response parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS + 6 dB | REFSENSLP-WUS + 7 dB | REFSENSLP-WUS + 9 dB |

Table 7.7M-3: Spurious response interferer parameters

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

## << Next change >>

## 7.8K	(Reserved)

## 7.8L	(Reserved)

## 7.8M	Intermodulation characteristics for LP-WUS

### 7.8M.1	General

The LP-WUS intermodulation response rejection is a measure of the UE’s ability to receive an LP-WUS signal at its configured RBs inside the configured channel in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M. For this requirement, the power in any non-LP-WUS DL RB is 0 mW.

### 7.8M.2	Wide band Intermodulation for LP-WUS

The wide band intermodulation requirement is defined using a CW carrier as interferer 1 and a modulated NR signal as interferer 2. The MDR criterion with the side conditions specified in 7.1M shall be met with parameters specified in Table 7.8M.2-1 and Table 7.8M.2-2.

Table 7.8M.2-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS transmission BW config 5 | dBm | REFSENSLP-WUS + 8 dB | REFSENSLP-WUS + 6 dB | REFSENSLP-WUS + 7 dB | REFSENSLP-WUS + (9 + 10log10(BWChannel /20)) dB |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
| BWInterferer 2 | MHz | 3 | 5 |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 – 4.5 /+BW/2 + 4.5 | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |
| NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the UE channel and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the UE channel and the center frequency of the modulated interferer. NOTE 5: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.8M.2-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | REFSENSLP-WUS + 6 dB |
| PInterferer 1 (CW) | dBm | -46 |
| PInterferer 2 (Modulated) | dBm | -46 |
| BWInterferer 2 | MHz | BWChannel |
| FInterferer 1 (Offset) | MHz | -2BWChannel / +2BWChannel |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |
| NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and the same SCS as the wanted signal. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the UE channel and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the UE channel and the center frequency of the modulated interferer. |  |  |

## << Next change >>

## 7.9J	Spurious emissions for ATG

For ATG UE, the spurious emissions as specified in clause 7.9 applies.

## 7.9M	Spurious emissions for LP-WUS

The spurious emissions as specified in clause 7.9 applies.

## << End of change >>