3GPP TSG-RAN WG4 Meeting #116	R4-2511346

Bengaluru, IN, August 25-29, 20205

Source:	AT&T

Title:	Coexistence discussion for the introduction of the North American 4.9GHz band

Agenda item:			6.14.2

Document for:	Discussion

# Introduction

The NR 4.9 GHz band work item was introduced at RAN#108[1]. The first objective of the WID is a study phase with the following tasks:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

  - Identify adjacent spectrum protection requirements

    - Identify any potential emission limits more stringent than the normal BS and UE emission masks

  - Identify other technologies operating in the 4940-4990 MHz band

    - Identify potential coexistence mechanism(s)

      - Frequency division sharing

      - Geographic separation

      - TDD synchronization

    - Identify any potential emission limits more stringent than the normal BS and UE emission masks

This document presents AT&T perspectives on the questions posed by this objective of the work item and proposes to commence the normative phase of the work item based on these perspectives.

# Discussion

The 4.9 GHz band (4.940-4.990 GHz) was allocated for public safety users by the FCC in 2002 [2], capturing the emissions standards in 47 CFR 90. The band has been used for a variety of public safety applications including P-MP, PTP broadband and robotics. This band has licensees throughout the continental United States, Alaska, and Hawaii.

Observation 1:	Devices and applications currently in use in the 4.9 GHz band are subject to 47 CFR 90 rules and will continue to be subject to those rules.

Observation 2:	Devices and applications currently using frequencies within the 4.9 GHz band and those using frequencies adjacent to the 4.9 GHz band must coexist with 4.9 GHz devices operating in compliance with 47 CFR 90.

Review of the FCC Universal Licensing System [4] shows a broad diversity of users using channel bandwidths spanning 1 to 20 MHz. The main usage is Fixed PTP and PMP OFDM-based services, with the remainder consisting of different types of mobile services.

Observation 3:	The existing environment has a broad diversity of incumbent 4.9 GHz and adjacent spectrum coexistence scenarios. These scenarios include non-3GPP systems, which are beyond the scope of RAN4 coexistence studies.

After collecting comments from public industry stakeholders and citing the success of the practice of assigning band managers for the 200 MHz band and 700 MHz guard bands, the FCC has decided that the appointment of a band manager will result in the most efficient use of the 4.9 GHz band. The roles and responsibilities of the band manager are presented in [2] and are further clarified in [3]. The band manager is empowered to provide frequency coordination and interference protection services, which involves frequency coordination, setting power limits and other tasks necessary to ensure coexistence. The FCC judged this as necessary due to the recognition that avoiding interference with incumbent users while still maximizing the use of the band requires comprehensive knowledge of the diversity of public safety systems.

Observation 4:	Recognizing the broad diversity of possible coexistence scenarios, the FCC has decided to appoint a band manager with expertise in public safety systems to manage 4.9 GHz frequency coordination.

The band manager is also directed to facilitate non-public safety uses of the band (subject to appropriate safeguards which protect public safety operations) to make most efficient use of the frequency resources. The band manager therefore has the responsibility to protect all users of the frequency band.

The FCC has also afforded the band manager the flexibility to approve applications which cause or receive more interference than provided for in 47 CFR 90 provided that the potential licensee and incumbent user(s) agree to the terms of the application and subsequent interference levels.

Observation 5:	The band manager is responsible for managing all aspects of RF coexistence between 4.9 GHz users (new and incumbent) and between 4.9 GHz users and users in adjacent bands and is therefore expected to account for adjacent channel spectrum requirements and will mediate cases where emission limits more stringent than the normal BS and UE-emission masks are indicated. This responsibility applies equally to potential 3GPP technologies or other technologies operating in the 4940-4990 MHz band.

Finally, the FCC has directed the band manager to make recommendations on best practices for use of technologies within the 4.9 GHz band based on their background and expertise. This is directed towards making the most efficient use of the band resources and protecting public safety operations.

Observation 6:	The band manager will make use of best practices for authorizing license applications, including supervision of frequency division sharing, geographic separation, and TDD synchronizations.

As discussed in [2], the FCC has amended 47 CFR 90 to include emission masks L and M in Part 1 §90.210 which can readily be applied to the 3GPP UE power classes. Mask M applies to high power devices (> 20 dBm) and Mask L applies to low power devices (< 20 dBm).

Proposal 1:	To minimize disruption with both incumbent 4.9 GHz users and users adjacent to the 4.9 GHz band, RAN4 should consider adoption of the relevant emissions requirements in 47 CFR 90 for the 3GPP 4.9 GHz band.

Proposal 2:	Having addressed the outstanding coexistence questions posed in [1], RAN4 should commence work on defining requirements for the 4.9 GHz band

# Conclusions

Observation 1:	Devices and applications currently in use in the 4.9 GHz band are subject to 47 CFR 90 rules and will continue to be subject to those rules.

Observation 2:	Devices and applications currently using frequencies within the 4.9 GHz band and those using frequencies adjacent to the 4.9 GHz band must coexist with 4.9 GHz devices operating in compliance with 47 CFR 90.

Observation 3:	The existing environment has a broad diversity of incumbent 4.9 GHz and adjacent spectrum coexistence scenarios. These scenarios include non-3GPP systems, which are beyond the scope of RAN4 coexistence studies.

Observation 4:	Recognizing the broad diversity of possible coexistence scenarios, the FCC has decided to appoint a band manager with expertise in public safety systems to manage 4.9 GHz frequency coordination.

Observation 5:	The band manager is responsible for managing all aspects of RF coexistence between 4.9 GHz users (new and incumbent) and between 4.9 GHz users and users in adjacent bands and is therefore expected to account for adjacent channel spectrum requirements and will mediate cases where emission limits more stringent than the normal BS and UE-emission masks are indicated. This responsibility applies equally to potential 3GPP technologies or other technologies operating in the 4940-4990 MHz band.

Observation 6:	The band manager will make use of best practices for authorizing license applications, including supervision of frequency division sharing, geographic separation, and TDD synchronizations.

Proposal 1:	To minimize disruption with both incumbent 4.9 GHz users and users adjacent to the 4.9 GHz band, RAN4 should consider adoption of the relevant emissions requirements in 47 CFR 90 for the 3GPP 4.9 GHz band.

Proposal 2:	Having addressed the outstanding coexistence questions posed in [1], RAN4 should commence work on defining requirements for the 4.9 GHz band

# References

[1]	RP-251887 New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu, RAN#108, June 2025

[2]	SEVENTH REPORT AND ORDER AND NINTH FURTHER NOTICE OF PROPOSED RULEMAKING, Federal Communications Commission, WP Docket No. 07-100, Released January 18, 2023

[3]	EIGHTH REPORT AND ORDER, Federal Communications Commission, WP Docket No. 07-100, Release October 22, 2024

[4]	FCC Universal License System search tool License Search - Advanced License Search