3GPP TSG-RAN WG4 Meeting #116	R4-2511356

Bengaluru, India, 25 – 29 August 2025

Source:	Ericsson, BT plc

Title:	TP for 38.750 to add PC2 2BUL for CA_n3-n7

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.750 to add PC2 2BUL for CA_n3-n7 as specified in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 5.X	CA_n3-n7

### 5.X.1	UE maximum output power

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A | n38 n78 CA_n3A-n7A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.X.2	Reference sensitivity requirements

For PC3 DL CA_n3A-n7A with UL CA_n3A-n7A, IMD4 MSD has been defined. This section will propose MSD for UL CA_n3A-n7A based on the PC3 MSD. The PC3 MSD for CA_n3-n7 is:

Table 5.X.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |

MSD for PC2 UL CA is calculated from IMD4 MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 5.X.2-2:

Table 5.X.2.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 21.8 | FDD | IMD4 |

### 5.X.3	∆TIB and ∆RIB values (Void)

---End of changes---

# Reference

[1] 	RP-251450, Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary Uplink (UL); China Telecom