3GPP TSG-RAN4 Meeting #116	R4-2511377

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.181 | CR | 0027 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing Correlation Matrix info (8.2 TC; 36.181; Rel 18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correlation Matrix information is not explicitly mentioned |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a note indicating ‘Low’ correlation in the table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | Y | Other core specifications |  |  |  | TS/TR .36.108.. CR ..0034. |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.2	Performance requirements for PUSCH

### 8.2.1	Performance requirements of PUSCH in multipath fading propagation conditions transmission on single antenna port for coverage enhancement

#### 8.2.1.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

The tests for CEModeA defined in clause 8.2.1 are applicable only to the space access nodes supporting coverage enhancement configured with CEModeA. The tests for CEModeB defined in clause 8.2.1 are applicable only to the space access nodes supporting coverage enhancement configured with CEModeB.

A test for a specific channel bandwidth is only applicable if the SAN supports it. For a SAN supporting FDD multiple channel bandwidths, only the tests for the lowest and the highest FDD channel bandwidths supported by the SAN are applicable.

#### 8.2.1.2	Minimum Requirement

The minimum requirement is in TS 36.108 [2] clause 8.2.1.

#### 8.2.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.1.4	Method of test

##### 8.2.1.4.1	Initial conditions

Test environment: Normal, see annex B.

RF channels to be tested: M; see clause 4.7.

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN TAB connectors for diversity reception via a combining network as shown in annex D.5.

##### 8.2.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth (MHz) | AWGN power level |
| --- | --- |
| 1.4 | -92.7dBm / 1.08MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.1.4.2-2. The index of the narrowband is set to 0. For reference channels using resource blocks less than 6, the resource blocks shall be allocated from the lowest number within the indicated narrowband.

Table 8.2.1.4.2-2: Test parameters for testing PUSCH

| Parameter | CEMode A | CEMode B |
| --- | --- | --- |
| Maximum number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | FDD: 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Duration of PUSCH segment transmission (Nsegmentprecompensation) | 8ms | 256ms |
| No. Repetitions | 8 | 256 |
| Frequency hopping | OFF | OFF |
| NOTE: Guard period shall be created according to TS 36.211, clause 5.2.5 [12]. |  |  |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

4)	Adjust the equipment so that required SNR specified in table 8.2.1.5-1 to 8.2.1.5-2 is achieved at the SAN input.

5)	The test signal generator sends frames, and the receiver tries to demodulate the content. The frames are sent with certain frequency offsets as described below in table 8.2.1.4.2-3 as well as certain timing offsets as described below in table 8.2.1.4.2-4.

Table 8.2.1.4.2-3: Frequency Offsets for testing PUSCH

| Channel | Frequency Offset (Hz) |
| --- | --- |
| CEmode A | 4 |
| CEmode B | 128 |

Table 8.2.1.4.2-4: Timing Offsets for testing PUSCH

| Channel | Step size Δt |
| --- | --- |
| CEmode A | 0.01 µs per subframe |
| CEmode B | 0.01 µs per subframe |

6)	For each of the reference channels in table 8.2.1.5-1 to 8.2.1.5-2 applicable for the access node, measure the throughput, according to annex A.

#### 8.2.1.5	Test Requirement

The throughput measured according to clause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5-1 for CEMode A tests and not be below the limits for the SNR levels specified in table 8.2.1.5-2 for CEMode B tests.

Table 8.2.1.5-1 Minimum requirements for PUSCH, 1.4 MHz channel bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (annex F) | FRC (annex A) | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode A | NTN-TDLA100-5 | A5-2 | 70% | -2.5 |
| 1 | 1 | Mode A | NTN-TDLC5-5 | A5-2 | 70% | -2.8 |
| 1 | 2 | Mode A | NTN-TDLA100-5 Low | A5-2 | 70% | -6.7 |
| 1 | 2 | Mode A | NTN-TDLC5-5 Low | A5-2 | 70% | -6.7 |

Table 8.2.1.5-2 Minimum requirements for PUSCH, 1.4 MHz channel bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Propagation conditions and correlation matrix (annex F) | FRC (annex A) | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode B | NTN-TDLA100-5 | A5-1 | 70% | -11.0 |
| 1 | 1 | Mode B | NTN-TDLC5-5 | A5-1 | 70% | -12.7 |
| 1 | 2 | Mode B | NTN-TDLA100-5 Low | A5-1 | 70% | -13.9 |
| 1 | 2 | Mode B | NTN-TDLC5-5 Low | A5-1 | 70% | -14.9 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.