3GPP TSG RAN WG4 Meeting #116	 R4-2511379

Bengaluru, India, 25 Aug. – 29 Aug. 2025

Agenda Item:	7.25.3.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss Event triggered measurement requirements based L1 measurement requirements.

# Discussion

During the last meeting, RAN4 agreed on the measurement delay requirements for event-triggered reporting. One of the remaining open issues is the applicability of the L1-RSRP reporting delay. The WF from the last meeting is listed below.

- Option 1: RAN4 considers following conditions in the test for L1-RSRP reporting delay requirements: (vivo, Xiaomi) - No indicate TCI change during the evaluation period. - No configuration change w.r.t measurement RS during the evaluation period - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period - Option 2: Seems no need to capture assumptions listed under option-1 into core requirements. (Apple) - Option 2a: The proposed L1-RSRP reporting assumptions are not captured in core requirements. During performance phase, RAN4 can consider ensuring serving beam kept unchanged (e.g., no indicate TCI change, and no configuration change) for L1 event trigger reporting test cases. (HW)

Among the options discussed, we believe our view aligns more closely with Option 2 and Option 2a. Even Option 1 only addresses the conditions during the test. Therefore, we believe this does not need to be included in the core part of the requirements.

- Following to be discussed as part of the test configuration and not the core part.

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period.

## Requirements for reporting criteria per measurement category

In previous meetings, there was a proposal to either share the capability of L3 events with LTM events or introduce a new capability. However, we believe coordination between the CU and DU will be challenging, as L3 measurements are typically configured by the CU, while L1 measurements are configured by the DU. This separation complicates the management of the Ecat capability for LTM L1 events.

Additionally, after consulting with our product implementation team, it appears that the existing Ecat capability is already fully utilized. Different applications may require configurations with different events, further complicating the potential for sharing capabilities between L3 and LTM events. Given these considerations, introducing a new capability may be necessary to accommodate diverse requirements and ensure efficient operation.

- The existing Ecat capability is already fully utilized in product.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

We think RAN4 first needs to discuss and decide on the granularity of the LTM L1 events capability. Legacy Ecat was defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. Since LTM CSI config may contain multiple cells consisting of both intra-frequency and inter-frequency measurements, we think sharing a combined Ecat between intra and inter-frequency allows the NW flexibility to configure different cells as per the channel or load conditions.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

RAN2 agreed on introducing the following events for the purpose of event triggering measurement reporting.

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

The events are designed to support the event evaluation on the PCell/serving cells and candidate cells. We think RAN4 should consider this when discussing Ecat capability. Since agreeing on actual number requires further analysis, we suggest the actual number to be FFS.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell events, and candidate cells’ events.

Assuming RAN4 agrees on separate reporting criterion for event triggered L1 measurement reporting, there are couple of proposals discussed during last meeting.

- Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

Under Option 1a, the UE can run a maximum of four events simultaneously at any given time. If the 4 events are running at a given time, UE is not required to track remaining event configuration. If the UE supports up to eight candidate cells, and all of them belong to the same frequency layer, the number of events that can be tracked concurrently remains limited to four. Since periodic reporting is also considered one of reporting criteria, the capacity for simultaneous event tracking per frequency layer becomes quite constrained. This limitation restricts the network’s ability to configure distinct event parameters, such as thresholds, for different cells and events.

In contrast, Option 1b enables reporting criteria to be defined per cell. With four events and eight cells and including periodic reporting as an additional criterion, this results in five reporting types per cell. Altogether, the UE can support 5 × 8 = 40 reporting criteria. Since report on leave is also supported for each of the 4 events, additional 4*8 reporting criteria may be needed for reporting on leave. Combining these two criteria makes 72 reporting criteria.

Although this number may initially seem high, it's important to highlight that LTE supports a significantly larger number of reporting criteria than what is proposed here. For reference, LTE’s reporting capabilities are summarized below.

The UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, and inter-RAT per supported RAT (i.e., without counting other categories that the UE shall always support in parallel), the UE need not support more than the total number of reporting criteria as follows: - 26 reporting criteria in total if the UE is not configured with any SCell or PSCell carrier frequency, - 35 reporting criteria in total if the UE is configured with one SCell carrier frequency, - 44 reporting criteria in total if the UE is configured with two SCell carrier frequencies, - 53 reporting criteria in total if the UE is configured with three SCell carrier frequencies, - 62 reporting criteria in total if the UE is configured with four SCell carrier frequencies, - 71 reporting criteria in total if the UE is configured with five SCell carrier frequencies, - 80 reporting criteria in total if the UE is configured with six SCell carrier frequencies, - 35 reporting criteria in total if the UE is configured with one PSCell carrier frequency, and - 44 reporting criteria in total if the UE is configured with one PSCell carrier frequency and one SCell carrier frequency.

- RAN4 to agree 72 as the L1 measurement reporting criteria for a UE supporting event triggered measurement reporting.

### Impact of event triggered reporting to LTM procedures

TCI state known condition

The network may configure the UE with both L3-based events and L1-based events simultaneously. For example, the network may trigger an LTM cell switch based on Event LTM3 and schedule early TCI state activation and PDCCH order-based RACH based on L3-based events. In this case, the current TCI state known conditions need to be updated if the early TCI state activation is based on an L3 measurement report and the cell switch is based on an event-triggered L1 report. This is because the existing known conditions (at least for TCI state activation) are only based on the periodic L1-RSRP report.

Rel-18 TCI state known conditions and the conditions (clause 8.25 of TS 38.133) under which TCI state activation requirements are applicable (clause 6.3.1 of TS 38.133) are summarized in the table below.

Known conditions of the TCI state: The candidate TCI state in the TCI state activation list is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. - The target downlink TCI state remains detectable during the downlink TCI state activation period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period - SNR of the downlink TCI state ≥ -3dB Otherwise, the candidate TCI state is unknown. TCI state activation requirements are applicable for: Known TCI state in FR1 and FR2 specifed in clause 8.25.2, and - Unknown TCI state in FR1 specifed in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

We may need to consider a scenario where event triggered reporting is configured but the event is not triggered. In this scenario, though UE was measuring the RS, since the report was not sent, the TCI state is still considered as unknown based on the existing known condition definition. The cell switch delay requirements are not applicable in this case, as per the Release 18 agreements. To avoid no requirement in this scenario, TCI state known condition defintion needs to be modified. Potential modification is add a sentence saying TCI state is known if the “UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured.

Another open issue discussed is listed below.

- For FR1:

  - UE has sent an event-triggered L1 report, and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

For the case 2, since NW do not know if the event entering condition is still be met at the time of sending the TCI state activation or cell switch command, existing cell known, and TCI state known conditions may not be able to be enhanced.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell events, and candidate cells’ events.

- RAN4 to agree 72 as the L1 measurement reporting criteria for a UE supporting event triggered measurement reporting.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - “The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2420101, WF on NR mobility enhancements Phase 4