3GPP TSG-RAN WG4 Meeting #116	R4-2511401

# Bangalore, India, 25th – 29th August 2025

Agenda item:		7.6.3.2.2

Source: 		Qualcomm Incorporated

Title:	 	NR NTN HPUE Rx requirements

Document for:		Approval

## 1	Introduction

RSD for PC1 and PC1.5 have not been concluded for bands n255 and n256. In this contribution we provide our view on the RSD for non-handheld PC1 UEs.

## 2	Discussion

When output power is increased from 26 to 31 dBm, i.e. from PC2 to PC1, meeting reference sensitivity requirements becomes increasingly difficult. This is due to two separate impacts

- PA emissions are stronger

- Own Tx wanted signal in receiver is stronger, causing higher noise floor due to Rx non-linearity.

Observation 1: Both Tx emissions and Rx non-linearity impact the amount of receiver desensitization.

For the second effect via Rx non-linearity, 2nd order non-linearity critical. Due to 2nd order effect, 1 dB increase in input power level in the non-linear component will result in 2 dB noise increase in the receiver.

Observation 2: In receiver second order non-linearity 1 dB increase in receiver input power results in 2 dB increase of noise floor.

Compared to PC2, it is however immediately so that Tx emissions are 5 dB higher due to 5 dB increase in output power and receiver non-linearity is 10 dB stronger. There are some possibilities to mitigate the impact. For example, non-handheld form factor allows for some possibility to design a larger form factor filters. This can offset part of the Tx noise increase. Similarly, larger form factor can allow for larger antenna isolation, meaning that Tx leakage via the antenna interface is slightly lower and victim receiver non-linearity does not increase by the full 10 dB. However, this help is available only in diversity receivers, which operate via different antenna than the main transmit branch.

Observation 3: Larger device form factor can help to offset some, but not all, of the noise increase both due to Tx and Rx effects.

Transceiver linearity parameters are not expected to be different for non-handheld form factor, i.e. receiver non-linearity (IIP2) is expected to be the same.

Based on our analysis, RSD in Table 1 is proposed for PC1 non-handheld UEs for band n255 and n256.

Proposal 1: Adopt the RSD in Table 1 for PC1 non-handheld NR NTN UE.

Table 1: RSD for PC1 non-handheld NR NTN UE

|  | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n255 | 4 | 4 | 4 | 4 |
| n256 | 3.5 | 3.5 | 3.5 | 3.5 |

## 3	Conclusion

In this contribution RSD for NR NTN HPUE was discussed and following observations and proposals were made:

Proposal 1: Adopt the RSD in Table 1 for PC1 non-handheld NR NTN UE.

Table 1: RSD for PC1 non-handheld NR NTN UE

|  | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n255 | 4 | 4 | 4 | 4 |
| n256 | 3.5 | 3.5 | 3.5 | 3.5 |

## 4	References