3GPP TSG-RAN WG4 Meeting #116	R4-2511404

# Bangalore, India, 25th – 29th August 2025

Agenda item:		7.28.3

Source: 		Qualcomm Inc.

Title:	 	IoT NTN UL Capacity Enhancements

Document for:		Approval

## 1	Introduction

IoT NTN UL capacity enhancement work is on-going in RAN1. RAN4 has had initial discussion on whether any specified scheme results in RAN4 specification impact. In this contribution RAN4 specification impact is further discussed.

## 2	Discussion

While the RAN1 part of the work item is officially complete, RAN1 is still working on the specification in the maintenance. Most likely outcome is that RAN1 may change their specifications on transmission gaps and segmentation when these happen together with OCC. Given that RAN4 requirements only reference RAN1 specifications, there would be no direct impact to RAN4 specifications.

Up to 40 ms gaps are possible in NB-IoT as UE needs to listen to DL signal for 40ms after 256ms of contiguous UL transmissions. Similarly, a gap may happen if UE transmits PRACH instead of PUSCH in a PRACH occasion (known from SIB). Similarly, a gap may happen for a segmented pre-compensation, where UE is meant to update the pre-compensation between the segments. It is recognized in RAN1 that these gaps can cause distortion to the signal, and therefore there is discussion on arranging the OCC so that it does not extend over a gap.

Observation 1: RAN1 is still discussing in maintenance arranging the OCC signals so that they do not extend over a transmission gap.

IoT NTN scheduling is fully controlled by the network. Network decides which UEs it pairs together, how many repetitions are configured and therefore has control over transmission gaps and segmentation. This means also that network has means to avoid worst case configurations.

A requirement to re-synchronize Tx frequency periodically would not be helpful, as the baseline frequency error requirement does not change and it would be sufficient to synchronize to just meet the frequency error. As part of normal operation, UE will synchronize with network and maintain the frequency error.

Observation 2: Network controls which UEs are paired together for OCC, and should do the pairing so that potential negative performance impacts are minimized.

Observation 3: It is part of normal operation of the UE to maintain sync with the network and not violate frequency error requirements.

Furthermore, all NTN IoT evaluations in RAN1 were made without assuming CFO grouping. Gains were still seen and the feature definition is on-going.

Observation 4: For IoT NTN CFO grouping is not assumed in RAN1

As a conclusion, no need has been found to update UE RF requirements based on the current status of RAN1 discussion.

Proposal 1: There is no need to update UE RF requirements based on the current status of RAN1 discussion.

## 3	Conclusion

In this contribution NTN IoT UL capacity enhancements were discussed and following observation and proposal was made:

Observation 1: RAN1 is still discussing in maintenance arranging the OCC signals so that they do not extend over a transmission gap, but work is not complete.

Observation 2: Network controls which UEs are paired together for OCC, and should do the pairing so that potential negative performance impacts are minimized.

Observation 3: It is part of normal operation of the UE to maintain sync with the network and not violate frequency error requirements.

Observation 4: For IoT NTN CFO grouping is not assumed in RAN1

Proposal 1: There is no need to update UE RF requirements based on the current status of RAN1 discussion.

## 4	References

[1]	RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)