3GPP TSG-RAN WG4 Meeting #116	R4-2511405

# Bangalore, India, 25th – 29th August 2025

Agenda item:		6.13.1

Source: 		Qualcomm Inc.

Title:	 	NR NTN L-band UE RF requirements

Document for:		Approval

## 1	Introduction

NR NTN L-band work item is on-going, with emission requirements and A-MPR being the remaining open item. In this contribution we provide our analysis.

## 2	Discussion

RAN4 has received an LS [1] from ETSI TC SES, who is responsible for developing harmonized standards for Europe, capturing the regulation relevant for operation in Europe. The LS captures agreements also for frequency bands n255/255. This is very relevant for the L-band work item, as ETSI requirements for the new L-bands are coming from the same ETSI document as requirements for n255, namely EN 301 681.

Specifically, for n255 ETSI has not identified any Rx requirements that are necessary to be carried over from existing ETSI harmonized standards, and Rx requirements in ETSI shall align with 3GPP requirements. Secondly, for Tx requirements SEM and general spurious emissions shall be aligned with 3GPP, but there is a need to capture in addition band 254 UL frequency range protection and radio astronomy protection. Band 254 protection is based on EN 301 681 and ITU-R M.1480, while radio astronomy protection should be based on 3GPP Tx OFF requirement.

It is also specifically mentioned that same conclusions are expected to apply for bands n253, n251 and n250.

Based on the LS, following requirements should be captured to n253, n251, n250

- SEM mask aligned with 3GPP, no additional SEM mask is needed

- General spurious emission mask aligned with 3GPP

- Additional spurious emissions for the range 1610-1626.5 MHz

As EN 301 681 already includes protection requirements for 1610-1626.5 MHz frequency range, it would be natural to re-use these requirements. ETSI has already used these, and it seems unnecessary to change the requirements for 3GPP-systems.

Proposal 1: Based on ETSI LS, capture following emission requirements for bands n250, n251 and n253

- SEM mask aligned with 3GPP

- General spurious emission mask aligned with 3GPP

- Additional spurious emissions for the range 1610-1626.5 MHz aligned with EN 301 681

Proposal 2: There is no need to copy over additional SEM requirements from EN 301 441, nor additional spurious emissions requirements above or between the ULs of the operating bands

Somewhat surprisingly ETSI did not include requirements for cross-device GNSS protection for frequency range 1559-1610 MHz. It would be useful to discuss if these requirements should be added.

Proposal 3: Further consider whether cross-device GNSS protection for frequency range 1559-1610 MHz is beneficial to include.

For radio astronomy protection current 3GPP requirement applies within the channel bandwidth. It should be discussed whether an additional requirement is needed for the radio astronomy frequencies. On one hand, in Tx OFF state emissions are expected to be lower outside the channel bandwidth, but on the other hand an additional measurement would confirm this.

Proposal 4: Consider whether an additional Tx OFF measurement is needed for the radio astronomy frequency range in addition to channel bandwidth.

An A-MPR study was done considering the following additional requirements

Table 1: Additional emission requirements for NS_10

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. |  |  |  |

There was also an agreement in NTN HPUE discussion to apply 10 dB filter rejection towards 1559-1610 MHz, this filter rejection was used. Otherwise the assumptions were

- PA calibrated to 1 dB MPR with 30 dBc ACLR with DFT-s-OFDM QPSK waveform

- -28 dBc IQ-image

- -28 dBc LO-leakage

- -60 dBc CIM3 and -70 dBc CIM5

Simulation results are provided in the appendix and A-MPR proposed A-MPR tables are below

Table 2: A-MPR regions for NS_10

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | LCRB*12*SCS | A-MPR |
| 5MHz | 1629 <= Fc <= 1632.5 | <= 0.9 |  | A1 |
| 10MHz | 1631.5 <= Fc <= 1636 | <= 2.7 | < 5.4 | A1 |
|  |  |  | >= 5.4 | A1 |
| 15MHz | 1634 <= Fc < 1641 | <= 4.32 |  | A2 |
|  |  | <= 6.12 | >= 7.2 | A2 |
| 20MHz | 1636.5 <= Fc < 1644 | <= 6.12 | <= 3.6 | A3 |
|  |  | <= 3.6 | > 3.6 | A2 |
|  |  | > 3.6 | > RBstart*12*SCS + 3.6 | A1 |

Table 3: A-MPR values for NS_10

|  | Modulation | A1 | A2 | A3 |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | 2 | 3 |
|  | QPSK | 2 | 2.5 | 3.5 |
|  | 16QAM | 2 | 2.5 | 4 |
|  | 64QAM | 2.5 | 3 | 4.5 |
| CP-OFDM | QPSK | 3.5 | 4 | 5.5 |
|  | 16QAM | 3.5 | 4 | 5.5 |
|  | 64QAM | 3.5 | 4 | 5.5 |

Proposal 5: Capture A-MPR requirements to 38.101-5 as in Table 2 and Table 3.

## 3	Conclusion

In this contribution combined L-bands emission requirements and A-MPR were discussed. Following proposals were made

Proposal 1: Based on ETSI LS, capture following emission requirements for bands n250, n251 and n253

- SEM mask aligned with 3GPP, no additional SEM mask is needed

- General spurious emission mask aligned with 3GPP

- Additional spurious emissions for the range 1610-1626.5 MHz aligned with EN 301 681

Proposal 2: There is no need to copy over additional SEM requirements from EN 301 441, nor additional spurious emissions requirements above or between the ULs of the operating bands

Proposal 3: Further consider whether cross-device GNSS protection for frequency range 1559-1610 MHz is beneficial to include.

Proposal 4: Consider whether an additional Tx OFF measurement is needed for the radio astronomy frequency range in addition to channel bandwidth.

Proposal 5: Capture A-MPR requirements to 38.101-5 as in Table 2 and Table 3.

## 4	References

[1]	R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI TC SES

## Appendix

Simulation results, displaying A-MPR due to the additional emission limits