# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511428

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			TP for TR 38.774_WUR RF requirement of Maximum input level

Agenda item:		7.24.3.3

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of IBB, OBB, intermodulation, spurious emissions and others .

# Discussion

In WF[1], the open issue relating to OBB and IBB is quoted below:

Issue 3-1-1: IBB, OBB requirements

Agreements:

- Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal.

Figure 1:WUR selectivity and blocking requirement overview

Figure 1 illustrated the overall receiver requirement adding the signal of both WUR and MR. It can be observed that there is bandwidth difference between WUR and MR. As the frequency offset of the interferer is set in relation of the MR channel BW and as such, if the WUR signal could be placed at the near edge of the MR channel where is adjacent to the interferer, it could be possible to test the WUR with one MR bandwidth only. This is because that when placing the WUS signal at the edge of the channel, there would be only the guard band of the MR channel between the WUS signal and interferer and therefore, the test case with minimum MR channel bandwidth could set the worst-case scenario. Therefore, WUR test could be tested with one MR bandwidth which reduce the test cases for WUR.

It is possible to define one test case for WUR IBB and OOB with min MR channel bandwidth.

For the wanted signal level of WUR, the starting point for WUR is the same allowance of relaxation of MR for IBB and OBB. But this could be confirmed when ACS test case is decided. For additional shifted RB, as IBB is the non-adjacent channel selectivity, there is no need to set the additional guard RB and wanted signal can be placed at the edge of the channel. There is good frequency distance to the IBB jammer and there is no reciprocal mixing issue.

There is no reciprocal mixing for IBB and OOB case for type 1 WUR.

The wanted signal level of WUR, the same allowance of relaxation for both type 1 and 2 WUR for IBB and OBB

# Conclusions

In this contribution, we present our view on WUR RF requirement for IBB, OBB, intermodulation, spurious emissions and others with below proposal:

Observation 1 It is possible to define one test case for WUR IBB and OOB with min MR channel bandwidth.

Observation 2 There is no need to consider the guard or shifted RB for IBB and OOB case.

Proposal-1:The wanted signal level of WUR, the same allowance of relaxation for both type 1 and 2 WUR for IBB and OBB

# References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, vivo