# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511430

# Bangalore, India, August 25-29, 2025

Source:		Ericsson

Title:			On WUR RF requirement testability

Agenda item:		7.24.3.4

Document for:		Approval

# Introduction

In this paper, we present our overview on the WUR RF requirement testability issue.

# Discussion

In WF[1], the open issue for testing aspects is quoted below:

Issue 4-1-1: How to verify LP-WUR

Agreement:

Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods)

- The ultimate test method including the potential test mode will be decided by RAN5

In above agreement, to detect the MSG1/3 in idle mode or ACK/NACK in connected mode, UE needs to detect both LP-WUS and the PDCCH for paging information. If UE miss the detection of the PDCCH, it will be counted as miss detection of the LP-WUS, and this should be avoided in test setup. But in requirement, the miss detection rate should include both LP-WUS and PDCCH as it is not possible to detach these two from requirement. However,  in the test, the condition of the PDCCH channel needs to be specified specifically so the MDR of LP-WUS will be tested.

PDCCH detection aspect is missing from RAN4 agreements.

MDR Requirement should include both PDCCH and LP-WUS as UE not possible to skip PDCCH detection.

PDCCH test for power saving is specified in 38.101-4 and quoted as below for discussion purpose. It can be observed that the Pm-dsg is specified as test metric for Rel-17 PEI feature (wake up signal for idle and inactive in Rel-17).

Table 5.3.2.2.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

To test the requirement of the LP-WUS MDR of 1% and considering the fact that UE will detect LP-WUS and PDCCH sequentially, the PDCCH power level should be set high enough so as the miss detection rate of PDCCH will be far less than 1%, supposing MDR of PDCCH to be 0.1%. In this case, the miss detection rate of detecting two sequential signals (LP-WUS + PDCCH) will be 1-(1-MDRLP-WUS) *(1- MDRPDCCH)=1-0.99*0.999=1.1% and in line with RAN4 agreement above.  In this way, the MDR of LP-WUS can be tested with a WUR not the MDR of PDCCH with the UE’s main receiver.

From the above discussion, there is a need to specify the PDCCH test condition in the LP-WUS testing metric and we suggest to reusing the test configuration in TS 38.101-4 for this purpose.

The PDCCH test condition to be added in WUR RF test metric.

The RAN4 agreement also includes an example of the ACK/NACK in connected mode and MSG1/3 in idle mode, however, there is no such satistic from SS. The test metric of Pm-dsg for PDCCH is defined as the ratio of (statDTX)/(NACK+ACK+statDTX) in TS 38.521-4. StatDTX, ACK and NACK are defined in H2.2 in TS 38.521-2 and quoted as below:

H.2.2	Mapping throughput to error ratio

a)	The measured information bit throughput R is defined as the sum (in kilobits) of the information bit payloads successfully received during the test interval, divided by the duration of the test interval (in seconds).

b)	In measurement practice the UE indicates successfully received information bit payload by signalling an ACK to the SS.
If payload is received, but damaged and cannot be decoded, the UE signals a NACK.

c)	Only the ACK and NACK signals, not the data bits received, are accessible to the SS.
The number of bits is known in the SS from knowledge of what payload was sent.

d)	For the reference measurement channel, applied for testing, the number of bits is different in different slots, however in a radio frame it is fixed during one test.

e)	The time in the measurement interval is composed of successfully received slots (ACK), unsuccessfully received slots (NACK) and no reception at all (DTX-slots).

f)	DTX-slots may occur regularly according the applicable reference measurement channel (regDTX).
In real live networks this is the time when other UEs are served. In TDD these are the UL and special slots.
regDTX vary from test to test but are fixed within the test.

g)	Additional DTX-slots occur statistically when the UE is not responding ACK or NACK where it should. (statDTX)
This may happen when the UE was not expecting data or decided that the data were not intended for it.

The pass / fail decision is done by observing the:

-	number of NACKs

-	number of ACKs and

-	number of statDTXs (regDTX is implicitly known to the SS)

The ratio (NACK + statDTX)/(NACK+ statDTX + ACK)is the Error Ratio (ER). Taking into account the time consumed by the ACK, NACK, and DTX-TTIs (regular and statistical), ER can be mapped unambiguously to throughput for any single reference measurement channel test

To be able to count the MDR of the LP-WUS+PDCCH, the methodology of the Pm-dsg should be used to simplify the test. As the LP-WUS and PDCCH are detected sequentially by WUR and main receiver respectively in WUR RF test, the same test metric for PDCCH in demodulation test can be reused for LP-WUS, the only difference is that the PDCCH signal power should be set high enough so the PDCCH miss detection rate can be ignored in test result after DUT make the sequential detection of the LP-WUS and PDCCH.

The same test metric of PDCCH can be reused for LP-WUS detection.

Applying a certain power level on PDCCH so the miss detection rate of PDCCH can be ignored in co-detection of the LP-WUS and PDCCH.

Besides, the antenna configureation of main receiver and main transmitter (number of Rx and number of Tx) in performance test specification may not be relevant when the PDCCH power is high enough. In this case, we suggest that the SNR as a side condition of the PDCCH signal level. For the test configuration of the PDCCH of main receiver, it is fine to fix the channel bandwidth as the main receiver is not tested from RF perspective.

PDCCH signal level could be set as a side condition, either specifically value or decided by RAN5.

The test metric for WUR RF test and the PDCCH test condition text proposal is below:

The miss detection rate of the LP-WUS together with PDCCH is determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). The Pm-dsg shall be less than 1%.

Table x.y-1: PDCCH test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 . |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 in TS 38.214 |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 in TS 38.214 OP.1 TDD as defined in Annex A.5.2.1 in TS 38.214 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 for FDD For TDD, specific to each TDD UL-DL pattern and as defined in Annex A.1.2 in TS 38.214. |
| CCE to REG mapping type |  |  |  | nonInterleaved |
| REG bundle size |  |  |  | 6 |
| Shift index |  |  |  | 0 |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 for tested channel bandwidth and subcarrier spacing. Note 2: The high layer parameter precoderGranularity equals to sameAsREG-bundle as defined in clause 7.4.1.3 of TS 38.211. |  |  |  |  |

Table x.y-2: Additional test parameter for PDCCH for FDD band

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | <Option 1>: PDCCH signal power level |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD in TS 38.101-4 | AWGN | SNR defined in TS 38.101-4 > [20] dB |
|  | <Option 2>: NOTE: The PDCCH power level should be set high so the miss detection rate of the PDCCH is below 0.1%. |  |  |  |  |  |

Table x.y-3: Additional test parameter for PDCCH for TDD band with 30 kHz SCS

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | <Option 1>: PDCCH signal power level |
| --- | --- | --- | --- | --- | --- | --- |
| 40 | 102 | 1 | 2 | R.PDCCH. 2-1.1 TDD | AWGN | SNR defined in TS 38.101-4 > [20] dB |
| <Option 2>: NOTE: NOTE: The PDCCH power level should be set high so the miss detection rate of the PDCCH is below 0.1%. |  |  |  |  |  |  |

There is another issue relating to the test, that is, it is up to the UE to decide to use WUR or MR according to RAN1 agreement. the relevant requirement is quoted below. The word “may” is used so even when entry/exist condition met, there is no mandate that UE use its WUR.  It can be up to RAN5 to make sure if the WUR is tested, not the MR monitoring the PDCCH during the test. One example is that SS can send the PDCCH for paging only without sending LP_WUS, if UE initiate the connection, it increases either false alarm rate or UE is not using the WUR in such a case. Alternatively, a test mode would be helpful to assure UE is using the WUR. Whether a specific test mode is designed by RAN5 to make sure the WUR always used will be still up to RAN5. At the same time, RAN4 can send a LS to convey such common understanding.

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

LS to RAN5 to convey the RAN4 common understanding regarding the potential WUR behavior.

The proposed LS text is below:

RAN4 understands that it is up to UE to use the WUR or not irrespective the entry/exit condition designed by RAN1 and RAN2, therefore, it is up to RAN5 to design test case that WUR is tested not the main receiver.

# Conclusions

In this contribution, we present our view on WUR RF testability with below proposal:

Observation 1 PDCCH detection aspect is missing from RAN4 agreements.

Observation 2 MDR Requirement should include both PDCCH and LP-WUS as UE not possible to skip PDCCH detection.

Observation 3 The PDCCH test condition to be added in WUR RF test metric.

Proposal-1: The same test metric of PDCCH can be reused for LP-WUS detection.

Proposal-2: Applying a certain power level on PDCCH so the miss detection rate of PDCCH can be ignored in co-detection of the LP-WUS and PDCCH.

Proposal-3:PDCCH signal level could be set as a side condition, either specifically value or decided by RAN5.

Proposal-4:LS to RAN5 to convey the RAN4 common understanding regarding the potential WUR behavior.

# References

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo