3GPP TSG-RAN WG4 Meeting # 116	R4-2511432

# Bangalore, India,  August 25 - 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.101-1: Maximum input power for WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LP-WUS is introduced in Rel-19 and WUR receiver requirement to be added, this CR is adding the maximum input power for WUR |  |  |  |  |  |  |  |  |
| Summary of change: |  | adding the maximum input power for WUR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No WUR requirment for Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 7.4M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* start change Option 1 according to WF */

## 7.4M	Maximum input level for WUR

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the the MDR criterion shall be met with the side conditions in clause [7.1M] with parameters specified in Table 7.4M-1 and with parameters specified in Table 7.4M-2a and Table 7.4M-2b.

Table 7.4M-1: Maximum input level for Type1 LR and Type2 LR

| Rx Parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 3, 5, 10, 15, 20 25, 30, 35, 40, 45, 50 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | -25 |

Table 7.4M-2a: Maximum input level for Type1 LR

| RX parameter | Units | Channel BW (MHz) |
| --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 7, 10, 15, 20 25, 30, 35, 40, 45, 50 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | -25 |
| Guard RB2 | PRB | 1 |
| Power of each non-LPWUS DL NR RB2,3 | dBm | -25 -10log10(NRB,LP-WUS) |
| NOTE 1: Guard RBs are zero-power and exist when NRB,LP-WUS < NRB.Guard RB is configured at the both sides of LP-WUS RB with specified number of RB, NRB is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 and NRB,LP-WUS is defined in Table 5.3.2M-1. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB |  |  |

Table 7.4M-2b: Maximum input level for Type2 LR

| RX parameter | Units | Channel BW (MHz) |
| --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 7, 10, 15, 20 25, 30, 35, 40, 45, 50 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | -25 |
| Guard RB1 | PRB | 0 |
| Power of each non-LPWUS DL NR RB2 | dBm | -25 -10log10(NRB,LP-WUS) |
| NOTE 1: Guard RBs are zero-power and exist when NRB,LP-WUS > NRB. Guard RB is configured at the both sides of LP-WUS RB with specified number, NRB is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 and NRB,LP-WUS is defined in Table 5.3.2M-1. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB |  |  |

/* start change Option 2 (specifying the WUR with MR BW)*/

## 7.4M	Maximum input level for WUR

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the the MDR criterion shall be met with the side conditions in clause [7.1M] with parameters specified in Table 7.4M-1 and with parameters specified in Table 7.4M-2a and Table 7.4M-2b .

Table 7.4M-1: Maximum input level for Type1 LR and Type2 LR

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | -25 | -25 + 10log10(BWChannel /20) | -20 |
| NOTE 1: 10log10(x) is rounded to the nearest 0.5dB value. |  |  |  |  |

Table 7.4M-2a: Maximum input level for Type1 LR

| RX parameter | Units | Channel BW (MHz) |  |  |
| --- | --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 7, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | -25 | -25 + 10log10(BWChannel /20) | -20 |
| Guard RB1 | PRB | 1 | 1 | 1 |
| Power of each non-LPWUS DL NR RB2 | dBm | -25-10log10(NRB,LP-WUS) | -25 + 10log10(BWChannel /20)-10log10(NRB,LP-WUS) | -20-10log10(NRB,LP-WUS) |
| NOTE 1: Guard RBs are zero-power and exist when NRB,LP-WUS< NRB , Guard RB is configured at the both sides of LP-WUS RB with specified number of RB, NRB is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 and NRB,LP-WUS is defined in Table 5.3.2M-1. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB |  |  |  |  |

Table 7.4M-2b: Maximum input level for Type2 LR

| RX parameter | Units | Channel BW (MHz) |  |  |
| --- | --- | --- | --- | --- |
| Applicable channel BW (MHz) |  | 3, 5, 7, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in LP-WUS | dBm | -25 | -25 + 10log10(BWChannel /20) | -20 |
| Guard RB1 | PRB | 0 | 0 | 0 |
| Power of each non-LPWUS DL NR RB1 | dBm | -25-10log10(NRB,LP-WUS) | -25 + 10log10(BWChannel /20)-10log10(NRB,LP-WUS) | -20-10log10(NRB,LP-WUS) |
| NOTE 1: Guard RBs are zero-power and exist when NRB,LP-WUS < NRB , Guard RB is configured at the both sides of LP-WUS RB with specified number of RB, NRB is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1 and NRB,LP-WUS is defined in Table 5.3.2M-1. NOTE2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB |  |  |  |  |

/* end change */