# 3GPP TSG-RAN WG4 Meeting # 116													R4-2511434

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson, Sony

Title:			A-IoT device requirement overview

Agenda item:		7.22.3.2

Document for:		Discussion

# Introduction

In this paper, the TP for backscatter power loss is proposed.

# TP proposal

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

#### 6.2.1.1	UE maximum output power for device type 1

The maximum backscatter loss is defined as the ratio of the input CW power at the device antenna to the backscatter power at the device antenna. The backscatter power is defined as mean filtered power measured over the duration of the D2R signal at the UE declared directionand. The basckscatter loss shall be met in Table 6.2.1.1-1 with the test parameters defined in Annex X, within the partial sphere and the UE declared direction described in Annex Y.

Table 6.2.1.1-1: Maximum backscatter loss

| Operating band | Maximum backscatter loss |
| --- | --- |
| n8 | [10] |

Annex X (normative):

The test configuration for backscattering loss

The test configuration in Table X.1-1 is defined to be used for testing the backscattering loss requirement defined in section 8.1.2

Table X.1-1: Test configuraton

| Test parameter | value |
| --- | --- |
| D2R channel bandwidth (kHz) | (2000*(1+Tb/(2Tc))/Tb)* 1.1/0.9 |
| CW frequency (MHz) | According to TS 38.192 |
| Filter centre frequency | CW frequency |
| Filter frequency offset | 1/Tc |
| Filter bandwidth | 2/Tb *1.1 |

Annex Y (normative):

The partial sphere definition for backscattering loss

The partial sphere within [±45°] angular width degrees centered at the UE declared test direction is illustrated in fig. X.1-1. The angle of CW signal and angle to the reader are both within the the Theta and Phi range from a partial sphere within [±45°] angular width degrees centered at the UE declared test direction.

The center direction of the partial sphere is declared by the UE during the conformance test.

The selection of angle of CW signal and the angle to the reader are selected according to the OTA test setup in section 8.1.2

Figure Y.1-1: the illustration of partial sphere where the backscattering loss core requirement applied