# 3GPP TSG-RAN WG4 Meeting # 116														R4-2511435

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			Tp for 38.191: maximum output power ,device D2R channel bandwidth

Agenda item:		7.22.2

Document for:		Approval

# Introduction

In this paper, the TP for maximum output power and device D2R channel bandwidth is proposed.

# TP proposal

### 5.3.2	D2R Channel bandwidth

The requirements in this specification apply to the combination of channel bandwidths, data bit duration Tb, chip bit duration Tc, and operating bands shown in Table 5.3.2-1.

Table 5.3.2-1: D2R Channel bandwidths for n8 (kHz)

|  | Tc=133.33 | Tc=66.7 | Tc=33.33 | Tc=16.67 | Tc= 8.33 | Tc= 4.17 | Tc= 2.08 | Tc= 1.04, | Tc= 0.69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb= 266.67 | 163.49 | 645.30 | 1370.35 | 2462.23 | 4997.19 | 9833.37 | 19653.92 | 37507.34 | 53652.42 |
| Tb=133.33 |  | 207.46 | 1539.63 | 2462.73 | 5188.09 | 9840.29 | 19662.69 | 37514.09 | 53654.72 |
| Tb= 66.67 |  |  | 359.83 | 2471.47 | 5490.93 | 9886.65 | 19677.76 | 37513.23 | 53663.66 |
| Tb= 33.33 |  |  |  | 604.68 | 6107.25 | 10328.16 | 19751.66 | 37540.43 | 53683.05 |
| Tb=16.7 |  |  |  |  | 1206.55 | 10722.37 | 20666.82 | 38802.00 | 53778.75 |
| Tb= 8.33 |  |  |  |  |  | 2468.70 | 21656.32 | 39436.84 | 56457.06 |
| Tb= 4.17 |  |  |  |  |  |  | 4667.25 | 41445.33 | 56712.04 |
| Tb= 1.39 |  |  |  |  |  |  |  | 14855.72 | 14933.20 |

## 6.2	Transmitter power

### 6.2.1	UE maximum output power

#### 6.2.1.1	UE maximum output power for device type 1

The effective radiated power is the power radiated by its antenna in its direction of maximum gain under specified conditions of measurement. The radiated power shall not exceed -5 dBm within the configured channel bandwidth.