3GPP TSG-RAN WG4 Meeting # 116						R4-2511436

Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		7.22.2

Document for:		Approval

# Introduction

In this paper, we present our views on issues in WF[1].

# Discussion

## System parameter

## Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement in RAN4#115:

-	Define 10KHz channel raster

-	FFS on channel raster offset

Last meeting our paper [1] illustrate the 10kHz raster is fine and observation in [1] does not have any offset needed. We assume the definition of the A-IoT DL channel centre frequency is the same with NR.

If the A-IoT DL channel center frequency has the same definition with NR, there is no offset needed for 10kHz raster.

For in-band operation, reuse the 10kHz enhanced channel raster for A-IoT BS without a need of any raster offset.

Issue 2-2-1: D2R bandwidth

There is RAN4 agreement that no D2R channel raster defined in RAN4. As the A-IoT BS test or CW emission test needs to configure a frequency, we think the D2R raster needs to be defined so the absolute frequency can be configured in A-IoT BS and CW node.  As DL channel raster is defined, we think the UL channel raster can be reused and duplex distance can be considered between UL and DL channel for FDD band.

Reuse the DL channel raster for UL so the D2R channel can be configured with NR-ARFCN for testing

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below observation and proposals:

Observation 1 If the A-IoT DL channel center frequency has the same definition with NR, there is no offset needed for 10kHz raster.

Proposal-1: For in-band operation, reuse the 10kHz enhanced channel raster for A-IoT BS without a need of any raster offset.

Proposal-2: Reuse the DL channel raster for UL so the D2R channel can be configured with NR-ARFCN for testing

# References

- R4-2508116, WF on A-IOT device requirements, CMCC

- R4-2507496, A-IoT general overview, Ericsson