# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511443

# Bengaluru, India, August 25-29, 2025

Source:		Ericsson

Title:			RF imact for OCC for IoT NTN

Agenda item:		7.28.3

Document for:		Approval

# Introduction

In this paper, we present our view on the RF impact for OCC feature specified in RAN1.

# Discussion

In WF[1], the OCC requirement is FFS

#### Issue 2-1: Whether UE is required to maintain phase continuity and power consistency for the duration of OCC group.

- Agreement:

  - FFS on UE RF requirements and whether related UE capabilities would be needed.

    - Take the NR-NTN DMRS bundling requirements as a starting point to evaluate the phase continuity requirements for IoT-NTN with slot level OCC.

The related RAN1 agreement on slot level OCC is below:

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode.

Agreement

For single tone 15kHz SCS Slot-level OCC for NPUSCH format 1, OCC length larger than 2 is not supported.

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot.

The simulation is done for NR NTN for OOC2 performance showing neglecting performance degradation due to CFO, We think there is no need to introduce additional requirement on OCC2 and WF in RAN4#114 is fine.

#### Issue 2-1: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2

- Agreement:

  - No RF impact on OCC2 for IoT NTN.

No RF impact on slot level OCC2.

# Conclusions

In this contribution, we present our view on RF impact of OCC:

Proposal-1:No RF impact on slot level OCC2.

# References

- R4- 2508767, Way Forward for [115][315] IoT_NTN_Ph3,MTK

- R4-2502287, Way Forward for [114][315] IoT_NTN_Ph3,MTK