3GPP TSG-RAN WG4 Meeting #116	R4-2511524

Bengaluru, India, 25th – 29th August, 2025

Agenda item:		7.9.3.2

Source:	Nokia

Title:	On OTA test enhancements

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#115 regarding OTA test enhancements. This contribution further elaborates Nokia’s views on the items related to OTA test enhancements.

# Discussion

## Capturing conclusions on Core aspects

The following was agreed in RAN4#114bis:

Way forward: Previous WF (R4-2502279, RAN4#114) on the timeline for BS-to-BS isolation data collection (i.e., ”conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting”) is amended as follows: - Close BS-to-BS isolation data collection by RAN4#115 (May 2025) to allow drawing initial conclusions. NOTE: Further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded. - For RAN4#115 (May 2025) provide Draft CRs to TR 37.941 with the initial summary of the findings on the following areas (work-split proposed below): - Background and motivation for CLTA-alternatives discussion (Huawei), - BS-to-BS isolation analysis, including related assumptions (Ericsson), - Impact of BS-to-BS isolation analysis on the legacy co-location requirements (ZTE), - CLTA alternatives: horn antenna (Nokia).

As indicated in the WF, we will provide draftCR for CLTA alternatives: horn antenna. A draftCR was available in RAN4#115 but due the lack of time, it was not discussed. The draftCR will be resubmitted for this meeting.

We have been providing wideband horn antenna measurements and analysis for several meetings with different configurations. Wideband horn antennas are extremely versatile to be used as alternative for CLTA. CLTA’s have a big problem when going higher in frequency. CLTA’s are not commercially available, which might lead to a situation where custom-made CLTA’s can be designed to work specifically well with certain DUT. These test antennas are not available for third party testing laboratories. One big problem also regarding CLTA’s is the vast number of needed CLTA’s for different frequencies. This is where wideband horn antennas would be very beneficial to be used as CLTA’s. One wideband horn antenna could cover up to five different CLTA’s.

Wideband horn antennas have a wide range of supported frequency and are available up to 110GHz. There are some basic differences compared to CLTAs:

- Wideband horn antennas are intended to be used directed towards the DUT.

- Wideband horn antennas are different size than CLTA.

- Wideband horn antennas are receiving different frequencies in different depths of the horn.

- Wideband horn antenna structure is semi-open from two sides and closed from two sides or closed from all four sides.

There are many aspects to be considered using wideband horn antennas as CLTAs. Current CLTA arrangements might not be optimal for wideband horn antennas, e.g. same plane orientation, 10cm offset enclosure to enclosure etc. According to the measurements we have done, wideband horn antennas provide very similar results than CLTA for certain frequency ranges but might give too pessimistic results for other frequencies.

Summary of measurements:

2 wideband horn antennas were measured and results were compared to CLTA for the FR1 frequencies, namely SH400 and LB-2080.

- SH400 performs better for lower frequency bands, where the LB-2080 performs better in the higher frequency bands.

- The size of the wideband horn antenna has a significant effect on the results.

- There is no immediate correlation between the coupling loss in the near-field, increasing distance from 10cm to 20cm or even to 50cm doesn’t guarantee higher coupling loss.

- CLTA arrangement as such is not optimal for wideband horn antennas due to the structure of the antenna.

- There is no straightforward one to one translation to use wideband horn antenna as CLTA, with the current CLTA arrangement.

Single column passive antennas are not readily available for all of the required frequency bands. They typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. Therefore, a new method for the co-location tests might be needed.

The above mentioned findings have been summarized in a text proposal:

=================== Start of Text Proposal ===================

### 6.4.4.4	CLTA alternatives - Wideband horn antenna

The use of wideband horn antenna as CLTA was studied in Rel-19. The use of wideband horn antenna has a number of advantages:

- Wideband horn antennas are easily available and cover wide frequency range.

- No need for multiple custom-made test antennas.

- Testing will be significantly simpler and less time-consuming.

There are some basic differences with wideband horn antennas and CLTAs:

- Wideband horn antennas are intended to be used directed towards the DUT.

- Wideband horn antennas are different size than CLTA.

- Wideband horn antennas are receiving different frequencies in different depths of the horn.

- Wideband horn antenna structure is semi-open from two sides and closed from two sides or closed from all four sides.

- After several measurements with CLTA arrangements and also different configurations, it can be concluded that:

- The size of the wideband horn antenna has a significant effect on the results.

- There is no immediate correlation between the coupling loss in the near-field, increasing distance from 10cm to 20cm or even to 50cm doesn’t guarantee higher coupling loss.

- CLTA arrangement as such is not optimal for wideband horn antennas due to the structure of the antenna.

- There is no straightforward one to one translation to use wideband horn antenna as CLTA, with the current CLTA arrangement.

Single column passive antennas are not readily available for all of the required frequency bands. They typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. Therefore, a new method for the co-location tests might be needed.

=================== End of Text Proposal ===================

We have prepared a draftCR with the same text to be endorsed during the meeting.

## BS-to-BS isolation analysis

Regarding the BS-to-BS isolation analysis, the following was agreed in RAN4#115:

Way forward: The following data is considered baseline for the BS-BS isolation conclusions derivation: - No beam steering consideration; intra-band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); ZTE (R4-2504358, M): -54 (SA-to-SA); ZTE (R4-2504358, M): -52 (SA-to-SA); ZTE (R4-2504358, M): -56 (SA-to-SA). Isolation in horizontal domain (dB); ZTE (R4-2504358, M): -31 (SA-to-SA); ZTE (R4-2504358, M): -34 (SA-to-SA) Huawei (R4-2504594, M): -46 Nokia (R4-2507633, M): -35 (A-to-SA); ZTE (R4-2504358, M): -46 (SA-to-SA) Huawei (R4-2507445, M): -50.6 (A-to-SA); Nokia (R4-2507633, M): -55 (A-to-SA). - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling - SA-to-SA: array to sub-array coupling. - Beam steering consideration; intra-band; same BS type: 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB). Isolation in horizontal domain (dB); Ericsson (R4-2506307, S): A-to-SA: -55±15 dB; ZTE (R4-2507511, M): -69 dB (A-to-SA); Ericsson (R4-2506307, S): A-to-A: -77±15; A-to-SA: -70±15. - Beam steering consideration; inter-band: 3.5 GHz ÷ 7 GHz. Isolation in vertical domain (dB). Isolation in horizontal domain (dB); Ericsson (R4-2506307, S): A-to-A: -54±10 dB A-to-SA: -57±10 dB. Based on date captured in bullets above, the following BS-BS isolation data summary is extracted: Intra-band BS-BS isolation results 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); -54; -52; -56. Isolation in horizontal domain (dB); -31; -34; -46; -55. As per previous agreement in R4-2504701 (RAN4#114bis), further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded.

Unfortunately we were not able to conclude measurement results for 4.9GHz before this meeting. However, upon our analysis of the results provided this far, we support the findings from ZTE i.e. isolation in horizontal domain for 4.9GHz agreed to be -46dB.

-46dB can be accepted as isolation in horizontal domain for 4.9GHz.

## Impacted RF requirements

The following was agreed in RAN4#115:

Way forward: - Based on the BS-to-BS isolation data, no RF (including core and conformance test) co-location requirements impact below 4.9 GHz was agreed. - OTA TX IMD: - Impact on the OTA TX IMD requirement is FFS, considering the argument that this is a regulatory requirement. - Provide further inputs to address the testability issue. - Further investigate impact of higher than 43dBm BS output power.

As stated in the WF, there is no expected impact on the requirements for frequencies below 4.9GHz. At least the requirements up to 3.5GHz region should use the existing assumed isolation value and have no impact on the requirements. What would be the actual frequency limit where the requirement changes is up to further study. Regarding the OTA TX IMD, there are still open questions with some listed in the WF. There is a strong common view that the co-location concept could be included in Rel-20 package and we feel that the requirement impact should be handled there. TX IMD is a regulatory requirement and, therefore, not easily removable from the requirements even for higher FR1 frequencies. TX IMD also includes the intra-system requirement that must not be forgotten.

Do not remove any requirements for Rel-19.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- -46dB can be accepted as isolation in horizontal domain for 4.9GHz.

- Do not remove any requirements for Rel-19.

# References

- R4-2508657 “Way Forward for CLTA”, RAN4#115, Malta, MT, 19th-23rd May, 2025.