3GPP TSG-RAN WG4 Meeting #116	R4-2511530

Bengaluru, India, 25th – 29th August, 2025

Agenda item:		7.21.2.2

Source:	Nokia

Title:	On UE-UE CLI problem statement and potential solutions

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#115 [1] regarding UE-UE CLI problem statement and potential solutions. This contribution displays Nokia’s view on this topic.

# Discussion

This contribution provides first an exhaustive analysis of the UE-UE CLI problem and discuss the potential solutions to address the problem. Furthermore, we also present our coexistence simulation results where the UE-UE CLI problem is evaluated.

## UE-UE CLI problem and potential solutions

The following was agreed in RAN4#115:

Way Forward - UE-to-UE CLI problem statement: - The scenario to be considered: - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator) - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference - FFS non-located assumption is always valid or not - detailed description provided in TR38.858 - The case to be considered: - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception - WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem: - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case - Parameters agreed in Rel-18 will be reused unless difference identified - FFS impact from different grid shift values - FFS impact from ACLR model - WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation. - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation. - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any. - WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects: - Benefit(s): - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s) - FFS how to determine one SBFD-aware UE is aggressor UE - The value of X can be chosen from the range of [1, 5] and other values may not be precluded. - How to determine the value of X or the range can be FFS - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1 - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116. - Other methods are not precluded - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc. - Expected RAN4 requirement impact, including - the impact to TS 38.104 - the impact to TS 38.101 series, e.g., Pcmax, etc. - Expected RAN2 requirement impact: - whether new or changed RAN2 signaling(s) is required: - Expected RAN3 requirement impact: - whether new or changed RAN3 signaling(s) is required: - The following table can be used to summarize the above aspects in the future RAN4 meeting: Expected benefit(s); Expected limitation(s); Expected RAN4 requirement impact; Expected RAN2 requirement impact; Expected RAN3 requirement impact. Solution-1. Solution-2. Solution-3. Solution-4. New solution if any.

As described in the WF, the scenario of interest is an intra-operator adjacent channel coexistence between a legacy TDD and new SBFD network. UE-to-UE CLI can occur in such scenario due to UL transmissions of a SBFD-aware UE in SBFD symbols that generate interference towards the DL reception of a UE in the legacy TDD network.

According to the original proposal in [R4-2505459], both networks may be owned by the same operator which could bring some advantages for handling the UE-to-UE CLI. One proposed option (Solution 2) is that the SBFD network adopts the DUD (40%, 20%, 40%) subband arrangement which maximizes the distance between the UL subband to the adjacent channel. While this is correct, it is important to highlight that this and other options are also applicable to the case of two different operators operating in adjacent channel (e.g. for this case, SBFD subband arrangement can be negotiated/agreed among the operators prior to the deployment of the SBFD cell). In other words, whether it is an intra-operator or inter-operator scenario doesn’t bring any significant technical advantages in terms of UE-to-UE CLI handling.

On the scenario of interest for the UE-to-UE CLI problem, Solution-1 to Solution-4 for CLI handling are relevant regardless of whether it is an intra-operator or inter-operator scenario. In other words, the assumption of intra-operator scenario doesn’t bring any significant technical advantages for handling the UE-to-UE CLI compared to the inter-operator scenario studied in Release 18.

Before diving into the different solutions and their corresponding impact on the different working groups, we would like to highlight that the current NR standard does not support the UE-to-UE CLI across channels, which is a pre-requisite for, at least, some of the solutions listed in the WF. Both layer-1 and layer-3 measurements are intended for co-channel UE-to-UE CLI, where a UE can measure the SRS-RSRP or CLI-RSSI. Performing measurements in the adjacent channel is not currently supported and would require additional efforts across multiple working groups.

Current UE-to-UE CLI measurements only support co-channel CLI measurements, and adjacent channel aggressor UE identification is currently not feasible. The introduction of measurements across channels requires additional efforts across multiple working groups.

Now, on the solutions listed on the way forward, starting with Solution 1, the intention is to reduce the maximum UE Tx power for transmissions in SBFD symbols since a small reduction of the fundamental signal power can result in a significant attenuation of the UE’s emissions in the adjacent channel. As the goal is to reduce the CLI between two adjacent-channel networks, the proposed reduction of p-max may need to be applied to most (if not all) the SBFD-aware UEs since neither the network nor the UEs can detect or be aware of a UE-to-UE CLI problem. Even if such UE-to-UE CLI problem could be detected, the fact that the reduced p-max value would be signaled via RRC (re)configuration means that the UE would apply the Tx power offset all the time even if the aggressor or victim UEs change position or are not scheduled at the same time. Considering the typical use of fractional pathloss compensation for the power control and that gNB UL reception during SBFD symbols is subject to higher self- and gNB-to-gNB interference, a transmission from a UE in the cell-edge that transmits with less than the maximum allowed power may in practice not be decodable by the gNB. This basically limits the ‘coverage benefit’ of SBFD which was the main motivation to study and specify SBFD in the first place. This was also highlighted in the simulation results provided by Qualcomm in R1-2504391, showing that 20 to 30 dB of power backoff is required which translates to very poor UL performance (15 to 25 dB worse coverage than legacy TDD).

In contrast, a more dynamic and simpler solution would be to control the UE Tx power based on gNB scheduling implementation, i.e. without standards impact (we assume this to be part of Solution 3). For instance, the standard already allows to configure different sets of power control parameters (p0, alpha) for UL transmissions in SBFD and non-SBFD symbols. For transmissions in SBFD symbols, the gNB can select a suitable set of (p0, alpha) parameters and additionally select a reasonable number of UL PRBs such that the UE doesn’t reach the maximum transmit power. The gNB may furthermore use the UE’s power headroom report (PHR) to know how far or close the UE is from reaching the maximum transmit power. Note that, contrary to Solution 1, Solution 3 allows to dynamically adapt the UE transmission power e.g. from slot to slot and/or based on the current coverage conditions of the UE, and doesn’t require any standardization impact.

Last but not least, Solution 4 consists of configuring the aggressor UE fallback to TDD operation (up to BS implementation). We understand this means in practice that only DL reception would be allowed for a SBFD-aware UE during SBFD symbols. This could be seen as a sub-variant of Solution 3 where the gNB basically reduces the UE transmit power to 0 watts e.g. due to the presence of a very severe UE-to-UE CLI problem.

Finally, we summarize the pros/cons of the different solutions Table 1. Note that the benefits and limitations in Table 1 are presented under the assumption that adjacent channel measurements and/or identification of aggressor UEs is possible, even though they are not supported in the current standard.

Table 1: Pros/Cons of solutions for handling UE to UE CLI.

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-1 (Reduced p-max in SBFD symbols) | + Reduces UE to UE CLI | -- power reduction is configured via RRC; i.e. applied for all UL transmissions in SBFD symbols even if the aggressor or victim UEs change position or are not scheduled at the same time -- Significant reduction of UL coverage or cell-edge performance -- Large specification impact compared to other solutions | At least TS 38.101 | Signaling of new p-max parameter (either as an offset or absolute value) |  |
| Solution-2 (DUD configuration) | + Reduces UE to UE CLI + No specification impact | -- On its own, this solution might be insufficient to completely solve the UE-to-UE adjacent CLI problem | None | None |  |
| Solution-3 (By proper scheduling of the aggressor UE, up to BS implementation) | + Reduces UE to UE CLI + UL transmission power can be dynamically adjusted based on UE position in the cell and potential presence of victim UE + No specification impact | -- Reduction of UL coverage or cell-edge performance depending on the level of power reduction | None | None | Exchange of information across cells of UE-to-UE CLI measurement results |
| Solution-4 (Configure the aggressor UE fallback to TDD operation, up to BS implementation) | + Avoids completely UE to UE CLI | -- no SBFD UL coverage benefit (i.e. same or worse performance than legacy TDD) | None | None | Exchange of information across cells of UE-to-UE CLI measurement results |
| New solution if any |  |  |  |  |  |

## Simulation assumptions

After RAN4#115, an email discussion was triggered to align the simulation assumptions among the companies. The agreed scenario to be simulated is aligned with the urban-hotspot scenario defined in TR 38.858 Annex E. During the study item, this scenario was identified as the most challenging in terms of UE-to-UE CLI given that UEs from different operators are placed in clusters of 25-meter radius. Companies agreed as well to re-use the Rel-18 parameters defined in Annex E and to use the baseline values for those parameters with multiple options.

The urban-hotspot scenario is the scenario used to evaluate the UE-to-UE CLI and the potential solutions.

There are some FFS captured in the WF related to simulation assumptions. The first FFS is on the grid-shift values. In our view, since this is about UE-to-UE CLI, the relevance of the grid-shift is expected to be minimal. We acknowledge that the position of the cluster centers is constrained by the distance towards the deployed base stations, but the impact should be minimal. Therefore, to reduce the simulation effort we should focus on 100% grid shift.

Assume a single value of 100% grid shift for the UE-to-UE CLI evaluations to reduce the simulations effort.

Another FFS is on the impact of the UE ACLR model. During the email discussion, companies agreed to simulate two different models. The first model, denoted as flat model, assumes a constant ACLR of 30 dBc. This model is the one defined for the Rel-18 coexistence studies. An additional model, which is frequency dependent, has also been proposed. This is denoted as step-wise model and proposes that the ACLR increases depending on the frequency distance between the victim and aggressor allocations. This latter model is defined in TR 36.942.

For the step-wise model and given that we assume that the SBFD network operates in a 100 MHz channel with [DUD] configuration, the frequency separation between the UL subband and the first DL resource of the victim network is of 60 MHz, i.e., 3 times the size of the victim’s allocation. According to TR 36.942, this separation corresponds to an ACLR value of 50 dBc (ACLR3), which we will use in our simulations.

Use flat ACLR model of 30 dBc and a step-wise ACLR model of 50 dBc for evaluating the UE-to-UE CLI

Similar discussion can be applied to the ACS value. Thus, a flat model of 33 dBc and a step-wise model are simulated. For the step-wise model, companies have derived the ACS value as an average between ACS1 and ACS2 given that the UL subband is in the middle between ACS1 and ACS2. Since ACS1 is 33 dBc and ACS2 is 34.3 dBc, the average is 33.6 dBc.

Use a flat ACS model of 33 dBc and a step-wise ACS model of 33.6 dBc for evaluating the UE-to-UE CLI.

## Simulation results

Our coexistence simulations use both ACLR/ACS models to evaluate the UE-to-UE CLI problem. Moreover, as suggested by the WF, different UE max. transmit power reductions which resemble the effect of Solution 1 above are simulated. We adopted a power reduction of [0, 1, 3, 6] dB for all SBFD UEs, and assessed the effect of such power reductions on the UE-to-UE CLI as well as on the SBFD operator UL performance.

As shown in Table 2, the UE-to-UE CLI problem is visible for any of the assumed configurations. UE-to-UE CLI is present especially in situations where a TDD DL UE and a SBFD UE are dropped in the same cluster. In such case, and because there are no indoor losses between the 2 UEs, the effect of path-loss (around 60-70 dB) and ACIR (28.2 dBc) is not enough to bring the UE-to-UE CLI below the noise floor. Note that, on many occasions, SBFD in clusters will transmit with high transmit power since they are indoor UEs and their propagation losses to the serving cell can be high.

The table also shows the positive effect of the step-wise and power reductions since both help on decreasing the impact of the UE-to-UE CLI. However, the UE-to-UE CLI is still a concern for both 5th percentile and mean DL throughput even with a 6 dB power reduction. The only configuration that ensure a DL degradation lower than 5% is when a power reduction of 6 dB and step-wise model is used.

In an urban-hotspot scenario, the presence of UE-to-UE CLI in the TDD DL operator due to its adjacent SBFD operator degrades the DL performance.

Table 2. Summary of the results with focus on the UE-to-UE CLI problem

We have also analyzed the impact of the power reduction on the UL performance of the SBFD network, as shown in Figure 1. As expected, the effect of reducing the maximum UE transmit power is visible on the UL spectral efficiency of SBFD. Compared to baseline (i.e., 0 dB power reduction), applying a Pmax reduction of 6 dB implies a spectral efficiency reduction of 19%. Note that applying this power reduction is not enough to maintain the DL throughput degradation below 5% according to our results in Table 1. Our expectation is that an even higher power reduction is required to ensure that UE-to-UE CLI is kept within acceptable margins.

Figure 1. CDF of the UL spectral efficiency [bits/Hz] for the SBFD network at different UE transmit power reductions.

Reducing Pmax for SBFD UEs up to 6 dB is impacting the performance of SBFD while not fully solving the UE-to-UE CLI problem.

# Conclusion

In this contribution, our views on the UE-UE CLI problem and potential solutions are presented. The following observations and proposals were listed:

- On the scenario of interest for the UE-to-UE CLI problem, Solution-1 to Solution-4 for CLI handling are relevant regardless of whether it is an intra-operator or inter-operator scenario. In other words, the assumption of intra-operator scenario doesn’t bring any significant technical advantages for handling the UE-to-UE CLI compared to the inter-operator scenario studied in Release 18.

Current UE-to-UE CLI measurements only support co-channel CLI measurements, and adjacent channel aggressor UE identification is currently not feasible. The introduction of measurements across channels requires additional efforts across multiple working groups.

The urban-hotspot scenario is the scenario used to evaluate the UE-to-UE CLI and the potential solutions.

In an urban-hotspot scenario, the presence of UE-to-UE CLI in the TDD DL operator due to its adjacent SBFD operator degrades the DL performance.

Reducing Pmax for SBFD UEs up to 6 dB is impacting the performance of SBFD while not fully solving the UE-to-UE CLI problem.

- Assume a single value of 100% grid shift for the UE-to-UE CLI evaluations to reduce the simulations effort.

Use flat ACLR model of 30 dBc and a step-wise ACLR model of 50 dBc for evaluating the UE-to-UE CLI.

Use a flat ACS model of 33 dBc and a step-wise ACS model of 33.6 dBc for evaluating the UE-to-UE CLI.

# References

- R4-2508708 “Way Forward on UE-UE CLI problem statement and potential solutions”, RAN4#115, Malta, 19th-23rd May, 2025