3GPP TSG-RAN4 Meeting #116	R4-2511531

# Bengaluru, India,  25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.104 - In-band blocking requirements for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducing the requirements assumption as deployment guidance for In-band blocking requirement for FR1 WA BS to TS 38.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 12.3.4 In-band selectivity and blocking for SBFD 12.6.5 OTA In-band selectivity and blocking for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements assumption might lead to erroneus deployment guidance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.4, 12.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

### 12.3.4	 In-band selectivity and blocking for SBFD

12.3.4.1	General

The FR1 WA SBFD BS in-band blocking requirements are derived based on an urban macro scenario with 500 meters inter-site distance between SBFD BSs. The adjacent TDD DL network has the same inter-site distance. The SBFD BSs and its adjacent channel network are equally spaced with a distance of 288 meters, which corresponds to a grid-shift of 100% between the 2 networks. The assumption results in the best-case scenario from an interference point of view as the coupling loss between the 2 networks is maximized. The in-band blocking power is expected to be higher with lower inter-site distances. The in-band blocking requirements are derived without accounting for SBFD BS self-interference and inter-sector interference, which would also impact the receiver performance since they are also received by the BS LNA. This requirement is derived based on coexistence simulations and the relevant assumptions can be found in Annex E TR 38.858. For those parameters captured with multiple options, baseline and 1st priority values were assumed.

<End of change 1>

# <Start of change 2>

### 12.6.5	 OTA In-band selectivity and blocking for SBFD

12.6.5.1	General

The FR1 WA SBFD BS in-band blocking requirements are derived based on an urban macro scenario with 500 meters inter-site distance between SBFD BSs. The adjacent TDD DL network has the same inter-site distance. The SBFD BSs and its adjacent channel network are equally spaced with a distance of 288 meters, which corresponds to a grid-shift of 100% between the 2 networks. The assumption results in the best-case scenario from an interference point of view as the coupling loss between the 2 networks is maximized. The in-band blocking power is expected to be higher with lower inter-site distances. The in-band blocking requirements are derived without accounting for SBFD BS self-interference and inter-sector interference, which would also impact the receiver performance since they are also received by the BS LNA. This requirement is derived based on coexistence simulations and the relevant assumptions can be found in Annex E TR 38.858. For those parameters captured with multiple options, baseline and 1st priority values were assumed.

<End of change 2>