3GPP TSG-RAN WG4 Meeting #115                                                               R4-2511538

Bengaluru, India, August 25th – 29th, 2025

Title:	Discussion on SBFD–SBFD Adjacent-Channel Coexistence Scenario

Source:	ISSDU

Agenda item:	7.21.2

Document for:	Discussion

Introduction

In RAN4#115, the group discussed whether and how to introduce specific requirements for SBFD–SBFD co-existence cases and reached the following consensus [1]:

Issue 1-1: Whether SBFD-SBFD co-ex case should be in Rel-19 scope - Way forward - RAN4 do not introduce the specific requirements for SBFD-SBFD co-ex case for FR1 WA or MR BS in Rel-19. SBFD-SBFD co-ex case can be considered for Rel-20.

Based on this consensus, this contribution investigates possible SBFD–SBFD co-existence configurations. Previous work proposed using the downlink sub-band of the operator in the adjacent frequency band as a guard-band, but only analysed the Downlink–Uplink/Uplink–Downlink (DU/UD) scenario. Here, we extend the study to include other feasible configurations that use the downlink sub-band of the adjacent-band operator as a guard-band to improve adjacent-channel isolation and reduce mutual interference. The goal is to evaluate the feasibility of these configurations and support more flexible deployment options.

Discussion

In the context of SBFD–SBFD adjacent-channel coexistence, simulation results show that, to constrain uplink (UL) throughput loss to within 5%—a threshold generally acceptable to most operators—the adjacent-channel interference ratio (ACIR) must be enhanced by approximately 36 dB for a 10% grid-shift, 28 dB for a 20% grid-shift, and 16 dB for a 50% grid-shift. Reference [2] further proposes leveraging the adjacent operator’s downlink (DL) sub-band as a guard-band, thereby achieving the required ACIR improvement without allocating additional guard-band resources or introducing extra implementation complexity.

Based on the foregoing approach, nine adjacent-band SBFD sub-band configuration pairs can be enumerated:
UD/DU, DU/DU, UD/UD, DU/UD, DUD/DU, DUD/UD, DU/DUD, UD/DUD, and DUD/DUD.

Observation 1: The rightmost sub-band of the left-hand operator must be DL ⇒ its configuration must end in “D” (i.e., UD or DUD).

Since uplink transmit power is substantially lower than downlink, our analysis will focus exclusively on protecting UL throughput. To realize the proposed guard-band filtering scheme, the boundary between two neighboring operators’ frequency allocations must consist of DL sub-bands, so that each guard-band filter can leverage the adjacent operator’s DL sub-band as its buffer.

Observation 2: The leftmost sub-band of the right-hand operator must be DL ⇒ its configuration must begin with “D” (i.e., DU or DUD).

Proposal 1: we propose four deployment configurations that satisfy the DL–DL boundary requirement:

- UD / DU

- UD / DUD

- DUD / DU

- DUD / DUD

The above configurations can be accommodated using existing solutions, whereas other configurations will require further study in future discussions. RAN4 to explicitly define the minimum performance requirements for RF filters—ensuring sufficient adjacent-channel interference ratio (ACIR) improvement even under less demanding deployment scenarios—by, for example, preventing downlink placement at critical spectrum edges or reserving additional guard-band resources. It is noted that no such minimum RF filter requirements have been specified in previous meetings.

UD/DU scenario

Compared to DUD-based configurations, the SBFD–SBFD co-existence UD/DU deployment features an enlarged downlink sub-band at the shared boundary, which provides the adjacent operator with a wider guard-band buffer and thereby permits the use of RF filters with more relaxed attenuation and transition-band requirements.

Observation 3: The DL sub-bands of both operators are relatively wide, so the RF filter requirements are less stringent compared to the DUD case.

However, if a third-party operator deploys an uplink sub-band at either outer spectrum edge, the system remains vulnerable to severe UL throughput degradation. Accordingly, we recommend imposing additional constraints on sub-band allocations at both outer boundaries—either to preclude downlink placement or to reserve supplementary guard-band resources.

Observation 4: In the UD/DU configuration, both UL sub-bands are located on the outer sides.

Proposal 2: RAN4 to restrict the frequency allocations of the third-party operator on both adjacent sides in the UD / DU scenario.

UD/DUD scenario

In the SBFD–SBFD co-existence DUD configuration, the uplink sub-band is sandwiched between two downlink sub-bands, causing each individual DL sub-band to be relatively narrow. Consequently, the adjacent operator must deploy RF filters with steeper transition-band requirements—covering adjacent-channel attenuation, transition-band steepness, insertion loss, and group-delay flatness—such that they meet the most stringent case, i.e., when adjacent to a DUD configuration. Compared to the UD/DU scenario, the DUD arrangement inherently isolates the UL sub-band from neighboring DL sub-bands on both sides, eliminating the need for downlink placement constraints on a third-party operator at the right-hand spectrum boundary.

Observation 5: The right-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations.

However, restrictions or guard-band reservations must still be applied to any third-party operator at the left-hand spectrum boundary, particularly to avoid DL placement adjacent to the UL sub-band.

Observation 6:
The UD configuration still requires restrictions on the adjacent third-party operator, because the UL sub-band is located on the outer edge.

Proposal 3: RAN4 to restrict the frequency allocations of the third-party operator on the left adjacent side in the UD / DUD scenario.

DUD/DU scenario

The DUD/DU and UD/DUD scenarios are analogous, differing only in that the DUD configuration is deployed in the lower-frequency band. In this arrangement, the left-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to DU and UD configurations, requiring RF filters with steeper transition-band characteristics—covering adjacent-channel attenuation, transition-band steepness, insertion loss, and group-delay flatness—such that they meet the most stringent case when adjacent to a DUD configuration. The DUD configuration places DL sub-bands on both sides, which inherently avoids interference to neighboring UL sub-bands on the left boundary.

Observation 7: The left-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations.

However, the DU configuration on the right side still positions its UL sub-band at the outer edge, making it vulnerable to interference. Therefore, downlink placement restrictions or guard-band reservations should be applied for any third-party operator at the right-hand spectrum boundary.

Observation 8: The DU configuration requires restrictions on the adjacent third-party operator, because its UL sub-band is located on the outer edge.

Proposal 4: RAN4 to restrict the frequency allocations of the third-party operator on the right adjacent side in the DUD / DU scenario.

DUD/DUD scenario

In the DUD/DUD co-existence scenario, both operators must adopt more stringent RF filter specifications—covering adjacent-channel attenuation, transition-band steepness, insertion loss, and group-delay flatness—such that they meet the most stringent case when adjacent to a DUD configuration. This is because each DL sub-band in the DUD arrangement is relatively narrow compared to DU and UD configurations.

Observation 9: Both side operators use the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations.

However, the inherent characteristic of the DUD configuration—where the uplink sub-band is naturally sandwiched by downlink sub-bands—eliminates the need to impose additional placement restrictions on third-party operators at either the left- or right-hand spectrum boundaries, thereby enhancing deployment flexibility.

Observation 10: No special restrictions are needed on the adjacent third-party operators’ frequency allocations.

Proposal 5: RAN4 to specify that no uplink restrictions from third-party adjacent-band operators are required in the DUD / DUD scenario.

Conclusion

Considering the need for a universal RF filtering solution across all SBFD–SBFD co-existence scenarios, we recommend specifying RF filter characteristics that meet the most stringent performance criteria derived from the DUD/DUD configuration. In this contribution, we have the following observations and proposals for SBFD–SBFD Adjacent-Channel Coexistence Scenario:

Observation 1: The rightmost sub-band of the left-hand operator must be DL ⇒ its configuration must end in “D” (i.e., UD or DUD).

Observation 2: The leftmost sub-band of the right-hand operator must be DL ⇒ its configuration must begin with “D” (i.e., DU or DUD).

Observation 3: The DL sub-bands of both operators are relatively wide, so the RF filter requirements are less stringent compared to the DUD case.

Observation 4: In the UD/DU configuration, both UL sub-bands are located on the outer sides.

Observation 5: The right-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations.

Observation 6: The UD configuration still requires restrictions on the adjacent third-party operator, because the UL sub-band is located on the outer edge.

Observation 7: The left-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations.

Observation 8: The DU configuration requires restrictions on the adjacent third-party operator, because its UL sub-band is located on the outer edge.

Observation 9: Both side operators use the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations.

Observation 10: No special restrictions are needed on the adjacent third-party operators’ frequency allocations.

Proposal 1: we propose four deployment configurations that satisfy the DL–DL boundary requirement:

- UD / DU

- UD / DUD

- DUD / DU

- DUD / DUD

Proposal 2: RAN4 to restrict the frequency allocations of the third-party operator on both adjacent sides in the UD / DU scenario.

Proposal 3: RAN4 to restrict the frequency allocations of the third-party operator on the left adjacent side in the UD / DUD scenario.

Proposal 4: RAN4 to restrict the frequency allocations of the third-party operator on the right adjacent side in the DUD / DU scenario.

Proposal 5: RAN4 to specify that no uplink restrictions from third-party adjacent-band operators are required in the DUD / DUD scenario.

- References

[1] R4-2508713, “Way Forward for [115][307] NR_duplex_evo_BSRF”, Huawei, RAN4#115

[2] RP-251531, “Discussion on SBFD-SBFD adjacent channel operation (Release 18)”, Huawei, RAN#108