3GPP TSG-RAN WG4 Meeting RAN4#116      				R4-2511539 Bengaluru, India

August 25th – 29th, 2025

Agenda Item:	6.8.5

Source:	Lenovo

Title:	On Uplink Tx Switching for HPUE

Document for:  Approval

- Introduction

# In RAN4#115, a CR [1] was submitted to enable HPUE to be used in the same circumstances as is currently allowed for downlink CA with a single uplink CC.  The relevant text is the following from TS 38.101-1 [2] Section 6.2A.0:

As an exception, HPUE applicability notes in 5.5A do not restrict power class indication for single configured UL CC with DL CA. UE can indicate power class for the single configured UL CC with DL CA as specified in Table 6.2.1-1 and if UE supports UL MIMO in this carrier, UE can indicate power class for the CA configuration as specified in Table 6.2D.1-1.

# The proposed change was to add the following text:

As an additional exception, HPUE applicability notes in 5.5A do not restrict the power class indication for each configured UL CC when the UE is configured for CA with uplink Tx Switching as specified in clause 6.3A.3.3 and uplinkTxSwitchingOption is configured for switchedUL without concurrent transmission of UL carriers.

# With this change, the UE would be allowed to use HPUE for each UL CC when the UE is configured for UL Tx switching with switched UL in the same cases in which it allowed for single uplink CC.  The proposed change was not agreed due to concerns including questions on the need for the change and on whether it would be possible to signal the single band power classes.

- Power Class for uplinkTxSwitchingOption Configured for switchedUL

# The downlink band combinations defined for carrier aggregation are the same regardless of whether uplink Tx switching is configured for the uplink.  The notes in the tables in 5.5A indicate the uplink configurations for which MSD is defined and thus HPUE is allowed.  As an example, consider the following two combinations taken from Table 5.5A.3.1-1a of TS 38.101-1.  In the first NR CA configuration, CA_n2A-n77A, the single uplink carrier n2 has Note 8 so that PC2 is applicable:

# NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

# The single uplink carrier n77 has Note 9 so that PC 1.5 is applicable:

# NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

# For the uplink CA configuration CA_n2A-n77A, Note 8 again applies which includes the wording “or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination.” This part of the note is unclear and seems to be intended for the case in which the uplink configuration is n2 only or n77 only and had to be included to allow the same Note 8 to be used for uplink configurations n2, n77 and CA_n2A-n77A. It does not seem to be intended to allow PC2 for n2 and PC2 for n77 with uplink Tx switching configured for switched uplink.

Table 5.5A.3.1-1a: NR CA configurations and bandwidth
combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| ··· | ··· | ··· | ··· | ··· |
| CA_n2A-n77A | n28 n778,9 CA_n2A-n77A8,13,14 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| ··· | ··· | ··· | ··· | ··· |
| CA_n25A-n41A | n258 n418,9 CA_n25A-n41A8, 13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| ··· | ··· | ··· | ··· | ··· |

The following notes are applied to the above tables: (only relevant power class notes are shown)

···

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

···

# Similarly, for this same downlink/uplink configuration, CA_n2A-n77A/ CA_n2A-n77A, there is nothing in the notes that indicates that PC 1.5 is allowed for the n77 uplink carrier in the case of uplink Tx switching configured for switched uplink.

# The same situation exists for the second NR CA configuration in the table below, CA_n25A-n41A.  Note 8 indicates that PC2 can be used when the uplink configuration is n25. Similarly, Note 9 indicates that PC 1.5 is allowed when the uplink configuration is n41. However, for the downlink/uplink configuration CA_n25A-n41A/CA_n25A-n41A, there is nothing in the notes to indicate that allows PC2 for n25 and PC2 for n41 with uplink Tx switching configured for switched uplink, or that PC 1.5 is allowed for the n41 uplink carrier with uplink Tx switching configured for switched uplink.

# To indicate that PC 2 is allowed for both n2 and n77 with uplink Tx switching configured for switched uplink, Note 8 could be modified as follows:

# Proposal 1:  	Modify Note 8 in Table 5.5A.3.1-1 of TS 38.101-1 as follows:

# NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band, for single uplink carrier with up to 2Tx antenna connectors in this downlink combination, or in this downlink combination with uplinkTxSwitchingOption configured for switchedUL.

# Alternatively, and maybe preferably, Note 8 can be split into two notes with one note for uplink configuration n2 and one note for uplink configuration CA_n2A-n77A.

# To indicate that PC 1.5 is allowed for n77 in these two scenarios with uplink Tx switching configured for switched uplink, Note 9 could be modified as follows:

# Proposal 2:  	Modify Note 9 in Table 5.5A.3.1-1 of TS 38.101-1 as follows:

# NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink combination or in this downlink combination with uplinkTxSwitchingOption configured for switchedUL.

# There are notes in the Tables 5.5A.3.1-2, 5.5A.3.1-3, and 5.5A.3.1-4 corresponding to Note 8 and Note 9 in Tables 5.5A.3.1-1 which should be modified as in Proposal 1 and Proposal 2, respectively. The notes in the later tables have some slight differences in wording which should be resolved for consistency.

# Proposal 3:  	Modify the notes in the Tables 5.5A.3.1-2, 5.5A.3.1-3, and 5.5A.3.1-4 corresponding to Note 8 and Note 9 in Tables 5.5A.3.1-1 as in Proposal 1 and Proposal 2, respectively.

#### Summary

# The downlink band combinations defined for carrier aggregation are the same regardless of whether uplink Tx switching is configured for the uplink.  The notes in the tables in 5.5A indicate the uplink configurations for which MSD is defined and thus HPUE is allowed.  In these tables, PC 2 and PC 1.5 is only allowed for cases where the uplink is configured for a single uplink band.  As a result, it seems that the tables do not allow for PC 2 and PC 1.5 to be used for with uplinkTxSwitchingOption configured for switchedUL.  As a result, we propose that the notes be modified as in the proposals below.

# Proposal 1:  	Modify Note 8 in Table 5.5A.3.1-1 of TS 38.101-1 as follows:

# NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band, for single uplink carrier with up to 2Tx antenna connectors in this downlink combination, or in this downlink combination with uplinkTxSwitchingOption configured for switchedUL.

# Proposal 2:  	Modify Note 9 in Table 5.5A.3.1-1 of TS 38.101-1 as follows:

# NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink combination or in this downlink combination with uplinkTxSwitchingOption configured for switchedUL.

# Proposal 3:  	Modify the notes in the Tables 5.5A.3.1-2, 5.5A.3.1-3, and 5.5A.3.1-4 corresponding to Note 8 and Note 9 in Tables 5.5A.3.1-1 as in Proposal 1 and Proposal 2, respectively.

# References

- R4-2507790, CR to TS 38.101-1: HPUE for Uplink Tx Switching with switchedUL, RAN4#115, May 20205, Lenovo, T-Mobile USA, Ericsson.

- TS 38.101-1 NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone; v19.2.0.