3GPP TSG-RAN WG4 Meeting #116	R4-2511554

Bengaluru, India, 25 – 29 August, 2025

Title: 	Further discussion on Tx spurious emissions co-existence limits re-evaluation

Agenda Item:	7.9.3.3

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

In RAN4#115 the WF [1] was approved and it was agreed to continue investigating both the MCL and the system simulation approach (with a 99th percentile).

In our previous papers [2] we have investigated correlated and uncorrelated gain scenarios for the antenna gains and applied then to the MCL scenario. Using the worst case assumptions we have argued that the existing co-existence requirements represent an absolute worst case. It is clear however that due to the dynamic nature of the AAS radiation pattern interference is a statistical parameter as opposed to its fixed nature when considering non-AAS systems, as such it could be argued that a 99th percentile interference level could be more appropriate.

In this paper we provide updates on the Tx spur co-ex emission limits re-evaluation, investigating further into how a 99th percentile interference requirement could be derived..

# 2	Discussion

## 2.1	Evaluation methodology

Worst case analysis in [2] was based on correlated out of band antenna gain assumptions and the aggressor beams pointing directly at the victims.

In the case of BS to BS interference this resulted in a worst-case calculation which was potentially 3dB more relaxed than existing requirement.

In the case of the BS-UE interference the level was approximately the same as the existing specification however as both the victim UE and the aggressor beam direction are variable the worst-case assumption was perhaps even lower probability than the BS-BS case.

If we consider a statistical system simulation it’s clear that the 99th percentile will be lower (i.e., relaxed) than these worst case assumptions.

## 2.2.	BS-BS

As BS are in fixed locations it is obvious that the path loss between them will not change; BS are assumed to be placed above building level.

### 2.2.1	Statistical analysis

The analysis in [3] suggests a 19-site full wrap-around network simulation, however for co-location in the same geographical area it’s not clear this is necessary (or appropriate). Neither is it clear if the interference metric at the victim should be the sum of all the BS, or just the closest one (as is the case with the MCL approach).

To study the effect of beam forming and UE location on the co-existence in same geographical area interference it is necessary only to examine a single cell with victim and aggressor BS and associated UE’s. The beam directions are based on the UE location.

The aggressor frequencies have been selected such that they are harmonics of the victim frequency, as such there is a possibility that the emission will be correlated, it can also be argued that emission so far out of band and will be un-correlated. This is an ongoing discussion in a number of forums and not to be answered here, as such both correlated and un-correlated assumptions have been studied. For both correlated and uncorrelated it is assumed that the sub-array uses fixed passive phase shifters and as such the sub-array patterns are always correlated.

The victim antenna beam pattern operating at the victim frequency is by definition correlated in all cases.

#### 2.2.1.1 Antenna Array models

The victim is a 6.4GHz antenna, the aggressors are:

fundamental frequency (6.4GHz),

-  2nd harmonic (3.2GHz), and

-  3rd harmonic (2.1GHz)

and are described in figure 2.2.1.1-1.

Table 2.2.1.1-1: Antenna Array models

|  |  | 2.1GHz | 3.2GHz | 6.4GHz | unit |
| --- | --- | --- | --- | --- | --- |
| elevation | No elements/sub-arrays | 4 | 4 | 8 |  |
|  | elements per sub-array | 3 | 3 | 3 |  |
|  | element separation | 0.7 | 0.7 | 0.7 | λ |
| Azimuth | No elements | 8 | 8 | 16 |  |
|  | element separation | 0.5 | 0.5 | 0.5 | λ |
| Directivity (zero steer) |  | 26.3 | 26.3 | 32.4 | dBi |
| Vertical Beam width |  | 6 | 6 | 3 | deg |
| Horizontal Beam width |  | 12.7 | 12.7 | 6.4 | deg |

Aggressor arrays radiating harmonic signals are scaled based on the frequency ratio and appear as sparse arrays, correlated beam forming weights are treated similarly.

Antennas have an ohmic loss of 1.5dB.

It is assumed the UE has an omni directional pattern.

#### 2.2.1.2	Element models

Previous simulation campaigns have only considered the element model at the fundamental frequency, in general we can assume that:

- elements radiating signals below their operating frequency tend to be wider but maintain a single radiation lobe in the same direction as the fundamental.

- Elements radiating signals above their operating frequency tend to be narrower and start to “split” where a null can appear in the direction of the fundamental.

Whilst it is difficult to represent every potential element used in array design a simple dipole model can be used to simulate this effect so we can observe its effect.

The simple element pattern of a dipole is given by:

,

Where, η is the impedance of free space, k is , I0 is the peak current, θ is the angle and L is the dipole length.

Analysis was done using both the standard element model and the above dipole model so the potential effect of this effect can be observed.

#### 2.2.1.3	Path loss models

For 6.4GHz the UMa model in table 7.4.1-1 of TR 38.901 has been used.

### 2.2.2	Results

#### 2.2.2.1	Fundamental - 6.4GHz aggressor

Considering an in-channel aggressor was not one of the use cases as co-existence in same geographical area is not a requirement in-band, however in many previous out of band analysis the worst case assumption was used that antenna models would be the same as in-band. As such it is a useful check point when doing the statistical simulation work (none of the models are operating out of band or as harmonics etc).

The dipole model is very similar to the standard element model at the fundamental frequency (as expected) however there is a clear difference between interference levels depending on the corelated and uncorrelated assumption, this difference is maintained at the 99% percentile.

#### 2.2.2.2	2nd Harmonic of 3.2GHz aggressor

Once again there is not a big difference between the dipole and the standard element model (at 1λ the dipole is still a single lobe), although the difference is somewhat exaggerated at the 99% point. There is a similar difference between correlated and uncorrelated assumptions, however the delta is smaller than at the fundamental, it is also notable that at the high percentiles the results between correlated and uncorrelated start to converge and even start to cross over.

#### 2.2.2.3	3rd Harmonic of 2.1 GHz aggressor

Again surprisingly little difference between the standard model and the dipole responses, this implies that in the direction of the victim BS the responses are similar. As with the 3.2GHz case they start to diverge a bit at the 99% point.

Similarly to the 2nd harmonic response the corelated and uncorrelated results converge at the high percentiles and also cross over at some point.

Looking at all the results together:

|  |  | Coupling Loss (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | MCL | 99th Percentile |  | Worst case |  |
|  |  | FSPL | Pathloss model |  | Pathloss model |  |
|  |  | dipole | Element model | dipole | Element model | dipole |
| Tx 2.1GHz AAS @ 6.4GHz | cor | 68.9 | 79 | 81.5 | 52.2 | 53.2 |
|  | un-cor | 84.2 | 74.5 | 77.5 | 58.4 | 55 |
| Tx 3.2GHz AAS @ 6.4GHz | cor | 85.3 | 80.5 | 75 | 60 | 47.5 |
|  | un-cor | 103.6 | 75 | 70.5 | 54.3 | 44.9 |
| Tx 6.4z AAS @ 6.4GHz | cor | x | 91.5 | 91.5 | 56.6 | 61.7 |
|  | un-cor | x | 78.5 | 79.5 | 57.9 | 56.3 |

Due to the remaining relatively large number of variables (correlation, element model, PL model) there are still quite a few results and statistical simulation is time consuming, the results at the 99th percentile as such are still a bit variable (it can be seen from the CCDF curves that the tail is not smooth).

To reduce the number of variables the results imply:

- In this case there is not much difference between the element model and the dipole model – as it is difficult to accurately simulate any particular element pattern (as they are very design dependent) the purpose of investigating the dipole model was to see if the splitting of the element pattern at high frequencies had a significant effect. As it has not we can use the element model only for this simulation.

- Un-correlated results are worse than corelated. This is slightly unexpected as uncorrelated gain worst case is lower than the corelated gain. However over most of the radiation pattern the correlated gain is higher as such statistically the uncorrelated results have a higher gain. The effect of the higher but statistically improbable correlated gain can be seen where the CCDF curve cross at lower probabilities and the worst case results show the correlated results as worse. As such we should perhaps continue to consider both correlation cases and maybe consider a compromise such as ρ=0.5 correlation.

Now we are considering fewer variables, more samples can be consider to smooth the tail and get more stable results.

The 50% correlation results are very similar to the uncorrelated results, although they can be seen on the CCDF plots to be in between the fully correlated and uncorrelated lines. Also at very low probabilities they cross over as the higher peaks gains start to occur.

In summary:

|  | Coupling Loss (dB) |  |  |
| --- | --- | --- | --- |
|  | 99th Percentile |  |  |
| correlation | 1 | 0.5 | 0 |
| Tx 2.1GHz AAS @ 6.4GHz | 78 | 75 | 74.5 |
| Tx 3.2GHz AAS @ 6.4GHz | 80 | 75 | 75 |
| Tx 6.4z AAS @ 6.4GHz | 91.5 | 79 | 79 |

From this data we can take a 99%-tile coupling loss of 75dB.

### 2.2.3	Summary

The existing co-existence spurious power requirement in the UL is -49dBm and this assumes a coupling loss of 67dB and desense of 0.8dB at the victim BS.

Using the statistical analysis approach we have a 99% percentile coupling loss of 75dB.

The emission requirement is hence:

Pco-existence_UL=-174+60+6+75-7 = -40dBm/MHz

(note BS NF is 6dB @ 6.4GHz)

Observation 1: using the 99% percentile approach the coexistence spurious emission output power in UL could be changed to -40dBm.

## 2.3.	BS-UE

It is assumed that the UE has an omni directional antenna, so if the BS aggressor is assumed to have a non-correlated radiation pattern the variation in the analysis comes from the location of the victim UE alone, the variation occurs in both the PL (due to distance and shadow fading) and direction. If the aggressor BS radiation pattern is assumed to be correlated then the aggressor network UE direction will affect the aggressor antenna gain pattern and this will be an additional variable.

As the victim UE is at ground level and well within the range of the UMa PL model only the path loss model is considered.

In the original analysis for BS-UE coupling the MR MCL of 53dB was used, for MR the minimum antenna distance is 5m not 35m, the 17dB difference being a rather simplistic difference in FSPL at 2GHz.

Whilst there is difference to antenna heights and definitions (and PL models) between WA and MR the effect of reducing the UE minimum distance to 5m was also investigated.

### 2.3.1	Results

Results were captured for 35m and 5m minimum distance, the difference was undetectable, this is because the probability of a UE falling closer than 35m is quite small, combined with the fading and the antenna gain variations it could not be detected in the number of samples investigated (it would no doubt show up in the worst-case figure if enough samples were run but the difference was not important at the 99% value). All results are captured with a 5m minimum distance.

#### 2.3.1.1	Fundamental - 6.4GHz aggressor

As the UE has an omni directional antenna we are seeing less variation, specifically for the uncorrelated where the BS antenna pattern is effectively fixed and the LOS/NLOS kind of the PL model can be identified in the results.

Similarly as the BS-BS results the uncorrelated is generally higher but in this case by the 99th percentile the lines have already crossed so the correlated case is worse.

As with the BS-BS the effect of the dipole model is minimal.

#### 2.3.1.2	2nd harmonic – 3.2GHz aggressor

Similar pattern to the fundamental results, the effect of the dipole model in this case is a bit more pronounced but still not great.

#### 2.3.1.3	3rd harmonic – 2.1GHz aggressor

Similar pattern to the fundamental results, the effect of the dipole model in this case is a bit more pronounced but still not great.

#### 2.3.1.4	Summary

Table 2.3.1.4. BS-UE coupling summary

Looking at the results we can see:

- Uncorrelated coupling is generally higher but at the high percentiles the CCDF curve cross over and at 99% the correlated results have higher coupling (for the BS-BS this cross over happens at even higher percentile values)

- As the direction of the victim is no longer fixed the impact of the element model is greater. The dipole model has most impact when considering correlated worst case. However at 99th percentile the impact is minimal.

- In almost all cases the statistical assumptions demonstrate lower coupling than the worst case MCL assumptions [2]

- Same frequency co-existence is not a use case but yields the highest coupling – a close frequency in a nearby band could be a use case so we should consider if this should be included.

### 2.3.2	Summary

The existing co-existence spurious power requirement in the DL is -52dBm and this assumes a coupling loss of 53dB and desense of 3dB at the victim UE.

Using the statistical analysis approach we have a 99% percentile coupling loss of 66dB.

The emission requirement is hence:

Pco-existence_DL=-174+60+9+75-0 = -39dBm/MHz

(note UE NF is 9dB @ 6.4GHz)

Observation 2: using the 99% percentile approach the coexistence spurious emission output power in DL could be changed to      -39dBm.

It has been noted that the existing requirement is based on the MR MCL value, it should be further discussed if simulations should be carried out for MR or micro cell scenarios.

Observation 3: It should be discussed further if micro cell scenarios should be investigated

### 2.4	TDD

As n104 is a TDD band the interference from FDD (or unsynchronised TDD) could be from either DL or UL as such we take the toughest requirement and apply it.

In the existing specification the UL requirement is -49dBm and the DL requirement is -52dBm. The TDD requirement is hence    -52dBm.

In the simulations done here

The UL requirement is -40dBm

The DL requirement is -39dBm

The TDD requirement would therefore be -40dBm.

Observation 4: using the 99% percentile approach the coexistence spurious emission output power in TDD band could be changed to -40dBm.

# 3	Conclusions

In this paper we have looked at the simulation approach to obtaining coupling loss assumptions between BS and UE of co-existing bands in the same geographical area.

In our previous paper [2] we used the MCL approach with modified assumptions about the AAS antennas and identified that this worst case approach yielded values similar to the existing requirements.

As AAS performance is more varied than the static MCL approach is able to incorporate a statistical simulation has been proposed and we have further investigated this.

Based on urban macro simulations we have made the following observations:

Observation 1: using the 99% percentile approach the coexistence spurious emission output power in UL could be changed to -40dBm.

Observation 2: using the 99% percentile approach the coexistence spurious emission output power in DL could be changed to -39dBm.

Observation 3: It should be discussed further if micro cell scenarios should be investigated.

Observation 4: using the 99% percentile approach the coexistence spurious emission output power in TDD band could be changed to -40dBm.

Based on these results the co-existence spurious emissions requirement in band n104 (and above) could be modified to -40dBm:

Proposal 1: The co-existence spurious emissions requirement in band n104 could be -40dBm.

# References

[1]	R4-2508762 Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: Tx spur co-ex	 Huawei

[2]	R4-2507446 Tx spurious emissions co-existence limits re-evaluation, Huawei

[3]	R4-2507670 On Transmitter Spurious Emission Co-existence discussion	 Ericsson