3GPP TSG-RAN WG4 Meeting #116	R4-2511558

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on PFD limits consideration for NTN UE RF specifications

Agenda Item:	7.7.5

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

During RAN4#115 meeting, power flux density (PFD) emission limits were discussed for VSAT operation in Ku bands.

In this contribution we provide our inputs to the discussion on the power flux density (PFD) requirements consideration in RAN4.

# 2	Discussion

# 2.1	Rel-18 Ka band discussion

During Rel-18 work item on NR NTN enhancements (NR_NTN_enh), the Ka bands were introduced. During the discussion on the SAN RF requirements, power flux density requirements for SAN were also discussed.

During RAN4#106 meeting, it was proposed in [1] to introduce SAN power flux density requirements (NTN band specific) based on regulations limits.

During the following RAN#106bis-e meeting, the WF in [2] captured the following agreement for SAN RF:

FFS if PFD regulatory requirements shall be included or only be referenced in RAN4 specifications.

Finally, it was decided that there is no need for the power flux density limits for NTN SAN operation in Ka bands.

# 2.2	Rel-19 Ku band discussion

During RAN4#115 meeting, WF on UE and SAN RF aspects was approved in [3], including the following agreement on PFD limits for NTN VSAT:

#### Issue 1-2-6: PFD limits for the Ku band - Proposals: - Proposal 1 (Ericsson): Capture PFD limits for NTN VSAT when specified by the Regulation. - FCC - A-ESIM, to protect fixed service network (when they have primary allocation in that band). 25.228(i): - CEPT - A-ESIM with NGSO – ECC Dec(18)05 - In 14.25-14.5 GHz where FS systems are authorized in that band: - L-ESIM with NGSO – ECC Dec(18)05 - In 14.25-14.5 GHz where FS systems are authorized in that band: PFD limit of -116 dBW/m²/MHz at 30 m height above ground - In 14.47-14.5 GHz: PFD threshold value at the observatory of -169 dBW/m²/(150 kHz), not to be exceeded more than 2% of the time (Ref: Recommendation ITU-R RA.769 - M-ESIM with NGSO – ECC Dec(18)05 - In 14.25-14.5 GHz where FS systems are authorized in that band: the PFD threshold value is -116 dBW/m²/MHz at a height of 80 metres above mean sea level at the low-water mark - In 14.47-14.5 GHz: PFD threshold value at the observatory of -169 dBW/m²/(150 kHz), not to be exceeded more than 2% of the time (Ref: Recommendation ITU-R RA.769) Wayforward: - Encourage companies to further check the PFD requirements, whether it is based on the measurement or analysis, before capture it to the specification.

Source contribution capturing the above VSAT PFD limit proposals was submitted in [4].

Referring to the ETSI Harmonized Standard developed by ETSI TC SES for Satellite mobile Aircraft Earth Stations (AESs) in ETSI EN 302 186 [6], it does capture PFD requirements for Aircraft Earth Stations. It shall be noted that ETSI TC SES is currently working on new Harmonized Standards for 3GPP-based IoT NTN UEs (ETSI EN 304 121) and NR NTN UEs (ETSI EN 304 122). Those two harmonized standards will be based on RAN4/RAN5 specifications. To keep technical consistency among both technical bodies, it is seen reasonable to keep the same approach and to reflect applicable PFD requirements also in RAN4.

Considering that the source of the discussed PFD limits comes from Regulation, those limits are anyway mandatory for VSAT operating in NTN Ku bands.

Additionally, during previous meetings companies were discussing whether PFD limits verification does depend on the measurements, or it can be verified based on calculations only. Referring to e.g., ETSI EN 302 186 [6], one can find the PDF test procedure where the PFD limitation procedure consists of the following:

- Measurement of the radiation pattern below the DUT (in this particular examples it was aircraft fuselage).

- Measurement of on-axis EIRP as a function of altitude.

- Finally, computation of the power flux density at the surface of the Earth, as extracted from ETSI EN 302 186 [6] below:

Therefore, the following is proposed:

Proposal 1: Reflect Ku band PFD limits in the NTN VSAT specification.

Whether the Ku band PFD limits shall be explicitly included in TS 38.101-5, or only be referenced can be further discussed. Still, for the Core specification TS 38.101-5, we shall at least refer to the related regulatory decisions.

Even though the current discussion is related to Ku work item only, the RAN4 approach to the PDF limits and related specification impact shall be aligned and consistent, i.e., if it will be decided to define PFD limits for Ku band, other NTN bands shall be handled in the same manner to avoid discrepancies.

Decision on the PFD limits consideration in RAN4 specifications shall not be band-specific, and shall be applicable in the same manner for all bands where related regulatory limits were defined.

Proposal 2: PFD limits consideration in NTN UE specifications shall be band-agnostic, and shall be applicable in the same manner for all bands, where related regulatory limits were defined.

# 3	Summary

Based on the above discussion, the following proposals were formulated:

Proposal 1: Reflect Ku band PFD limits in the NTN VSAT specification.

Proposal 2: PFD limits consideration in NTN UE specifications shall be band-agnostic, and shall be applicable in the same manner for all bands, where related regulatory limits were defined.

# References

[1]	R4-2301467, NTN enhancements – SAN RF requirements, RAN4#106

[2]	R4-2305927, WF on SAN RF requirements – NTN Enhancements, RAN4#106bis-e

[3]	R4-2508778, Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL, RAN4#115

[4]	R4-2506144, NTN Ku bands –VSAT RF, Ericson, RAN4#115

[6]	ETSI EN 302 186, Satellite Earth Stations and Systems (SES); Satellite mobile Aircraft Earth Stations (AESs) operating in the 11/12/14 GHz frequency bands; Harmonised Standard for access to radio spectrum