3GPP TSG-RAN WG4 Meeting #116		R4-2511567

Bangalore, India 25th – 29th August, 2025

Agenda Item:	7.14.3.2

Source: 		Qualcomm Incorporated

Title: 			Enhancement on FR2 CSSF

Document for:	Discussion

# 1.	Introduction

In this paper, we present our view on the enhancement of FR2 CSSF.

# 2.  Past Agreements

Following agreements happened during the last few meetings [1].

Issue 1-1: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - the CSSF optimization can be applied to the CSSF tables for the related CA/DC scenarios. - For solution 1: CSSF tables for CA, NR-DC, EN-DC and NE-DC - For solution 3: CSSF tables for CA, NR-DC, EN-DC with FR1 NR only Details of whether/how the CSSF factor is modified can be left to CR discussion Agreement in online session: - Solution 3 includes NR-DC of FR1 and FR1 + FR2 Agreement in online session: - This requirement applies regardless of whether measurement gap is configured or not. Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG. Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF Agreement: Support Solution 3 in Rel-19: - FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only. - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC). - For FR1+FR2 scenario, PCell is FR1 - FFS whether NR-DC is included or not Design for solution 3 (agreed in ad-hoc session) - Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future. - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing Agreement: - RAN4 to introduce new individual capability(s) for CSSF reduction in R19. - delay the capability detail discussion to the end of the core part. ### Issue 2-7: Other WID scope discussion Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable Agreement during RAN4 115: - For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule, the 3rd searcher resource sharing is same as the second searcher sharing. Issue 1-2-2: applicability limitation for solution 3 - For UE supporting both per-FR measurement gap and new 3-searcher capability, - If network configures per-FR measurement gap, UE will use legacy requirement (i.e., 2-searcher requirement) when at least one SMTC occasion of outside gap measurement objects in one FR are overlapped by per-FR measurement GAP in another FR. Otherwise, the 3-searcher requirement applies. - For UE supporting new 3-searcher capability, - If network configures per-UE measurement gap and does not configure per-FR measurement gap, the 3-searcher requirement always applies. ### Issue 1-3: issues related UE capability for CSSF enhancement Issue 1-3-1: UE capability for CSSF enhancement option 1 (measure one carrier per band) - For CSSF enhancement solution 1 in Rel-19, a new optional per-UE capability, apply for FR2-1 only. Note: the UE capability can be revisited if this feature is expanded to FR1 case.

# 3. 	Discussion

## 3.1 RRC Signaling regarding CSSF Optimization

RAN2 sent the following reply LS to RAN4 during this meeting [4]

RAN2 have discussed the RRC configuration to support the CSSF optimization and reached the following agreements: Introduce the UE specific RRC configuration (per UE) to enable/disable the CSSF optimization feature. About the network indication of a specific SCC per-band for measurement, RAN2 discussed the following two alternatives: - Alternative 1: Introduce new RRC configuration to indicate which SCC to be measured. - When CSSF optimization feature is enabled, UE only performs the serving and intra-frequency neighbor measurement on the indicated SCC per band. - Alternative 2: No additional RRC signaling is needed. - When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement. - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.. Majority’s view in RAN2 is Alternative 2, but RAN2 did not reach a consensus. Therefore, RAN2 would like to ask RAN4 to decide which alternative to adopt for SCC indication and provide feedback during August meeting so that RAN2 can prepare the RRC CR for approval in August meeting

Above LS shows that RAN2 discussed the possibility of legacy signaling vs new RRC signaling to enable CSSF feature. Majority view in RAN2 is to use legacy signaling but RAN2 could not reach a consensus.

Observation 1: Majority view in RAN2 is to use legacy RRC signalling to enable or disable Rel-19 CSSF optimization feature.

During initial configuration of a SCell, network can choose to configure or not configure servingCellMO. ServingCellMO IE is optional for SCells (see Appendix a).

Observation 2: During initial configuration of a SCell, network can choose to configure or not configure servingCellMO. ServingCellMO IE is optional for SCells

RAN2 spec shows that servingCellMO is a “Need M” IE for SCells. Hence, once network configures servingCellMO for a SCell, UE will retain the config of this servingCellMO even if network sends servingCellConfig without including servingCellMO IE in it.

However, even legacy spec allows network to remove servingCellMO from a Scell in two steps after initially configuring the Scell with servingCellMO.

- Step 1: Network releases the Scell using SCellToReleaseList IE.

- Step 2: Network adds the Scell using SCellToAddList IE without configuring servingCellMO for that Scell.

Observation 3: Legacy spec allows network to remove servingCellMO from a Scell in two steps after initially configuring the Scell with servingCellMO.

- Step 1: Network releases the Scell using SCellToReleaseList IE.

- Step 2: Network adds the Scell using SCellToAddList IE without configuring servingCellMO for that Scell

After step 2, UE will not have configuration for the MO of that particular serving cell. Obviously, UE won’t have to measure any MO associated with that serving cell.

Each SCell addition and release message causes 1 ms interruption to other inter-band cells and roughly 6ms interruption to other intra-band cells. However, network does not need to remove servingCellMO from a Scell frequently. Hence, the Scell release and addition based option won’t cause too much interruption, in total, and the whole procedure can be implemented through legacy signalling and procedure.

Observation 4: Network does not need to remove servingCellMO from a Scell frequently. Hence, the interruption due to Scell release and addition-based removal of servingCellMO should not be an issue.

Hence, RAN4 should agree with RAN2’s majority position and use legacy signalling to allow UE to measure one serving CC per band.

RAN4 made the following agreement before.

Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap.

If legacy signaling is used to allow UE to measure one serving CC per band, RAN4 does not need to define following procedure:

  - “Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.”

Hence, this portion of the previous agreement can be removed. The remaining portion of previous RAN4 agreement is still valid since UE can use that part of the agreement to get the benefit of measuring one serving CC per band when this feature is enabled.

Proposal 1: RAN4 agrees with the majority position of RAN2 and does not introduce any new signaling, except the network indication to enable/disable the feature, in Rel-19 CSF enhancement. RAN4 updates its previous agreement to the following:

Previous agreement: (changes to the previous agreement are shown in red)

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Note: Following the WID, the above agreement applies for the measurement without gap.

# 4.	Conclusion

Observation 1: Majority view in RAN2 is use to legacy RRC signalling to enable or disable Rel-19 CSSF optimization feature.

Observation 2: During initial configuration of a SCell, network can choose to configure or not configure servingCellMO. ServingCellMO IE is optional for SCells

Observation 3: Legacy spec allows network to remove servingCellMO from a Scell in two steps after initially configuring the Scell with servingCellMO.

- Step 1: Network releases the Scell using SCellToReleaseList IE.

- Step 2: Network adds the Scell using SCellToAddList IE without configuring servingCellMO for that Scell

Observation 4: Network does not need to remove servingCellMO from a Scell frequently. Hence, the interruption due to Scell release and addition-based removal of servingCellMO should not be an issue.

Proposal 1: RAN4 agrees with the majority position of RAN2 and does not introduce any new signaling, except network indicate to enable/disable Rel-19 CSF enhancement feature. RAN4 updates its previous agreement to the following:

Previous agreement: (changes to the previous agreement are shown in red)

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Note: Following the WID, the above agreement applies for the measurement without gap.

# 5.  Reference

[1] R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN3 #113.

[2] 3GPP TS 38.331 V 18.2.0

[3] R4-2502592, WF on RRM requirements for NR_RRM_Ph5_Part1, 3GPP TSG-RAN WG4 #114.

[4] R2-2504728/ R4-2502662, Reply LS on CSSF optimization for NR RRM Phase 5

# 6. Appendix A: ServingCellMO and SCellConfig in RAN2 Spec

Following spec snippets are obtained from 38.331 [2]:

sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig OPTIONAL, -- Need N sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex OPTIONAL, -- Need N ..., SCellConfig ::= SEQUENCE { sCellIndex SCellIndex, sCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Cond SCellAdd sCellConfigDedicated ServingCellConfig OPTIONAL, -- Cond SCellAddMod ..., …. } ServingCellConfig ::= SEQUENCE { …. servingCellMO MeasObjectId OPTIONAL, -- Cond MeasObject ..., [[ Conditional Presence; Explanation. AsyncCA; This field is mandatory present for SCells whose slot offset between the SpCell is not 0. Otherwise it is absent, Need S.. MeasObject; This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For RedCap UEs, this field is optionally present, Need M..