3GPP TSG-RAN4 Meeting #116		R4-2511569

# Bangalore, India 25th – 29th August 2025

Source: 	Qualcomm Incorporated

Title: 	Generalization Aspects of AI-ML

Agenda item:	7.17.2

Document for:	Discussion

# Introduction

RAN4 made the following agreements during the last few meetings [4]

RAN4 #114bis Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID Agreement: - RAN4 has the following understanding and need further check whether the understanding is aligned with RAN1 and RAN2: - AI/ML functionality activation refers to the process of enabling an applicable functionality to perform inference - functionality deactivation refers to the process of network deactivating an active functionality Agreement: - FFS on whether the deactivation requirements will be introduced - The starting point is when the UE receives the signalling to commence deactivation - FFS when the endpoint for deactivation is - FFS whether fallback delay is the same as deactivation delay or has some other components Agreement: - For beam management use case - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML - FFS on the definition of first reporting Agreements - There shall be consistency between applicable conditions signalling and testing - No test will be defined that implies a change of network condition or associated ID during the test. Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario. RAN4 #115 Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI Switching delay and Fallback delay: - Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion.

Figure 1: Steps for configuration and applicability reporting of different AI-ML association IDs

# Discussion

## Activation Delay

RAN4 has already agreed that activation delay starts at the reception of DCI or MAC-CE. RAN4 also agreed to define functionality activation delay requirements for the first aperiodic reporting based on AI/ML. RAN4 has not yet agreed regarding the end point of activation delay requirements.

Figure 1 shows the different steps that network and UE go through for configuration and applicability reporting of different AI-ML association IDs. The first aperiodic reporting can be triggered only after these five steps. Any RAN4 requirement, especially performance test, can only be checked via UE’s action through OTA reporting. Hence, activation delay can only be checked based on the time of UE’s inference report.

As Figure 2 shows, activation delay includes both measurement and inference delay. UE should require same number of samples to measure setB reference signals, irrespective of the reference signals being transmitted immediately after or long after the RRC reconfiguration of step 5 of Figure 1.

The main question lies if inference timeline differs between first aperiodic/semi-persistent predicted L1-RSRP reporting and subsequent AP/SP predicted L1-RSRP reporting. The answer depends on how UE loads the model for running inference in different scenarios.

Figure 3 shows different steps for UE to load the model from server and then eventually generate inference report. During the first AP/SP predicted L1-RSRP reporting, UE has to load model from off-chip memory to on-chip memory and then run inference. For subsequent AP/SP predicted L1-RSRP reporting, UE could keep the model in on-chip memory and then just run inference when report gets triggered. However, on-chip memory is limited. UE may support multiple AI-ML features. Hence, UE may need to run many AI-ML models in time. As a result, it will not be possible for UE to store all different AI-ML models in on-chip memory. Therefore, a typical UE would keep different AI-ML models in off-chip memory and load the model every time it has to run inference and generate report. From that perspective, inference timeline would be same during first and subsequent AP/SP reporting.

RAN1 has introduced only one set of parameters (d & d’) to capture inference timeline for AP reporting of predicted L1-RSRP. This set of parameters does not differ between first and subsequent reporting of predicted L1-RSRP.

Figure 2: Activation of Aperiodic predicted L1-RSRP reporting in AI-ML beam prediction

Figure 3: Required steps to load AI-ML models from server, run inference and generate inference report

Observation 1: RAN4 has already agreed that activation delay starts at the reception of DCI or MAC-CE.

Observation 2: Activation delay can only be checked based on the timing of UE’s inference report.

Observation 3: The required measurement delay of a particular AI-ML model should be same during first and subsequent AP/SP reporting of predicted L1-RSRP.

Observation 4: UE may need to run many AI-ML models in time. UE cannot store all different AI-ML models in on-chip memory.

Observation 5: A typical UE would keep different AI-ML models in off-chip memory and load the model every time it runs inference and generate report.

- With above approach, inference timeline of a particular AI-ML model should be same during first and subsequent AP/SP reporting of predicted L1-RSRP, too.

Observation 6: RAN1 does not differentiate inference time between first and subsequent aperiodic reporting of predicted L1-RSRP.

Proposal 1: RAN4 agrees inference delay to be same between first and subsequent aperiodic reporting of predicted L1-RSRP after RRC reconfiguration

- Note: This means that the required time separation between following channels should be same for both first and subsequent aperiodic reporting of predicted L1-RSRP after RRC reconfiguration

  - DCI and inference report

  - SetB reference signal and inference report

## Deactivation delay

RAN4 defined the starting point of functionality deactivation before. However, RAN4 could not decide the end point of functionality deactivation. RAN4 performance tests rely on UE’s action through over-the-air reporting. It is not clear what UE is supposed to report after deactivating a functionality. Hence, RAN4 should deprioritize defining deactivation delay requirement.

Observation 7: It is not clear what UE is supposed to report after deactivating an AI-ML functionality.

Proposal 2: RAN4 deprioritizes defining deactivation delay requirement.

# Conclusions

Observation 1: RAN4 has already agreed that activation delay starts at the reception of DCI or MAC-CE.

Observation 2: Activation delay can only be checked based on the timing of UE’s inference report.

Observation 3: The required measurement delay of a particular AI-ML model should be same during first and subsequent AP/SP reporting of predicted L1-RSRP.

Observation 4: UE may need to run many AI-ML models in time. UE cannot store all different AI-ML models in on-chip memory.

Observation 5: A typical UE would keep different AI-ML models in off-chip memory and load the model every time it runs inference and generate report.

- With above approach, inference timeline of a particular AI-ML model should be same during first and subsequent AP/SP reporting of predicted L1-RSRP, too.

Observation 6: RAN1 does not differentiate inference time between first and subsequent aperiodic reporting of predicted L1-RSRP.

Observation 7: It is not clear what UE is supposed to report after deactivating an AI-ML functionality.

Proposal 1: RAN4 agrees inference delay to be same between first and subsequent aperiodic reporting of predicted L1-RSRP after RRC reconfiguration

- Note: This means that the required time separation between following channels should be same for both first and subsequent aperiodic reporting of predicted L1-RSRP after RRC reconfiguration

  - DCI and inference report

  - SetB reference signal and inference report

Proposal 2: RAN4 deprioritizes defining deactivation delay requirement.

# References

- RAN4 Chair’s Notes, 3GPP TSG RAN WG4 #114bis

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #120bis

- R1-2502829, Specification support for AI-ML based beam management, Qualcomm Incorporated, 3GPP RAN1 120bis.

[4] R1-2504992, “Introduction of AI/ML BM Prediction”, 3GPP TSG-RAN WG#121