3GPP TSG-RAN4 Meeting #116		R4-2511570

# Bangalore, India 25th – 29th August, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	7.17.4

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

RAN4 made the following agreements during the last few meetings [7][8][9][14]

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-4: QCL Source RS for TCI states Agreement: wait for RAN1 discussion to conclude without sending any LS Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point. Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

RAN1 made the following agreements in [5][6]:

- RAN1#117: Working Assumption - Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). - RAN1#118 agreement: For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list are associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

# Discussion

## 2.1 Selection of Beam Prediction Metrics

RAN4 made the following agreements during the last couple of meetings.

RAN4 114 Agreement If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options RAN4 114bis Agreement - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

RAN4 114 agreement ensures defining both beam prediction accuracy and RSRP accuracy metrics if RSRP and beam ID are reported. RAN4 114bis agreement, however, defined beam prediction accuracy metric when only beam IDs are reported. That means, without any further RAN4 agreement, UE will have to meet only RSRP accuracy requirements when both beam ID and RSRP are reported.

One of the main objectives of AI-ML beam prediction is to allow gNB being informed of the strongest ground truth beam with some accuracy without actually transmitting the corresponding reference signal. If UE only has to meet RSRP accuracy metrics, UE’s predicted top beam can be very far from the strongest ground truth beam. Even if UE meets L1-RSRP accuracy for its predicted top beam, the reported top 1 predicted beam might not be a suitable beam as a reference for changing TCI state and uplink spatial relationship.

Observation 1: If K > 1, network has to transmit the top-K predicted beams in a 2nd round of beam sweep and collect UE’s report regarding the strongest beam among top-K predicted beams before changing active TCI state.

Observation 2: Currently agreed procedures do not allow network to dynamically configure (e.g., via DCI or MAC-CE) the 2nd round of beam sweep based on UE’s inference results regarding top-K predicted beams.

Observation 3: The issue of 2nd round of beam sweep can be avoided by making K = 1.

Proposal 1: Update the previous agreement with the following changes:

- “The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among of top-K1 predicted beams being larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

- Note: if x=0 and N=1, it means Top-K1/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K1 predicted beams"

Observation 4: In legacy BM,

- The core requirement of L1-RSRP absolute accuracy applies to all reported beams.

- Performance test of absolute L1-RSRP accuracy only uses reporting of one beam.

Proposal 2: RAN4 ensures the following in AI-ML BM case 1,

- The core requirement of absolute accuracy of predicted L1-RSRP applies to all top-K predicted beams.

- Performance test of absolute accuracy of predicted L1-RSRP uses prediction and reporting of one beam, i.e., K = 1.

Previous Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Proposal 3: RAN4 defines relative RSRP accuracy requirement for Case 1 and uses the agreed baseline with the following changes:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i – reported predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

Observation 5: In legacy BM,

- The core requirement of L1-RSRP relative accuracy applies to all reported beams.

- Performance test of absolute L1-RSRP accuracy only uses reporting of two beams.

Proposal 4: RAN4 ensures the following in AI-ML BM case 1,

- The core requirement of relative accuracy of predicted L1-RSRP applies to all top-K predicted beams.

- Performance test of relative accuracy of predicted L1-RSRP uses prediction and reporting of two beams, i.e., K = 2.

## 2.2 TCI State Status of Predicted Tx Beam

Legacy spec defines the conditions for known TCI state in the following way [18]:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB

RAN4 agreed to the following during the last meeting.

Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

Recent RAN4 agreement clearly shows that the ability to predict RX beam of a selected Tx beam will be a UE capability. The TCI state switch time gets extended if UE does not know the RX beam that it can use to receive new TCI state.

UE’s knowledge of RX beam influences TCI state switch time. Hence, the known vs unknown definition of TCI state definition is influenced based on RX beam knowledge. Proposal 5 captures the impact of predicted L1-RSRP and new capability regarding RX beam on the known conditions for TCI state in AI-ML beam prediction.

In legacy spec, a TCI state would be classified as “known” only if measured SNR of the corresponding RS is >= -3. Same procedure is followed in proposal 5. Please note that AI-ML output does not through include SNR. On the othetr hand, legacy spec enforces a TCI state to be known only when the measured SNR of the corresponding spec >= 03 dB.

In this scenario, UE may not get the chance to measure the RS of setA close to the TCI state switch timeline. That’s why, we proposed the predicted SNR to be > -3 dB. Although, Rel-19 specs focus on L1-RSRP prediction. It would be easy to translate L1-RSRP prediction to SNR prediction. That’s why minimum SNR threshold is defined based on predicted SNR instead of the measured SNR.

Proposal 5:

Known conditions for TCI state in AI-ML beam prediction:

For BM-case 1, the TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS configured via resourcesForChannelMeasurement RS resource that gets used to generate for the predicted-L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for of predicted-L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource configured via resourcesForChannelMeasurement for beam reporting or measurement

- 	UE supports [UE-side beam prediction] feature or the RS in target TCI state is known according to clause 8.10.2 or an RS that is QCL-ed Type-D with the RS in target TCI state is known according to clause 8.10.2

-	The UE has sent at least 1 predicted L1-RSRP report for the target TCI state before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	predicted SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

## 2.3 Inference Timeline of AI-ML Beam Prediction

RAN4 made several agreements regarding delay timeline of AI-ML based beam prediction.

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2

RAN4 has not discussed and defined inference delay for AI-ML beam prediction. RAN1 has started discussing inference delay for AI-ML beam prediction. RAN1 has already agreed to extend the legacy L1-RSRP reporting timeline to accommodate inference timeline in AI-ML based predicted L1-RSRP reporting.

RAN1 captured the following in its most recent CR that focuses on Re-19 AI-ML air interface [20]:

Above text shows that Z3, the minimum time separation between DCI trigger and AP L1-RSRP report, and Z3’, the minimum time separation between RS and AP L1-RSRP report, will be extended by d and d’ respectively in AI-ML beam prediction. Besides, the value of d and d’ can differ between spatial only beam prediction and spatio-temporal beam prediction. RAN1 is expected to discuss the specific values of d and d’ during August 2025meeting. These d and d’ values are expected to capture the inference timeline of UE.

The inference timelines mentioned in RAN4 spec should be aligned with RAN1 spec. RAN4 is discussing inference timeline in the context of prediction delay, i.e., the time separation between resources for inference and the report containing predicted RSRP. This is exactly what RAN1 is trying to capture through the parameter d’. Hence, the inference timeline of RAN4 spec should just refer to d’.

Observation 11: RAN1 has agreed to increase the minimum time separation between reference signal and aperiodic L1-RSRP report by d’ in AI-ML beam prediction.

- Exact value of d’ is TBD in RAN1.

Observation 12: RAN4 has agreed to include inference delay in prediction delay of AI-ML BM. This is equivalent to the parameter d’ of RAN1 spec.

Proposal 6: The inference timelines mentioned in RAN4 spec should be aligned with d’ in RAN1 spec.

- Note 1: d’ is the extension in time separation between RS and AP L1-RSRP report in AI-ML based beam prediction.

- Note 2: d’ can vary between spatial only and spatio-temporal beam prediction. Exact values of d’ will be discussed on August 2025 meeting in RAN1.

# Conclusions

Observation 1: If K > 1, network has to transmit the top-K predicted beams in a 2nd round of beam sweep and collect UE’s report regarding the strongest beam among top-K predicted beams before changing active TCI state.

Observation 2: Currently agreed procedures do not allow network to dynamically configure (e.g., via DCI or MAC-CE) the 2nd round of beam sweep based on UE’s inference results regarding top-K predicted beams.

Observation 3: The issue of 2nd round of beam sweep can be avoided by making K = 1.

Observation 4: In legacy BM,

- The core requirement of L1-RSRP absolute accuracy applies to all reported beams.

- Performance test of absolute L1-RSRP accuracy only uses reporting of one beam.

Observation 5: In legacy BM,

- The core requirement of L1-RSRP relative accuracy applies to all reported beams.

- Performance test of absolute L1-RSRP accuracy only uses reporting of two beams.

Observation 6: RAN1 has agreed to increase the minimum time separation between reference signal and aperiodic L1-RSRP report by d’ in AI-ML beam prediction.

- Exact value of d’ is TBD in RAN1.

Observation 7: RAN4 has agreed to include inference delay in prediction delay of AI-ML BM. This is equivalent to the parameter d’ of RAN1 spec.

Proposal 1: Update the previous agreement with the following changes:

- “The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among of top-K1 predicted beams being larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

- Note: if x=0 and N=1, it means Top-K1/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K1 predicted beams"

Proposal 2: RAN4 ensures the following in AI-ML BM case 1,

- The core requirement of absolute accuracy of predicted L1-RSRP applies to all top-K predicted beams.

- Performance test of absolute accuracy of predicted L1-RSRP uses prediction and reporting of one beam, i.e., K = 1.

Proposal 3: RAN4 defines relative RSRP accuracy requirement for Case 1 and uses the agreed baseline with the following changes:

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i – reported predicted L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

Proposal 4: RAN4 ensures the following in AI-ML BM case 1,

- The core requirement of relative accuracy of predicted L1-RSRP applies to all top-K predicted beams.

- Performance test of relative accuracy of predicted L1-RSRP uses prediction and reporting of two beams, i.e., K = 2.

Proposal 5:

Known conditions for TCI state in AI-ML beam prediction:

For BM-case 1, the TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS configured via resourcesForChannelMeasurement RS resource that gets used to generate for the predicted-L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for of predicted-L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource configured via resourcesForChannelMeasurement for beam reporting or measurement

- 	UE supports [RxBeamKnownDuringTxBeamBeamPrediction] feature or the RS in target TCI state is known according to clause 8.10.2 or an RS that is QCL-ed Type-D with the RS in target TCI state is known according to clause 8.10.2

-	The UE has sent at least 1 predicted L1-RSRP report for the target TCI state before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	predicted SNR of the TCI state ≥ -3dB

Otherwise, the TCI state is unknown.

Proposal 6: The inference delay mentioned in RAN4 spec should be aligned with d’ in RAN1 spec.

- Note 1: d’ is the extension in time separation between RS and AP L1-RSRP report in AI-ML based beam prediction.

- Note 2: d’ can vary between spatial only and spatio-temporal beam prediction. Exact values of d’ will be discussed on August 2025 meeting in RAN1.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R1-2308259, “Evaluations on AI/ML for Beam Management”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 #114.

- R4-2417211, “Simulation assumptions for beam prediction”, 3GPP TSG-RAN WG4 #112bis

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- R4-2420480, Updated Simulation Assumptions for Beam Prediction, RAN4 #113.

[14] R4-2502856, “WF on the requirements for AI/ML air interface”, 3GPP TSG-RAN WG4 Meeting#114

[15] R4-2505105, “WF on requirements for AI/ML air interface normative work”, 3GPP TSG-RAN WG4 Meeting#114bis

[16] R4-2505152, “Updated simulation assumptions for AI/ML BM”, 3GPP TSG-RAN WG4 Meeting#114bis

[17] 3GPP TSG WG RAN1 Chairman’s notes, RAN1 #120bis

[18] 38.133 V 18.7.0, Requirements for support of radio resource management.

[19] R4-2502856, WF on the requirements for AI/ML air interface, 3GPP RAN WG4 Meeting#114

[20] R1-2504992, “Introduction of AI/ML BM Prediction”, 3GPP TSG-RAN WG#121