3GPP TSG-RAN4 Meeting #116	R4-2511573

# Bangalore, India 25th – 29th August, 2025

Source: 	Qualcomm Incorporated

Title: 	CSI Prediction: Simulation Results

Agenda item:	7.17.7

Document for:	Discussion

# Introduction

In this paper, we present simulation results for AI-ML CSI prediction.

# Simulation Framework

During the last meeting, RAN4 agreed to a diverse set of options to simulate the performance of AI-ML based CSI prediction. Many companies did not submit SGCS2 and SGCS3 results during the last meeting. Hence, companies were requested to submit results in terms of SGCS and throughput during RAN4 116 [1]-[3]. This contribution focuses on those options and contains SGCS centric simulation results. The simulation results have been re-submitted from our last meeting’s contribution [4].

Table 1 shows the set of simulation assumptions that were used to generate result for this contribution.

Table 1: Simulation assumptions for AI/ML based CSI prediction of this contribution

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD |
| Carrier frequency | 2GHz |
| Bandwidth | 10MHz |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB |
| Subcarrier spacing | 15kHz for 2GHz |
| gNB TX antennas | 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20, 50 and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). |
| Rank per UE | Rank 1, 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step-2 and Step-3: take CSI feedback into account Use Rel-18 eTypeII-Doppler, N4 = 1 |
| CSI feedback overhead | Option4: PC7 |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Step-1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Step-2: - check SGCS after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads. |
| Number of samples | 25000 chunks for each Doppler Hz. Channel gets refreshed at the beginning of each chunk. Each chunk consists of estimated channels of 10 CSI-RS symbols where each CSI-RS symbol is separated by CSI-RS periodicity in time domain. Since we predict CSI-RS based on 5 observations, each chunk consists of 5 samples, in total, in terms of CSI-RS prediction. For individual Doppler training: 20,000 chunks get used for training. 5,000 chunks get used for testing. For mixed Doppler training: 60,000 chunks get used for training. 15,000 chunks get used for testing. |

# Simulation Results

## Calculation of SGCS after CSI feedback in 2nd step

Figure 1: Calculation of SGCS after CSI Feedback

Figure 1 shows two potential calculations of SGCS after CSI feedback, SGCS2 and SGCS3. It is not clear whether SGCS2 or SGCS3 need to be reported for SGCS after CSI feedback. We report both SGCS2 and SGCS3 in this contribution.

Observation 1: Both SGCS2 and SGC3 of Figure 1 are reported as SGCS after CSI feedback in 2nd step.

## Results

Table 2: SGCS results at step 1, 2 and 3 with rank 2

| Prediction algorithm | Doppler during training (Hz) | Doppler during test (Hz) | Layer | Rank | Step 1 SGCS1 | Step 2 SGCS2 | Step 2 SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample and hold | -- | 20 | 0 | 2 | 0.7805 | 0.7102 | 0.7157 |
| Sample and hold | -- | 20 | 1 | 2 | 0.6245 | 0.5363 | 0.5427 |
| AI-ML20 | 20 | 20 | 0 | 2 | 0.9938 | 0.8957 | 0.9435 |
| AI-ML20 | 20 | 20 | 1 | 2 | 0.9892 | 0.8351 | 0.9071 |
| AI-ML Mix | 20+50+100 | 20 | 0 | 2 | 0.9926 | 0.8947 | 0.9410 |
| AI-ML Mix | 20+50+100 | 20 | 1 | 2 | 0.9866 | 0.8332 | 0.9029 |
| Sample and hold | -- | 50 | 0 | 2 | 0.3127 | 0.2970 | 0.2959 |
| Sample and hold | -- | 50 | 1 | 2 | 0.1734 | 0.1662 | 0.1694 |
| AI-ML50 | 50 | 50 | 0 | 2 | 0.9572 | 0.8653 | 0.8931 |
| AI-ML50 | 50 | 50 | 1 | 2 | 0.9173 | 0.7788 | 0.8167 |
| AI-ML Mix | 20+50+100 | 50 | 0 | 2 | 0.9590 | 0.8668 | 0.8945 |
| AI-ML Mix | 20+50+100 | 50 | 1 | 2 | 0.9197 | 0.7806 | 0.8187 |

Table 3: SGCS results at step 1, 2 and 3 with rank 1

| Prediction Algorithm | Doppler during training (Hz) | Doppler during test (Hz) | Layer | Rank | Step 1 SGCS1 | Step 2 SGCS2 | Step 2 SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample and hold | -- | 20 | 0 | 1 | 0.7805 | 0.7102 | 0.7157 |
| AI-ML20 | 20 | 20 | 0 | 1 | 0.9938 | 0.8957 | 0.9435 |
| AI-ML Mix | 20+50+100 | 20 | 0 | 1 | 0.9926 | 0.8947 | 0.9410 |
| Sample and hold | -- | 50 | 0 | 1 | 0.3127 | 0.2970 | 0.2959 |
| AI-ML50 | 50 | 50 | 0 | 1 | 0.9572 | 0.8653 | 0.8931 |
| AI-ML Mix | 20+50+100 | 50 | 0 | 1 | 0.9590 | 0.8668 | 0.8945 |

Table 2 and table 3 show the following things:

- The SGCS of AI-ML based CSI prediction in step 1 and step 2 are significantly higher than those of “sample and hold” methods in 20 and 50Hz.

- The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

Observation 2: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 4: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

## Complexity

RAN4 made the following observation during the last meeting

Way forward for next step simulation for CSI prediction: …………. - Companies can also provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models.

The number of parameters of our AI-ML CSI prediction model to generate the results of this contribution is in the order of several millions.

Observation 5: The number of parameters of our AI-ML CSI prediction model, that got used to generate the results of this contribution, is in the order of several millions.

# Conclusions

Observation 1: Both SGCS2 and SGC3 of Figure 1 are reported as SGCS after CSI feedback in 2nd step.

Observation 2: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 3: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

Observation 4: The number of parameters of our AI-ML CSI prediction model, that got used to generate the results of this contribution, is in the order of several millions.

# References

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN WG4 #114bis..

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, RAN4 #114bis.

- R4-2508102, “WF on updated simulation assumptions for AI based CSI prediction”, RAN4 #115

- R4-2505513, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 #115.