# 3GPP TSG RAN WG4 Meeting#116	R4-2511591

# Bengaluru, India, Aug 25 – Aug 29, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements of LP-WUS/WUR for NR

Agenda item:	7.24.7

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for LP-WUS/WUR including various simulation assumptions were discussed [1]. In this paper, we present our perspectives on the remaining open issue for defining demodulation performance requirements.

2. Testability Aspects

2.1 Whether to define a test mode for LP-WUS/WUR

Whether to define a test mode for LP-WUS/WUR

- Agreement

- Take RF approach as a starting point to test MDR for demodulation requirements.

RF agreement

Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods)

- FFS whether test mode is recommended for MDR testing.

During the last meeting, there was a consensus to use the RF approach as the starting point for evaluating MDR requirements for LP-WUS/WUR. This includes verifying LP-WUR performance based on a 1% MDR threshold, which can be tested via the UE’s response to the network or test equipment—such as ACK/NACK in CONNECTED state or MSG1/3 in IDLE state. The corresponding WF is presented below.

It is noted that introducing a dedicated test mode for MDR testing would necessitate a specific implementation solely for RAN4 testing purposes. From a UE implementation standpoint, this is not desirable, as it introduces additional complexity and development overhead.

Observation 1: Introducing a test mode solely for RAN4 testing is not preferred, as it would require a dedicated implementation on the UE side.

Proposal 1: Do not introduce a test mode for MDR testing.

2.2 Whether to consider shortening the test duration (if a test is introduced)

The potential introduction of a FAR test and its feasibility, including the use of a dedicated test mode, was briefly discussed during the last meeting. The WF is outlined below.

Whether to consider shortening the test duration (if a test is introduced)

- Agreement:

- FFS whether to introduce FAR test and further study feasibility of introducing FAR test

- FFS whether test mode is recommended for FAR testing.

In our view, MDR is anticipated to have a more significant impact on demodulation performance, as in this case the MR will not monitor paging (idle mode) or PDCCH occasions (connected mode). Conversely, FAR may not affect the demodulation performance but will result in a power penalty, as the MR will incorrectly wake up to monitor paging or PDCCH occasions. Therefore, it is more suitable to only consider MDR as part of the demodulation performance requirements.

Observation 2: The WUR might incorrectly indicate the presence of a WUS when no transmission has occurred, resulting in a false alarm. A high FAR primarily impacts the power saving performance.

Proposal 2: Do not introduce an FAR test for LP-WUS/WUR.

2.3 Whether to define separate test cases for idle/inactive and connected modes

Whether to define separate test cases for idle/inactive and connected modes

- WF

  - Option 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LP-WUR, which mode is selected for test should be up to UE declaration.

The WF is outlined below.

The issue of whether to define separate test cases for IDLE/INACTIVE and CONNECTED modes should be revisited once discussions around simulation parameters have progressed further, particularly to determine if distinct test assumptions are needed for each mode.

Proposal 3: Defer this discussion whether separate test cases for idle/inactive and connected modes.

3. Demodulation Requirements

3.1 Whether to introduce FAR test

Whether to introduce FAR test

- WF

  - Option 1: Yes.

  - Option 2: No

The WF is outlined below.

Regarding the potential introduction of a FAR test, we refer to Observation 2 and Proposal 2 above. In our view, FAR primarily affects power-saving performance and does not reflect any demodulation aspects of LP-WUS/WUR. Therefore, we believe such a test should not be introduced as part of the demodulation performance requirements.

3.1 LP-WUS waveform

LP-WUS waveform

- WF

Option 1: Consider both OOK-1 and OOK-4

There were proposals as to which waveform to consider for LP-WUS. The corresponding WF is captured below.

In this context, we believe the consideration should be made alongside the parameter M, i.e., number of chips in the OFDM symbol. We note that OOK-1 can be viewed as a special case of OOK-4 when M=1. Therefore, we consider it sufficient to focus solely on OOK-4 for the LP-WUS waveform.

Proposal 4: Consider only OOK-4 for LP-WUS waveform.

3.2 Receiver Type

Receiver type

- WF

  - Option 1: Consider both OOK and OFDM receivers

The corresponding WF is outlined below.

Based on the RAN1 design, two types of WUR implementations may exist depending on UE capabilities. For instance, a UE may declare support for either an OOO receiver or an OFDM receiver. These receiver types employ different detection techniques, which can influence detection performance metrics such as MDR. For example, OOK detection may rely on envelope detection, while OFDM receivers typically use coherent detection. Additionally, these receivers are often tailored to different use cases, such as OFDM receivers for eMBB use cases and envelope detectors for IoT scenarios. Therefore, we support defining performance requirements for both receiver types.

Proposal 5: Support option 1 (consider both OOK and OFDM receivers for LP-WUS demodulation requirements).

3.3 Frequency Range

The WF is outlined below

Frequency range

- WF

  - Option 1: Consider both FR1 and FR2.

  - Option 2: Prioritize FR1; FFS FR2.

.

For the frequency range, we support defining requirements for both FR1 and FR2.

Proposal 6: Support option 1 (Consider both FR1 and FR2).

3.4 SCS and Bandwidth

In the RAN4 demodulation specification for FR1, 15 kHz SCS is used for FDD with 10 MHz bandwidth, while 30 kHz SCS is applied for TDD with 40 MHz bandwidth. For FR2, 120 kHz SCS is predominantly paired with 100 MHz bandwidth. Accordingly, we propose adopting these same SCS and bandwidth configurations for defining LP-WUS/WUR demodulation requirements in the context of FR1 and FR2.

Proposal 7: Consider 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for TDD FR2.

3.5 Number of RBs

The WF is outlined below

Number of RBs

- WF

  - Option 1: 11RB

.

The current agreement from RAN1 supports 11 RBs for LP-WUS for channel bandwidths equal to or greater than 5 MHz.

Proposal 8: Support option 1 (11 RB).

Antenna configuration

- WF

- Option 1: 1T1R

3.6 Antenna Configuration

The WF is outlined below.

In our view, 1T1R antenna configuration could be a practical and feasible option for FR1.

Proposal 9: Support option 1 (1T1R) for FR1.

3.7 Channel model

Channel model

- WF

  - Option 1: TDLA30-10 only

  - Option 2: TDLC300-100 only

  - Option 3: Both TDLC300-100 and TDLA30-10

The WF is outlined below.

Regarding the channel model, we believe that TDLC300-100 alone is sufficient for evaluating demodulation requirements, as it represents a more challenging scenario compared to TDLA30-10. Therefore, if a UE can meet the performance criteria under TDLC300-100, it should be considered adequate.

Proposal 10: Support option 2 (TDLC300-100 only).

3.8 Frequency offsets

Frequency offsets

- WF

  - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm

  - Option 1: up to 20 ppm

The WF is outlined below.

We note that OFDM receivers are generally more sensitive to frequency offsets than OOK receivers. Therefore, it is reasonable to apply a lower frequency offset tolerance for OFDM-based reception compared to OOK.

Proposal 11: Support option 1 (For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm).

3.9 Performance requirements for LP-SS

The WF is outlined below

Whether to define performance requirements for LP-SS

- WF

  - Option 1: No

  - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

.

In RAN4 demodulation requirements, we typically do not define explicit tracking performance requirements. Instead, such performance is implicitly captured through demodulation requirements, for example, via MDR testing in this case.

Proposal 12: Support option 1 (Don’t define performance requirements for LP-SS).

Whether to define single requirement for Type 1 and Type 2 receivers

- WF

  - Option 1: Further discuss based on simulation results if one or two sets of requirements are to be defined

3.10 Whether to define single requirements for Type 1 and Type 2 receivers

The WF is outlined below.

In our view, this topic should be revisited after simulation results from both receivers have been analyzed. Depending on the chosen simulation assumptions, it may or may not be feasible to define a unified requirement for both receiver types. Therefore, we propose postponing this discussion until a meaningful comparison can be made.

Proposal 13: Defer the decision on whether to define a unified requirement until simulation results for both receivers have been thoroughly analyzed.

3.11 Information bits for LP-WUS

To reduce simulation complexity, we propose using 3 information bits for LP-WUS.

Proposal 14: Consider using 3 information bits for LP-WUS.

4. Conclusions

In this paper, we share our perspectives on the open issues regarding the demodulation requirements for LP-WUS/WUR. The following proposals have been made.

Observation 1: Introducing a test mode solely for RAN4 testing is not preferred, as it would require a dedicated implementation on the UE side.

Proposal 1: Do not introduce a test mode for MDR testing.

Observation 2: The WUR might incorrectly indicate the presence of a WUS when no transmission has occurred, resulting in a false alarm. A high FAR primarily impacts the power saving performance.

Proposal 2: Do not introduce an FAR test for LP-WUS/WUR.

Proposal 3: Defer this discussion whether separate test cases for idle/inactive and connected modes.

Proposal 4: Consider only OOK-4 for LP-WUS waveform.

Proposal 5: Support option 1 (consider both OOK and OFDM receivers for LP-WUS demodulation requirements).

Proposal 6: Support option 1 (Consider both FR1 and FR2).

Proposal 7: Consider 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for TDD FR2.

Proposal 8: Support option 1 (11 RB).

Proposal 9: Support option 1 (1T1R) for FR1.

Proposal 10: Support option 2 (TDLC300-100 only).

Proposal 11: Support option 1 (For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm).

Proposal 12: Support option 1 (Don’t define performance requirements for LP-SS).

Proposal 13: Defer the decision on whether to define a unified requirement until simulation results for both receivers have been thoroughly analyzed.

Proposal 14: Consider using 3 information bits for LP-WUS.

5. References

[1] R4-2508626, “WF for [115][332] NR_LPWUS_Demod,” 3GPP TSG RAN WG4 Meeting #115, St. Julian’s, Malta, May 19 – 23, 2025.