# 3GPP TSG RAN WG4 Meeting#116	R4-2511594

# Bengaluru, India, Aug 25 – Aug 29, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for 6Rx devices

Agenda item:	7.1.4

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, the scope of the performance requirements for 6Rx WI including various simulation assumptions were discussed [1]. In this paper, we present our perspectives on the remaining open issues for defining demodulation and CSI reporting requirements.

2. General

2.1 Test applicability rule for 6Rx test cases

Test applicability rule for 6Rx tests

- Proposals

- Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signaling to differentiate UE type introduced by RAN2.

- Option 2: Introduce test applicability rule only for 6 layers cases

- Option 3: Postpone the discussion after RAN2 finishes the UE capability signaling design

Although the topic wasn’t discussed during the previous meeting, there was a proposal regarding the test applicability rules for the 6Rx test. The Way Forward is outlined below.

Regarding this issue, our view is that it can be postponed until the UE capability signaling is completed.

Proposal 1: Support option 3 (Postpone the discussion after RAN2 finishes the UE capability signaling design)

2.2 Test applicability rule for different number of Rx antenna ports

Test applicability rule for different number Rx antenna ports

- Proposals

- Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass.

- Other options

Similar to the test applicability rule for the 6Rx test, the topic concerning applicability rules for different numbers of Rx antenna ports was not discussed during the previous meeting. The Way Forward is outlined below.

Similar to the topic addressed in Section 2.1, we believe that the applicability rules for tests involving different numbers of Rx antenna ports should be discussed after UE capability signaling and requirements have been defined.

Proposal 2: Postpone the discussion on the applicability rule for different number of Rx antenna ports after UE capability signaling and requirements have been defined.

2.3 Specification structure

Specification structure

- Proposals

- Option 1: Add a new clause for 6Rx

- Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

Several proposals were discussed regarding the inclusion of 6Rx demodulation requirements in the specification. The details of these proposals are outlined below.

To properly capture the 6Rx requirements, we suggest adding a new clause under Section 5.2 of the current RAN4 demodulation specification, for example, Section 5.2.5 for 6Rx Requirements.

Proposal 3: Support option 1 (Add a new clause for 6Rx, such as 5.2.5 6Rx Requirements)

3. PDSCH Requirements for 6Rx

3.1 Performance requirements for 4 Layers

3.1.1 MIMO correlation for 4 MIMO layers

The MIMO correlation for 4 MIMO layers was also discussed during the last meeting. The way forward is outlined below.

MIMO correlation for 4 MIMO layers for Handheld UE testing

- Agreement

- Introduce correlation matrix for 6Rx in the specification:

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β1625*β925*β425*β125*1

- Proposal

- Option 1: with α = 0 and β = 0.07179

- Option 2: with α = 0.3 and β = 0.07179

- Option 3: with α = 0 and β = 0

It was agreed during the last meeting that RAN4 would consider low correlation for the 6L case. Although 6L applies only to FWA devices, we believe it is still feasible to consider the low correlation for the 4L case, which is applicable to both handheld and FWA devices.

Proposal 4: Support option 3 (α = 0 and β = 0)

3.1.2 Test scenarios for 4 MIMO layers

Test cases for 4 MIMO layers for FWA and Handheld UE

- Proposals

- Option 1: Define one single test case

  - Option 1a: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE

  - Option 1b: One single test case for 4 MIMO layers corresponding to a Handheld UE

- Option 2: Define two sets of 4 layers tests with different MIMO correlation

RAN4 has agreed to introduce 4-layer requirements for both FWA and handheld UE devices. Several proposals were discussed during the last meeting. The way forward is outlined below.

To this end, our preference is to consider a single test case for the 4 MIMO layers that applies to both FWA and handheld UE devices. This approach would minimize the number of test cases for the 4-layer PDSCH demodulation requirements.

Proposal 5: Support option 1 (One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UEs)

3.1.3 MCS for 4 MIMO layers case

The way forward from the previous meeting is outlined below.

MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

- Agreement: Down select the following two options in the next RAN4 meeting

- Option 1: MCS 20 (Table 2)

- Option 2: MCS 17 (Table 1)

There are two proposals currently under consideration for MCS choices in the context of 4 MIMO layers. Our simulation results suggest that peak throughput is achieved around 26dB with MCS20 (256QAM). Accounting for the impairment margin and given the SNR cap for the EVM limit (3% for 256QAM), MCS 20 remains feasible with 4L scenario. Therefore, we propose using MCS 20 from MCS Table 2 for the 4 MIMO layers scenario.

Proposal 6: Support option 2 (MCS 20 from Table 2)

3.2 Performance requirements for 6 Layers

3.2.1 DMRS configuration for 6 MIMO layers case

DMRS configuration for 6 MIMO layers case

- Proposals

  - Option 1: Rel-15 Type 1 DMRS 2+2

  - Option 2 : Rel-18 eType1 DMRS 1+1

The DMRS configuration for 6 MIMO layers case was briefly discussed during the last meeting. The way forward is outlined below.

We note that Rel-18 eType1 DMRS is likely to become more prevalent in advanced 5G use cases due to its reduced overhead and better support for higher-order MIMO. Additionally, RAN4 has already introduced requirements for eType1 DMRS during the Rel-18 timeframe [2]. Additionally, based on the simulation results provided in our contribution for RAN4#114 [3], it is evident that Rel-18 eType1 DMRS offers comparable performance with just one frontloaded DMRS symbol compared to the Rel-15 2+0 case. It also provides robustness in high Doppler scenarios due to the additional DMRS symbol. While the Rel-15 2+2 DMRS configuration could be considered for 6Rx requirements, it is important to note that this configuration significantly increases the DMRS overhead, resulting in lower peak throughput. After evaluating all these options and analyzing their pros and cons, we believe that eType1 DMRS should be considered for 6-layer performance requirements for 6Rx devices.

Proposal 7: Support option 2 (Rel-18 eType1 DMRS 1+1)

3.3 SDR requirements for 6Rx

3.3.1 Static channel and precoding for 6Rx

During the previous meeting, static channel matrices for 1T6R, 2T6R, and 4T6R configurations were finalized. However, the static channel matrix for the 8T6R case remains undecided. Two proposals are currently under consideration. Our position is that applying a scaling factor to the static channel matrix is unnecessary, as the precoders will be normalized for power allocation on the base station side.

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

    - Option 1:

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

    - Option 2:

- H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 8: Support option 1 (without scaling of the static channel matrix).

4. SDR Requirements for 6Rx

4.1 SDR requirements for 1024QAM with 4 layers

SDR requirements for 1024QAM

- Agreement

- Define SDR requirements for 1024QAM with 2 layers

- FFS Define SDR requirements for 1024QAM with 4 layers

During the last meeting, RAN4 agreed to introduce SDR requirements for 1024QAM with 2 layers. However, the decision on whether to introduce SDR requirements for 1024QAM with 4 layers was deferred for future study. The outlined way forward is as follows:

We believe that the SNR cap is too low due to EVM limitations for meaningful 1024QAM tests with 4 layers. Therefore, we propose not to introduce SDR requirements for 1024QAM with 4 layers.

Proposal 9: Support option 1 (Do not introduce SDR requirements for 1024QAM with 4 layers).

5. CSI Requirements for 6Rx

5.1 CQI test for 6Rx

During the last RAN4 meeting, the introduction of CQI reporting requirements for 6Rx was discussed. Multiple proposals with several test scenarios were considered and subsequently narrowed down.. The way forward is as follows.

CQI test for 6Rx

- Proposals

- Option 1: Not define CQI test

- Option 2: Only define one CQI requirements with rank 4 under static channel

We believe that 6Rx can support higher CQI (and hence MCS levels), even at rank 4 when compared to 4Rx alone. This justifies the rationale for introducing CQI reporting requirements.

Proposal 10: Support option 2 (only define one CQI requirement with rank 4 under static channel).

5.2 RI test for 6Rx

During the last meeting, RAN4 briefly discussed whether to introduce RI reporting requirements for 6Rx. However, no agreements were reached. The way forward is as follows.

RI test for only Rank 6

- Proposals

- Option 1: Not define RI test for 6 layers

- Option 2: Define RI test for 6 layers

We note that there were no RI tests with 6 layers for 8Rx WI during Rel-18. Furthermore, the feasibility study for 6-layer handheld requirements found that rank 6 is feasible for 6Rx, even for handheld UE devices. Therefore, it would be justified to consider introducing an RI test with 6 layers for 6Rx FWA UE requirements.

Proposal 11: Support option 2 (Define RI test for 6Rx)

6. Conclusions

In this paper, we share our perspectives on the remaining open issues regarding test scopes and simulation assumptions for the 6Rx demodulation and CSI reporting requirements. The following proposals have been made.

Proposal 1: Support option 3 (Postpone the discussion after RAN2 finishes the UE capability signaling design)

Proposal 2: Postpone the discussion on the applicability rule for different number of Rx antenna ports after UE capability signaling and requirements have been defined.

Proposal 3: Support option 1 (Add a new clause for 6Rx, such as 5.2.5 6Rx Requirements)

Proposal 4: Support option 3 (α = 0 and β = 0)

Proposal 5: Support option 1 (One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UEs)

Proposal 6: Support option 2 (MCS 20 from Table 2)

Proposal 7: Support option 2 (Rel-18 eType1 DMRS 1+1)

Proposal 8: Support option 1 (without scaling of the static channel matrix).

Proposal 9: Support option 1 (Do not introduce SDR requirements for 1024QAM with 4 layers).

Proposal 10: Support option 2 (only define one CQI requirement with rank 4 under static channel).

Proposal 11: Support option 2 (Define RI test for 6Rx)

7. References

[1] R4-2508727, “Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx,” 3GPP TSG RAN WG4 Meeting #115, St. Julian’s, Malta, May 19 – 23, 2025.