# 3GPP TSG RAN WG4 Meeting#116	R4-2511596

# Bengaluru, India, Aug 25 – Aug 29, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for RedCap and eRedCap UEs over NR-NTN

Agenda item:	7.27.7

Document for:	Discussion

# 1. Introduction

During the last RAN4 meeting, demodulation performance requirements for RedCap and eRedCap devices were discussed [1]. This paper outlines our perspectives on the remaining open issues.

2. Demodulation Requirements for RedCap UEs

2.1 PDSCH demodulation requirements for RedCap

PDSCH demodulation requirements for RedCap

Agreement:

For 2Rx UEs,

- Reuse Rel-17 NTN PDSCH demodulation requirements.

  - Companies can check whether all the Rel-17 NTN features can be applicable for RedCap.

Way forward:

For 1Rx UE,

- FFS whether to add new PDSCH requirements with NTN channel or not.

In the last RAN4 meeting, it was agreed that the NTN PDSCH demodulation requirements defined in Rel-17 could be reused for 2Rx RedCap UEs. The way forward is outline below.

In our view, NTN-specific parameters such as K_offset are expected to have minimal impact on demodulation performance requirements. However, the number of HARQ processes can have a non-negligible effect on performance and warrants further evaluation.

Proposal 1: When reusing NTN requirements for RedCap UEs with 2Rx, the impact of the number of HARQ processes should be considered.

Unlike 2Rx UEs, there are no existing 1Rx NTN requirements defined in Rel-17. Therefore, we believe it may be necessary to evaluate PDSCH performance requirements for RedCap UEs under NTN channel models to ensure alignment with realistic deployment conditions.

Proposal 2: Evaluate PDSCH requirements using NTN channel models for 1Rx RedCap UEs.

2.2 PDSCH test case for 1Rx RedCap UE (if defined)

Test case of PDSCH for RedCap 1Rx (if defined)

Way forward:

- Option 1:

  - MCS4, NTN-TDLA100-200, 1x1, Disabled HARQ

  - MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes

  - MCS4, NTN-TDLA100-200, 1x1, 32 HARQ processes

- Option 2:

  - MCS4, NTN-TDLA100-200, 1x1, 52RBs

  - MCS13, NTN-TDLC5-200, 1x1, 52RBs

- Option 3: Similar test configurations as the existing PDSCH cases

Several proposals were presented regarding the MCS configuration for 1Rx RedCap UEs. The way for is outlined below.

While it remains uncertain whether RAN4 will define specific requirements for 1Rx RedCap UEs, our view is that only mandatory features, such as support for 16 HARQ processes, should be considered. This approach helps minimize the testing burden while maintaining alignment with essential functionality.

Observation 1: Limit the evaluation of 1Rx RedCap UEs to mandatory features, such as only considering 16 HARQ processes, to reduce test complexity.

Proposal 3: Support option 1 with MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes.

3. Demodulation Requirements for eRedCap UEs

3.1 PDSCH demodulation requirements for eRedCap UEs

PDSCH demodulation requirements for eRedCap

Way forward:

For 2Rx UEs,

- Option 1:

  - Add new PDSCH requirements with BW reduction (25RB) with NTN channel.

- Option 2:

  - Reuse Rel-17 NTN PDSCH demodulation requirements without BW reduction.

    - Applicable test cases (TBS ≤ 10,000 bits): TS 38.101-5 Table 8.2.1.2.2.1.1-3 Tests 1-1, 1-3, and 1-4.

For 1Rx UE,

- FFS whether to add new PDSCH requirements with NTN channel or not.

The way forward for this issue is outlined below.

We note that eRedCap UEs are designed for low-complexity, low-power applications, such as IoT and NTN scenarios. Reducing the baseband bandwidth (e.g., to 25RB) helps minimize power consumption, reduce hardware complexity, and lower cost, which are critical for eRedCap deployments. Therefore, we think it is more practical to consider BW reduction for eRedCap UEs.

Proposal 4: Support option 1 for 2Rx UE (Add new PDSCH requirements with BW reduction i.e., 25RB with NTN channel).

Similar to the 1Rx RedCap UE, we think that it may be necessary to evaluate PDSCH performance requirements for eRedCap UEs under NTN channel models to ensure alignment with realistic deployment conditions.

Proposal 5: Evaluate PDSCH requirements using NTN channel models for 1Rx eRedCap UEs.

3.2 PDSCH scheduling for HD-FDD eRedCap UEs

PDSCH scheduling for HD-FDD

Way forward:

- Option 1: Define new reference channel for HD-FDD (e)RedCap with the same performance requirements as for FDD.

The way forward for this issue is outlined below.

It is important to note that the slot pattern used for PDSCH scheduling differs between HD-FDD and FD-FDD modes. Consequently, it is necessary to define a new reference channel specifically for HD-FDD eRedCap UEs to ensure accurate and consistent performance evaluation.

Proposal 6: Support option 1 (Define new reference channel for HD-FDD (e)RedCap with the same performance requirements as for FDD).

3.3 PDSCH test case for 1Rx and 2Rx eRedCap UEs (if defined)

Test case of PDSCH for eRedCap 1Rx (if defined)

Way forward:

- Option 1:

  - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RB), Disabled HARQ

  - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes

  - MCS4, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 32 HARQ processes

  - MCS13, NTN-TDLA100-200, 1x1, without reduced BB BW (40RBs), 16 HARQ processes

- Option 2:

  - MCS4, NTN-TDLA100-200, 1x1, with reduced BB BW (25RBs)

  - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RBs)

- Option 3: Similar test configurations as the existing PDSCH cases

Test case of PDSCH for eRedCap 2Rx (if defined)

Way forward:

- Option 1:

  - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RB), Disabled HARQ

  - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes

  - MCS4, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 32 HARQ processes

  - MCS13, NTN-TDLA100-200, 1x2, without reduced BB BW (40RBs), 16 HARQ processes

- Option 2:

  - MCS4, NTN-TDLA100-200, 1x2, with reduced BB BW (25RBs)

  - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RBs)

- Option 3: Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

The way forward for this issue is outlined below.

Similar to RedCap UEs, our view is that mandatory features, such as support for 16 HARQ processes, should be considered for eRedCap UEs as well.

Proposal 7: The following test cases to be considered for eRedCap UEs:

  - MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes for 1Rx eRedCap UE.

  - MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes 2Rx eRedCap UE.

4. Applicability of time-varying Doppler shift and propagation delay model for NGSO Scenarios

There remains an open question regarding the inclusion of the Rel-19 channel model in the context of RedCap and eRedCap UEs. We do not have a strong preference currently and are open to further discuss the feasibility of considering it for the NGSO scenarios.

Proposal 8: Further discuss the feasibility of considering Rel-19 channel model for NGSO scenarios in the context of redcap/eRedCap UEs.

5. Conclusions

In this paper, we present our perspectives on the demodulation performance requirements for RedCap and eRedCap UEs over NR-NTN. The following proposals have been made.

Proposal 1: When reusing NTN requirements for RedCap UEs with 2Rx, the impact of the number of HARQ processes should be considered.

Proposal 2: Evaluate PDSCH requirements using NTN channel models for 1Rx RedCap UEs.

Observation 1: Limit the evaluation of 1Rx RedCap UEs to mandatory features, such as only considering 16 HARQ processes, to reduce test complexity.

Proposal 3: Support option 1 with MCS13, NTN-TDLC5-200, 1x1, 16 HARQ processes.

Proposal 4: Support option 1 for 2Rx UE (Add new PDSCH requirements with BW reduction i.e., 25RB with NTN channel).

Proposal 5: Evaluate PDSCH requirements using NTN channel models for 1Rx eRedCap UEs.

Proposal 6: Support option 1 (Define new reference channel for HD-FDD (e)RedCap with the same performance requirements as for FDD).

Proposal 7: The following test cases to be considered for eRedCap UEs:

- MCS13, NTN-TDLC5-200, 1x1, with reduced BB BW (25RB), 16 HARQ processes for 1Rx eRedCap UE.

- MCS13, NTN-TDLC5-200, 1x2, with reduced BB BW (25RB), 16 HARQ processes 2Rx eRedCap UE.

Proposal 8: Further discuss the feasibility of considering Rel-19 channel model for NGSO scenarios in the context of redcap/eRedCap UEs.

6. References

[1] R4-2508755, “Way Forward for [115][333] NR_NTN_Ph3_demod,” 3GPP TSG RAN WG4 Meeting #115, St. Julian’s, Malta, May 19 – 23, 2025.