3GPP TSG-RAN WG4 Meeting # 116	R4-2511609

Bengaluru City, IN, Aug 25 – Aug 29, 2025

Agenda Item: 7.20.4.1

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for UE-initiated event-driven beam management

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. The WID [1] is divided into two parts to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “UE-initiated/event-driven beam management”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- RRM requirement on UE-initiated/event-driven beam management

- Capability on UE-initiated/event-driven beam management

## 2.1 RRM requirement on UE-initiated/event-driven beam management

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement” - To check with RAN1 whether the parameter can be configured if RAN1 identify any issue

In legacy, the number of samples (M) is decided by whether RAN1 would have time domain restriction for channel measurements. For example, M=1 if higher layer parameter of time domain restriction (timeRestrictionForChannelMeasurement) is configured, and M=3 otherwise. In RAN4 we can reuse same values as legacy, it can be revisited if RAN1 make a different agreement otherwise.

Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. If timeRestrictionForChannelMeasurement is not configured, measurement period is based on 3 sample.

In RAN4#114b, RAN4 reached the following agreement as captured in WF [3].

RAN4 #114b WF [3]: Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

There’s open issue on the P (sharing factor) for current beam and new beam in L1-RSRP measurement period. Since the L1-RSRP measurement for event trigger beam reporting has not much difference than legacy. RAN4 to consider reuse the P of SSB based or CSI-RS based L1-RSRP measurement in legacy. We provided our view that whether the same or different P used in L1-RSRP measurement for current beam and new beam as below.

P value is same for intra-cell SSB resources included both current and new beam since they are from the same SSB burst in our understanding.

P value is different for inter-cell SSB resources included both current and new beam, defined as

- P = PL1 sharing * Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

- P = PL1 sharing * Ntotal / Navailable in FR2 with Navailable > 0

  - Ntotal is the total number of SSB resource occasions including those overlapped with GAP occasions or SMTC occasions, and

  - Navailable is the number of SSB resource occasions that are not overlapped with any gap occasion nor any SMTC occasion.

  - Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is not overlapped SSB symbols for L3 measurement. Otherwise, Psharing factor = 3.

  - PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

P value is different for CSI-RS resources included both current and new beam, defined as

- P = PL1 sharing * Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

- P = PL1 sharing * Ntotal / Navailable in FR2 with Navailable > 0

  - Ntotal is the total number of CSI-RS resource occasions including those overlapped with GAP occasions or SMTC occasions, and

  - Navailable is the number of CSI-RS resource occasions that are not overlapped with any gap occasion nor any SMTC occasion.

  - Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside gap is not overlapped SSB symbols for L3 measurement. Otherwise, Psharing factor = 3.

  - PL1_sharing = 2, if CSI-RS resources from serving cell is configured for L1-RSRP measurement, and any symbol of the CSI-RSs from serving cell are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

Proposal 2: RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline:

- P value is same for intra-cell SSB resources included both current and new beam

- P value is different for inter-cell SSB resources included both current and new beam

- P value is different for CSI-RS resources included both current and new beam

Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > - Option 1: (Qualcomm, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 - Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19 TS 38.133 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

We prefer to define requirements for both no time window based and with time window based with eventInstanceCount-r19. In the case of “no time window”, this is the basic feature (M instance = 1), could discussed in issue 1-1. In the case of “time window with configurable M”, UE initiate beam reporting only if the number of instances for event 2 (i.e., the quality of new beam becomes a threshold value better than current beam) is larger than or equal to event instances threshold. Also, the evaluation should be done within time window. Otherwise, UE should re-evaluate whether the event condition is met since the time window is changed.

Whether to consider the uncertainty time of event condition is met to be part of event triggered reporting delay, we can either follow legacy L3 event triggered reporting principle (i.e., to exclude this uncertainty time) or to include the uncertainty time for UL reporting.

For the case of when time window is configured, during the configured time window there might be N number of instances to check for event 2, where N can be ≥ M event instances threshold. Based on RAN1 design, UE initiate beam reporting only if at least M number of event instances satisfy event 2. Therefore, if we want to define the requirements for this case, RAN4 need to consider the worst-case scenario. For example, when only M out of N instances meet event 2 condition, for which these M instances could occur anywhere within the configured time window as shown the figure below. There can be two ways to define the requirements for this case. One way is that the reporting delay with configured time window can reuse the same principle of L3 event triggered reporting in the legacy requirements. Another way is that the reporting delay can be defined based on the multiple (M) times of L1-RSRP measurement period within the time window.

Figure 1: the case of time window with configurable M

Proposal 3: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”:

Proposal 4: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window.

Proposal 5: For the case of time window with configurable M (event instances threshold):

- Option 1: Measurement reporting delay can re-use the same principle as L3 event triggered reporting defined in legacy.

- Option 2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

  - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

  - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > - Option 1: (Nokia, Ericsson, MTK) - No need to capture this scenario in RAN4 requirements - Option 2: (Xiaomi, Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

In our understanding, all the events evaluation is at least included indicated TCI state which is current beam. Therefore, all the requirements should be in the scope of indicated TCI state. If indicated TCI state is changed, then no requirement is applied.

Proposal 6: No requirement is applied if indicated TCI states is changed during the L1 measurement time.

## 2.2 Capability on UE-initiated/event-driven beam management

Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > - Option 1: (Samsung, Apple, Huawei, Ericsson) - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: (Nokia, MTK) - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9]. Issue 1-9: Compatibility of capabilities with LTM <Way forward > If such capabilities are defined in Issue 1-8, then discuss on: - Option 1: - The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO. - Option 2: - The capability for supported number of events triggered L1 measurement reporting is defined as common for Rel-19 mobility and Rel-19 MIMO. TS 38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NGRAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequencyvSS-RSRP, SS-RSRQ, SS-SINR, CSIRSRP, CSI-RSRQ, and CSI-SINR for NG-RANvinter-frequency cells. …; …; …. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode.

In legacy, capabilities for support of event triggered L3 measurement reporting had been specified in TS 38.133 section 9.1.4. For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

In our understanding, we could follow the similar principle to define the number of events supported for UE-initiated beam reporting in MIMO. Since the same RS type for current beam and new beam is agreed in RAN1. We suggest the following proposal to consider 3 events (event 1, event 2 and event 7) for intra-cell SSB resources, inter-cell SSB resources and CSI-RS resources.

Proposal 7: Define UE reported capabilities for numbers of event refer to legacy L3 (like legacy L3 in section 9.1.4).

- For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. If timeRestrictionForChannelMeasurement is not configured, measurement period is based on 3 sample.

Proposal 2: RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline:

- P value is same for intra-cell SSB resources included both current and new beam

- P value is different for inter-cell SSB resources included both current and new beam

- P value is different for CSI-RS resources included both current and new beam

Proposal 3: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”:

Proposal 4: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window.

Proposal 5: For the case of time window with configurable M (event instances threshold):

- Option 1: Measurement reporting delay can re-use the same principle as L3 event triggered reporting defined in legacy.

- Option 2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

  - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

  - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 6: No requirement is applied if indicated TCI states is changed during the L1 measurement time.

Proposal 7: Define UE reported capabilities for numbers of event refer to legacy L3 (like legacy L3 in section 9.1.4).

- For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2] R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1 (Samsung, RAN4#115)

[3] R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1(Samsung, RAN4#114b)