3GPP TSG-RAN4 Meeting #116	R4-2511612

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3038 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_CADC_R18_2BDL_xBUL-Core] CR to TS38.101-1-ia0: Correction to harmonic mixing MSD - CatF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions Inc., Anritsu, Qualcomm France. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_2BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR brings the corrections agreed in WF R4-2502956. There is no direct-hit for CA_n28-n78 harmonic mixing MSD requirements. The PC3 band n7 UL configuration for band n26 near-miss harmonic mixing MSD requirements are corrected as agreed in WF R4-2502956. Missing MSD requirements for CA_n14-n30 are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC3 Table 7.3A.4-4: - CA_n7-n26: corrected band n7 UL CBW to 25MHz and UL RB to 128 - CA_n14-n30: added missing MSD requirements and new Note 14, - CA_n28-n78: corrected MSD to 6.1dB, changed note to Note 10 and corrected to “UL1/DL5 near-miss”. PC2 Table 7.3A.4-4a-1: - CA_n28-n78: corrected MSD to 8.2dB, added Note 6, corrected to “UL1/DL5 near-miss” and removed second MSD test point to align with PC3 test point. PC1.5 Table 7.3A.4-4b CA_n28-n78: corrected MSD to 13.0dB, added Note 10, corrected to “UL1/DL5 near-miss” and removed second MSD test point to align with PC3 test point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CA_n14-n30, CA_n7-n26 and CA_n28-n78 harmonic mixing MSD requirements remain erroneous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

<UNCHANGED SECTIONS ARE SKIPPED>

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n1053 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | n713 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | n713 | 5 | 15 | 25 | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n3 | n5 | 5 | 15 | 25 | 5 | 4 | NOTE 7 | UL1/DL2 |
| n3 | n26 | 5 | 15 | 25 | 5 | 3.7 | NOTE 7 | UL1/DL2 |
| n7 | n263 | 25 | 15 | 128 | 5 | 2.0 | NOTE 10 | UL1/DL3 near miss |
| n7 | n71 | 5 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n25 | n41 | 5 | 15 | 6 | 10 | 1.3 | NOTE 9 | UL4/DL3 |
| n25 | n713 | 5 | 15 | 25 | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n25 | n713 | 5 | 15 | 25 | 20 | 15.6 | NOTE 4 | UL1/DL3 |
| n30 | n14 | 10 | 15 | 50 | 5 | 1.6 | NOTE 14 | UL1/DL3 near miss |
| n39 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 9 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 9 | UL4/DL3 |
| n40 | n203 | 10 | 15 | 25 | 5 | 27.8 | NOTE 4 | UL1/DL3 |
| n40 | n203 | 10 | 15 | 25 | 20 | 20.6 | NOTE 4 | UL1/DL3 |
| n40 | n283 | 10 | 15 | 25 | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n283 | 10 | 15 | 25 | 20 | 30.1 | NOTE 4 | UL1/DL3 |
| n41 | n53 | 10 | 15 | 25 | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 10 | 15 | 25 | 5 | 26.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 10 | 15 | 25 | 15 | 21.3 | NOTE 4 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 4.3 | NOTE 12 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 0.8 | NOTE 12 | UL3/DL4 |
| n41 | n48 | 10 | 15 | 6 | 5 | 12.3 | NOTE 9 | UL4/DL3 |
| n41 | n48 | 10 | 15 | 6 | 100 | 2.3 | NOTE 9 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 10 | 8.9 | NOTE 9 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 2.1 | NOTE 9 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 9.3 | NOTE 9 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 2.3 | NOTE 9 | UL4/DL3 |
| n46 | n7 | 20 | 15 | 25 | 5 | 10.9 | NOTE 7 | UL1/DL2 |
| n46 | n7 | 20 | 15 | 25 | 50 | 1 | NOTE 7 | UL1/DL2 |
| n46 | n48 | 20 | 15 | 12 | 5 | 26.8 | NOTE 2 | UL2/DL3 |
| n46 | n48 | 20 | 15 | 12 | 100 | 13.5 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 20 | 15 | 12 | 10 | 20.6 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 20 | 15 | 12 | 100 | 10.6 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 12 | 10 | 21.1 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 12 | 100 | 11.1 | NOTE 2 | UL2/DL3 |
| n48 | n12 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n48 | n12 | 10 | 15 | 25 | 15 | 18 | NOTE 5 | UL1/DL5 |
| n48 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n48 | n26 | 10 | 15 | 25 | 20 | 1 | NOTE 8 | UL1/DL4 |
| n48 | n29 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n48 | n29 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n2 | 10 | 15 | 25 | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 2.8 | NOTE 7 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 5.7 | NOTE 7 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 20 | 2.2 | NOTE 7 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 14.7 | NOTE 2 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 50 | 2.2 | NOTE 2 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 0.8 | NOTE 8 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 18 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 5.6 | NOTE 7 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.3 | NOTE 7 | UL1/DL2 |
| n77 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n77 | n28 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 11.4 | NOTE 5 | UL1/DL5 |
| n776 | n29 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n776 | n29 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 7.6 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n776 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 7 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 18 | NOTE 5 | UL1/DL5 |
| n78 | n2 | 10 | 15 | 25 | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n78 | n2 | 10 | 15 | 25 | 20 | 2.8 | NOTE 7 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 5 | 5.7 | NOTE 7 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 20 | 2.2 | NOTE 7 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n78 | n12 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n26 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 50 | 5 | 6.1 | NOTE 10 | UL1/DL5 near-miss |
| n78 | n40 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 10 | 15 | 12 | 100 | 3.6 | NOTE 2 | UL2/DL3 |
| n78 | n67 | 10 | 15 | 25 | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n67 | 10 | 15 | 25 | 10 | 27.8 | NOTE 5 | UL1/DL5 |
| n79 | n5 | 10 | 15 | 25 | 5 | 27.5 | NOTE 5 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 | 5 | 25 | NOTE 5 | UL1/DL5 |
| n96 | n48 | 20 | 15 | 25 | 5 | 31 | NOTE 7 | UL1/DL2 |
| n96 | n48 | 20 | 15 | 25 | 100 | 17.5 | NOTE 7 | UL1/DL2 |
| n102 | n13 | 20 | 15 | 25 | 5 | 30 | NOTE 4 | UL1/DL3 |
| n104 | n78 | 20 | 15 | 25 | 10 | 29 | NOTE 7 | UL1/DL2 |
| n104 | n78 | 20 | 15 | 25 | 100 | 18.8 | NOTE 7 | UL1/DL2 |
| NOTE 1: Void. NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 9: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75 and FUL_lowLB+BWChannelLB/2≤fULLB≤FUL_highLB-BWChannelLB/2 with fULLB the UL carrier frequency and BWChannelLB the channel bandwidth configured in the lower band, both in MHz. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band NOTE 11: Void. NOTE 12: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 13: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 14: The requirements should be verified for the highest NR ARFCN of the affected DL (lower) band and for the lowest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 6 | 10 | 2.213 3.214 | NOTE 8 | UL4/DL3 |
| n25 | n713 | 5 | 15 | 25 | 5 | 29.513 34.514 | NOTE 7 | UL1/DL3 |
| n25 | n713 | 5 | 15 | 25 | 20 | 18.213 23.314 | NOTE 7 | UL1/DL3 |
| n39 | n41 | 5 | 15 | 6 | 5 | 10.6 | NOTE 8 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 6 | 100 | 4.9 | NOTE 8 | UL4/DL3 |
| n40 | n28 | 10 | 15 | 25 | 5 | 40.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 10 | 15 | 25 | 20 | 33.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 29.3 | NOTE 7 | UL1/DL3 |
| n41 | n186 | 10 | 15 | 25 | 15 | 24.3 | NOTE 7 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 6.1 | NOTE 9 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 1.5 | NOTE 9 | UL3/DL4 |
| n41 | n77 | 10 | 15 | 6 | 10 | 11.6 | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 3.6 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 12.1 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 3.8 | NOTE 8 | UL4/DL3 |
| n77 | n2 | 10 | 15 | 25 | 5 | 9.2 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 4.4 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 17.7 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 50 | 3.6 | NOTE 3 | UL2/DL3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 7.9 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.5 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 14.1 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 10 | 30.8 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 10.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n772 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 4 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 21 | NOTE 1 | UL1/DL5 |
| n78 | n3 | 10 | 15 | 25 | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n78 | n5 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n5 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 1.5 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 50 | 5 | 8.2 | NOTE 15 | UL1/DL5 near-miss |
| n78 | n40 | 10 | 15 | 12 | 10 | 14.5 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n79 | n8 | 10 | 15 | 25 | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: Void NOTE 11: Void. NOTE 12: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 13: Applicable to UE’s supporting PC2 with 1Tx NOTE 14: Applicable to UE’s supporting PC2 with 2Tx NOTE 15: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a-2: Void

Table 7.3A.4-4b: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 10 | 15 | 25 | 5 | 43.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 10 | 15 | 25 | 20 | 36.3 | NOTE 7 | UL1/DL3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 10.5 | NOTE 6 | UL3/DL4 |
| n41 | n39 | 10 | 15 | 8 | 40 | 3.0 | NOTE 6 | UL3/DL4 |
| n41 | n77 | 10 | 15 | 6 | 10 | 14.5 | NOTE 8 | UL4/DL3 |
| n41 | n77 | 10 | 15 | 6 | 100 | 5.5 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 10 | 14.9 | NOTE 8 | UL4/DL3 |
| n41 | n78 | 10 | 15 | 6 | 100 | 5.8 | NOTE 8 | UL4/DL3 |
| n77 | n2 | 10 | 15 | 25 | 5 | 11.8 | NOTE 4 | UL1/DL2 |
| n77 | n2 | 10 | 15 | 25 | 20 | 6.5 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 5 | 10.7 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 10 | 15 | 25 | 40 | 1.4 | NOTE 4 | UL1/DL2 |
| n77 | n5 | 10 | 15 | 25 | 5 | 10.7 | NOTE 5 | UL1/DL4 |
| n77 | n5 | 10 | 15 | 25 | 20 | 2.6 | NOTE 5 | UL1/DL4 |
| n77 | n7 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n77 | n7 | 10 | 15 | 12 | 100 | 5.5 | NOTE 3 | UL2/DL3 |
| n77 | n12 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n12 | 10 | 15 | 25 | 15 | 24 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 | 10 | 33.8 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 | 10 | 33.8 | NOTE 1 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 | 5 | 10.6 | NOTE 4 | UL1/DL2 |
| n77 | n25 | 10 | 15 | 25 | 40 | 0.8 | NOTE 4 | UL1/DL2 |
| n77 | n28 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 10 | 15 | 25 | 30 | 17 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n772 | n29 | 10 | 15 | 25 | 10 | 33.8 | NOTE 1 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 | 5 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 12 | 10 | 13 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 10 | 15 | 12 | 100 | 7.9 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n77 | n41 | 10 | 15 | 12 | 100 | 7.9 | NOTE 3 | UL2/DL3 |
| n772 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 4 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 | 5 | 37 | NOTE 1 | UL1/DL5 |
| n77 | n85 | 10 | 15 | 25 | 15 | 24 | NOTE 1 | UL1/DL5 |
| n78 | n3 | 10 | 15 | 25 | 5 | 10.7 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 10 | 15 | 25 | 40 | 1.4 | NOTE 4 | UL1/DL2 |
| n78 | n8 | 10 | 15 | 25 | 5 | 10.6 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 10 | 15 | 25 | 20 | 2.6 | NOTE 5 | UL1/DL4 |
| n78 | n26 | 10 | 15 | 25 | 5 | 10.1 | NOTE 5 | UL1/DL4 |
| n78 | n28 | 10 | 15 | 50 | 5 | 13 | NOTE 10 | UL1/DL5 near-miss |
| n78 | n40 | 10 | 15 | 12 | 10 | 17.5 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 10 | 15 | 12 | 100 | 7.9 | NOTE 3 | UL2/DL3 |
| n79 | n8 | 10 | 15 | 25 | 5 | 31.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 6: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75 and FUL_lowLB+BWChannelLB/2≤fULLB≤FUL_highLB-BWChannelLB/2 with fULLB the UL carrier frequency and BWChannelLB the channel bandwidth configured in the lower band, both in MHz. NOTE 9: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

<END OF CHANGES>