3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2511620

# Bengaluru, India, 25th – 29th August, 2025

Title: 	Input on NR TDD 4.9GHz Band for US Operation

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.14.1 General aspects and work plan

Document for:	Approval

# 1	Background

In RP#108, a new spectrum WI [1] targeting Release 20 was agreed to introduce a new 4.9GHz TDD band for US operation. In this contribution, we discuss the opportunity to reuse the work on Band n79 to enable the Band n79 hardware reuse for the UE.

# Discussion

## 2.1 WI objectives

The WI [1] has a study phase in Release 19 but targets Release 20, the objectives are copied below.

A one quarter study phase to identify and document coexistence requirements for the new band:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

    - Identify adjacent spectrum protection requirements

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

    - Identify other technologies operating in the 4940-4990 MHz band

      - Identify potential coexistence mechanism(s)

        - Frequency division sharing

        - Geographic separation

        - TDD synchronization

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

Following the completion of the study phase, the core objectives of this WI are:

-	Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz to support the following SCS and channel bandwidths:

| SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

- Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band.

- Address potential BS and UE co-existence issues, if any.

  - Specify band specific BS emissions requirements if needed

  - Specify UE additional emission limits and A-MPR if needed

- Specify the BS RF requirements.

- Specify the UE RF requirements for the new bands. Reuse NR band n79 UE hardware assumption if possible.

- Update the related technical specifications to include support for the new band

- CRs shall not be approved until ongoing litigation is resolved to avoid specification rework and potential NBC changes

Note: The new NR band will be introduced in a REL-independent way starting from REL-15.

Observation:

- This new band fully is a subset of Band n79 bandwidth and CBW except for 15MHz and 25MHz

- On the UE side it is unclear why those channel bandwidth could not simply be added to Band n79 as this would not require much work (this does not introduce a new lowest or highest CBW)

- In the same spirit, if coexistence with adjacent systems require A-MPR or to restrict the frequency range, this could be covered by an NS

- Even if filtering is required on the BS side for coexistence and possibly require a new band, this does not mean that the UE could not at least reuse the Band n79 hardware to support the spectrum in the US.

## 2.2 Benefits of Band n79 reuse on the UE side

More and More UEs support Band n79 for Japan and China including for UEs developed for Europe and the US for roaming purpose. In the high-end UEs, 2Tx, UL MIMO, HPUE features are also making their way into Band n79. So, although the ecosystem may not be as wide as for Band n77/48 or Band n41, there is more and more support for it and adding the US operation in 4940-4990 MHz would further consolidate it.

It should also be noted that the Band n79 ecosystem also supports operation with Band n77/78, especially if no SimRx/Tx is required.

Proposal for the study phase on the UE side:

- Use Band n79 definition and specification as the default input for the new band UE requirements on the UE side to enable full or partial reuse of the Band n79 hardware in the phone.

- Based on the study, consider if a new band should be introduced or whether Band n79 can be used on the UE side with the proper additions.

# Conclusion

In this contribution, we suggest that Band n79 UE hardware and were possible, the Band n79 definition and specification is reused and is the starting point for the new TDD 4940-4990MHz band.

Proposal for the study phase on the UE side:

- Use Band n79 definition and specification as the default input for the new band UE requirements on the UE side to enable full or partial reuse of the Band n79 hardware in the phone.

- Based on the study, consider if a new band should be introduced or whether Band n79 can be used on the UE side with the proper additions.

# References

[1] RP-251887 New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu Limited ​, RP#108