3GPP TSG-WG4 Meeting #116	R4-2511625

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

# Title:	Corrections to UE co-existence for n28 B28 CA28 and Japan n41

# Agenda Item:	5.3.7

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution proposes corrections to the UE coexistence spurious emission requirements for Band 28, Band n28, and CA_28. Corrections to Band n41, Note 49 are also discussed.

- Discussion

  - Corrections to the Rel-19 Band 28/n28 UE coexistence requirements

Table 1 compares the Rel-19 spurious emissions requirements for UE co-existence of Band 28 and Band n28.

Table 1: Rel-19 B28 and n28 spurious emission requirements for UE co-existence.

| Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA B28 Protected band | NR n28 Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n100, n101 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n100, n101, n109 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| E-UTRA Band 1 | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
| E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73 NR Band n79, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
| E-UTRA Band 11, 21 | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
| Frequency range | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
| Frequency range | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
| Frequency range | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
| Frequency range | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
| Frequency range | Frequency range | 773 | - | 803 | -50 | 1 |  |
| Frequency range | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. Applicable when the assigned E-UTRA carrier is confined within 715 MHz and 718 MHz and when the channel bandwidth used is 3 MHz. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. Applicable when the assigned E-UTRA carrier is confined within 715 MHz and 718 MHz and when the channel bandwidth used is 3 MHz. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

    - Rel-19 Corrections to Bands 28 and n28 UE Coexistence Requirements

Regarding the protection of Bands n100 and n101, the –50dBm/MHz requirements for Bands 28 and n28 UE are relaxed per Note 2. However, there is no harmonic relation between Bands 28 and n28 and Bands n100 and n101.

Observation 1: Bands n100 and n101 are not harmonically related to Bands 28 and n28. Note 2 does not apply to these requirements.

    - Rel-19 Corrections to Band 28 UE Coexistence Requirements.

Observation 2: Band 73 is not harmonically related to Band 28. Note 2 does not apply to this requirement.

Observation 3: Band n109 is protected by a Band n28 UE but not protected by a Band 28 UE. Band 28 is harmonically related to Band n109. The –50dBm/MHz protection requirement per Note 2 is missing.

Observation 4: Bands n105, 39, and 71 are protected by a Band n28 UE but not protected by a Band 28 UE. The 
–50dBm/MHz protection requirement is missing.

Proposal 1: Consider adopting the corrections in red for the Rel-19 Band 28 spurious emission UE coexistence requirements.

| E-UTRA band | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n100, n101, n109 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73, 87, 88 NR Band n79, n100, n101, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

    - Rel-19 Corrections to Band n28 UE Coexistence Requirements.

# Observation 5: Bands 87 and 88 are protected by a Band 28 UE but not protected by a Band n28 UE. The 
–50dBm/MHz protection requirement is missing.

Proposal 2: Consider adopting the corrections in red for the Rel-19 Band n28 spurious emission UE co-existence requirements.

| E-UTRA band | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n100, n101, n109 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73, 87, 88 NR Band n79, n100, n101, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

  - Corrections to CA_28 UE Coexistence Requirements

CA_28 spurious emission requirements need to be aligned with those of B28.

Proposal 3: Port the B28 corrections of proposal 1 to the CA_28 requirements.

  - Corrections to n41 UE Coexistence Requirements

The following observations are drawn from the comparison of Band B41 and Band n41 spurious emissions requirements shown in Table 2.

Observation 6: B41 protects Band 9 DL range while n41 does not. It is not clear why B41 should protect B9 if Band n41 does not.

Observation 7: Note 30 applies to B41 protection requirements of DL bands 9, 11, 18, 19, 21 and a frequency range of 1884.5 to 1915.7 MHz. Note 30 was voided for n41/TS 38.101-1. It is not clear why should this note be kept for B41.

Observation 8: It seems that Note 49 for Band n41 was directly copied from B41 Note 46. However, n41 supports CBW greater than the LTE 20 MHz maximum CBW and:

- The Note 46/49 carrier configuration restrictions in range 2545 to 2575 MHz can only apply for CBW up to 20 MHz,

- For Japan (range 2545 to 2575 MHz), both 25 MHz and 30 MHz CBW could, in theory, be deployed. For these CBW:

  - Note 49 does not apply to CBW > 20 MHz, and

  - NS_47 is specified only 30 MHz CBW.

  - Note 5 of NS_47 refers to the UE coexistence requirements for CBW < 30 MHz but does not clarify how to handle 25 MHz CBW.

Table 2: Rel-19 B41 and n41 spurious emission requirements for UE coexistence.

| E-UTRA B41 Spurious emission requirements |  |  |  |  |  |  |  |  |  | NR n41 Spurious emission requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE | Protected band | Frequency range (MHz) |  |  |  |  |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) |  |  | NOTE |
| E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103, 106 NR Band n77, n78, n105 | FDL_low | - | FDL_high | -50 |  | 1 |  |  |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 52, 54, 65, 66, 70, 71, 73, 74, 85, 103, 106 NR Band n77, n78, n105 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | E-UTRA Band 40 | FDL_low |  | - |  | FDL_high |  | -40 |  |  | 1 |  |  |  |  |  |
| NR Band n79 | FDL_low | - | FDL_high | -50 |  | 1 |  | 2 |  | NR Band n79 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  | 2 |  |
| E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 |  | 1 |  | 30 |  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  |  | 1 |  |  |  |  |
| Frequency range | 1884.5 |  | 1915.7 | -41 |  | 0.3 |  | 8, 30 |  | Frequency range |  | 1884.5 |  |  |  | 1915.7 |  | -41 |  |  | 0.3 |  |  | 8 |  |
| Frequency range | 2530 | - | 2535 | -25 |  | 1 |  | 46 |  | Frequency range |  | 2530 |  | - |  | 2535 |  | -25 |  |  | 1 |  |  | 49 |  |
| Frequency range | 2505 | - | 2530 | -30 |  | 1 |  | 46 |  | Frequency range |  | 2505 |  | - |  | 2530 |  | -30 |  |  | 1 |  |  | 49 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 46: Applicable when carrier is contained within 2545 – 2575 MHz in Japan. Carrier shall be confined so that there is at least BWChannel separation between 2535 MHz and lower BWChannel edge in the current release. With this BWChannel placement the requirement is covered by general SEM and the spurious emission limits. |  |  |  |  |  |  |  |  |  | NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 30: Void. NOTE 49: Applicable when contained within 2545 – 2575 MHz in Japan. Channel bandwidth shall be confined so that there is at least BWChannel separation between 2535 MHz and lower BWChannel edge in the current release. With this BWChannel placement the requirement is covered by general SEM and the spurious emission limits. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: Based on observation 6 and 7, and for consistency with Band n41 requirements, adopt the following changes in Table 6.6.3.2-1 B41 UE coexistence requirements:

- Remove protection of DL Band 9,

- Remove Note 30 and void this note.

Proposal 5: Based on observation 8, adopt the following correction to Note 49 in TS 38.101-1 Table 6.5.3.2-1.

NOTE 49:   Applicable for channel bandwidth up to 20MHz when contained within 2545 – 2575 MHz in Japan. Channel bandwidth shall be confined so that there is at least BWChannel separation between 2535 MHz and lower BWChannel edge in the current release. With this BWChannel placement the requirement is covered by general SEM and the spurious emission limits.

Proposal 6: Based on observation 8, adopt the following correction to the Note 5 of “Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)”:

NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. PC1 operation is not allowed. BWChannel less than 30 MHz 25 MHz are addressed in Table 6.5.3.2-1.

- Conclusion

This contribution summarizes the changes proposed in companion CR [xx] where we make the following observations and proposals.

# Observation 1: Bands n100 and n101 are not harmonically related to Bands 28 and n28. Note 2 does not apply to these requirements.

# Observation 2: Band 73 is not harmonically related to Band 28. Note 2 does not apply to this requirement.

# Observation 3: Band n109 is protected by a Band n28 UE but not protected by a Band 28 UE, and Band 28 is harmonically related to Band n109. The –50dBm/MHz protection requirement per Note 2 is missing.

# Observation 4: Bands n105, 39, and 71 are protected by a Band n28 UE but not protected by a Band 28 UE. The 
–50dBm/MHz protection requirement is missing.

Proposal 1: Consider adopting the corrections in red for the Rel-19 Band 28 spurious emission UE coexistence requirements.

| E-UTRA band | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n100, n101, n109 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73, 87, 88 NR Band n79, n100, n101, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

# Observation 5: Bands 87 and 88 are protected by a Band 28 UE but not protected by a Band n28 UE. The 
–50dBm/MHz protection requirement is missing.

Proposal 2: Consider adopting the corrections in red for the Rel-19 Band n28 spurious emission UE coexistence requirements.

| E-UTRA band | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n100, n101, n109 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73, 87, 88 NR Band n79, n100, n101, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

Proposal 3: Port the B28 corrections of proposal 1 to the CA_28 requirements.

Observation 6: B41 protects Band 9 DL range while n41 does not. It is not clear why B41 should protect B9 if Band n41 does not.

Observation 7: Note 30 applies to B41 protection requirements of DL bands 9, 11, 18, 19, 21, and frequency range 1884.5 to1915.7 MHz. Note 30 was voided for n41/TS 38.101-1. It is not clear why this note should be kept for B41 since these protection requirements are specific to Japan.

Observation 8: It seems that Note 49 for Band n41 was directly copied from B41, Note 46. However, n41 supports CBW greater than the LTE 20 MHz maximum CBW and:

- Note 46/49 carrier configuration restrictions in range 2545 to 2575 MHz can only apply for CBW up to 20 MHz,

- For Japan (range 2545 to 2575 MHz), both 25 MHz and 30 MHz CBW could, in theory, be deployed. For these CBW:

  - Note 49 does not apply to CBW > 20 MHz, and

  - NS_47 is specified only 30 MHz CBW.

  - For CBW > 20 and < 30 MHz (i.e., 25 MHz), Note 5 of NS_47 refers to the UE coexistence requirements for CBW < 30 MHz but does not clarify how to handle 25 MHz CBW.

Proposal 4: Based on observation 6 and 7, and for consistency with Band n41 requirements, adopt the following changes in Table 6.6.3.2-1 B41 UE coexistence requirements:

- Remove protection of DL Band 9,

- Remove Note 30 and void this note.

Proposal 5: Based on observation 8, adopt the following correction to Note 49 in TS 38.101-1 Table 6.5.3.2-1.

NOTE 49:   Applicable for channel bandwidth up to 20MHz when contained within 2545 – 2575 MHz in Japan. Channel bandwidth shall be confined so that there is at least BWChannel separation between 2535 MHz and lower BWChannel edge in the current release. With this BWChannel placement the requirement is covered by general SEM and the spurious emission limits.

Proposal 6: Based on observation 8, adopt the following correction to the Note 5 of “Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)”:

NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. PC1 operation is not allowed. BWChannel less than 30 MHz 25 MHz are addressed in Table 6.5.3.2-1.

# References