3GPP TSG-RAN WG4 Meeting #116	R4-25011633

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.24.6

Source:	Qualcomm Incorporated

Title: 		RRM performance requirements for LP-WUR

Document for:	Discussion

# Introduction

RAN4 has been working on defining the RRM requirements for LP WUR/WUS over the past few meetings and is getting close to complete the core part of WI [1]. In this paper, we discuss some performance aspects of LP-WUR.

# Test cases

In our view, there are two aspects of the LP-WUR:

(1) monitoring LP-WUS for paging monitoring and

(2) performing LP-WUR based measurements to evaluate entry/exit criteria for LP-WUS monitoring and measurements.

As we know, (1) is basically a replacement of Paging Early Indication (PEI) feature. The main thing that the network cares about here would be that the UE shouldn’t miss any paging message because of LP-WUR operation. Two things that may cause this are: LP-WUS missed-detections and/or long MR wake-up times. Now, we don’t have any RRM test-case(s) on PEI, and the missed detection performance is being (or will be) discussed the RF (or demod) room, so perhaps no need to discuss it in the RRM room. We may still need to verify that the MR wake-up delays, so that the UE doesn’t take longer than what it indicates as the UE capability of MR wake-up delay. Since the delay requirement is not introduced by RAN4, we are open to discuss whether we need to test it in RAN4. Considering the wake-up delays are separately defined for IDLE and CONNECTED mode, we further need to discuss if we need to test them for both modes.

Observation 1: The main thing network cares about for LP-WUR is that the UE shouldn’t miss any paging message.

Observation 2: UE may miss paging message either due to LP-WUS missed detections or longer MR wake-up delay.

Observation 3: There are no RRM test-cases defined for PEI, which the LP-WUS is designed to replace.

Observation 4: The missed-detection performance of LP-WUS is being discussed in RF/Demod room.

Proposal 1: No need to discuss LP-WUS missed detection or false-alarm rate performance for RRM.

Proposal 2: Discuss whether to introduce a test-case to verify the MR wake-up delay after receiving the LP-WUS.

Now let’s talk about the second aspect: LP-WUR based measurements and evaluations. In RRM, we currently test requirements on cell-reselection where the network wants to ensure that the UE performs cell-reselection within specified delays. With LP-WUR, as we know, if the WUR is operating either in the relaxed measurement mode or measurement offloading mode, it needs to evaluate the exit criteria to ensure that the UE does not stick around on the current cell even if it does not meet the cell reselection criteria. Note that the LP-WUR may also perform measurements to evaluate the entry criteria to RRM relaxation or RRM offloading mode, but these are not really testable as the UE cannot be forced to turn on the WUR and perform such evaluations. So we think what we can really test here are the following:

- UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode

- UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode.

Observation 5: The evaluations of entry conditions of LP-WUR are not testable because the UE cannot be forced to turn on the WUR at any point of time

Proposal 3: Introduce the following test cases where the exit conditions can be tested:

- UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode

- UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode.

# Conclusion

Observation 1: The main thing network cares about for LP-WUR is that the UE shouldn’t miss any paging message.

Observation 2: UE may miss paging message either due to LP-WUS missed detections or longer MR wake-up delay.

Observation 3: There are no RRM test-cases defined for PEI, which the LP-WUS is designed to replace.

Observation 4: The missed-detection performance of LP-WUS is being discussed in RF/Demod room.

Proposal 1: No need to discuss LP-WUS missed detection or false-alarm rate performance for RRM.

Proposal 2: Discuss whether to introduce a test-case to verify the MR wake-up delay after receiving the LP-WUS.

Observation 5: The evaluations of entry conditions of LP-WUR are not testable because the UE cannot be forced to turn on the WUR at any point of time

Proposal 3: Introduce the following test cases where the exit conditions can be tested:

- UE meets the cell reselection requirements with initial conditions being WUR operating in the full offloading mode

- UE meets the cell resection requirements with initial conditions being WUR operating in the relaxed measurement mode.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103