3GPP TSG-RAN WG4 Meeting #116	R4-2511639

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.4.2

Source: 	Qualcomm Incorporated

Title: 		RRM requirements for SBFD operation

Document for:	Discussion

# Introduction

In Release 19, one of the main objectives of the 3GPP work item on the Evolution of NR Duplex Operation: Sub-band Full Duplex (SBFD) [1] is to specify SBFD operation in TDD bands. Most of the Radio Resource Management (RRM) aspects related to SBFD have already been discussed, and several agreements have been reached. This paper focuses on the basic feature introduced for SBFD aware UEs and its impact on SSB based serving cell measurements.

# Basic feature for SBFD aware UEs

FG 60-1 Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures

  - Support of cell-common configuration of time and frequency location of SBFD subbands

  - Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols

  - Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling

  - Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1)

  - Support of separate CSI reports for SBFD and non-SBFD symbols

  - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols

  - Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols

  - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols

  - Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols

  - FFS: Whether to include two DL subband support in component 2

  - FFS: other component (which may be separate FG)

RAN1 has defined a basic feature for SBFD-aware UEs, supporting DL reception in only one DL subband and is currently evaluating whether these UEs should support DL reception in both DL subbands during SBFD symbols. If RAN1 decides not to support DL reception in both subbands, RAN4 must assess whether this restriction also applies to SSB-based measurements.

If such a restriction is applicable, it raises concerns for serving cell measurements. For instance, if a UE is configured to receive DL in one subband, but the SSBs used for measurements are transmitted in the other DL subband, the UE may be unable to receive those SSBs. This issue arises particularly when the UE’s active Bandwidth Part (BWP) does not include the subband carrying the SSBs (as illustrated in the figure below).

SSB

To address this, one straightforward approach is for the network to ensure that the UE is always scheduled in the DL subband that contains the SSB. If this cannot be guaranteed, the network could instead transmit a NCD-SSB in the active subband and instruct the UE to perform measurements based on the NCD-SSB rather than the CD-SSB.

An alternative solution is to allow the UE to perform measurements using measurement gaps, but this may lead to significant data interruptions and reduced efficiency.

Observation 1: A UE supporting the basic SBFD feature may only support DL reception in one DL subband during SBFD symbols.

Observation 2: It is unclear how such a UE can measure SSBs transmitted in the other DL subband.

Proposal 1: A UE supporting the basic SBFD feature may assume that DL scheduling always occurs in the subband where SSBs are transmitted. If this is not feasible, the network should configure NCD-SSBs in the active DL subband and indicate the UE to perform both L1 and L3 measurements based on the NCD-SSB.

# Conclusion

Observation 1: A UE supporting the basic SBFD feature may only support DL reception in one DL subband during SBFD symbols.

Observation 2: It is unclear how such a UE can measure SSBs transmitted in the other DL subband.

Proposal 1: A UE supporting the basic SBFD feature may assume that DL scheduling always occurs in the subband where SSBs are transmitted. If this is not feasible, the network should configure NCD-SSBs in the active DL subband and indicate the UE to perform both L1 and L3 measurements based on the NCD-SSB.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104