3GPP TSG-RAN4 Meeting #116	R4-2511643

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:		7.10.3.3

Source:				Keysight Technologies UK Ltd

Title:					TP to TR 38.762: on performance metrics

Document for:		Approval

# Introduction

This contribution is incorporating Proposal 1 from [1], endorsed in RAN4#115 WF [2]

Issue 3-2-2: device rotations per orientation Agreements: - Include the 4 device rotations per orientation in the definitions of the TMTs.

to update [3].

# Discussion

Proposal 1: approve the TP to correct TMT definition to include the 4 device rotations per orientation in the definition of the TMTs.

# References

- R4-2505844, On TMT and CTMT metric definitions, Keysight Technologies, 3GPP TSG-RAN4 Meeting #115, June 2025

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #115, May 2025

- TR 38.762, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

# 5	Performance metrics

Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations and for each of the L rotations per orientation, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e.,

TMTUMi=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMiTMTUMi=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMi

and

TMTUMa=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMa

TMTUMa=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMa

with m = 1 to M=4 and l = 1 to L=4.

Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF

-	CTMTUMi,10%=CDF10%TMTUMi

-	CTMTUMi,50%=CDF50%TMTUMi

-	CTMTUMi,90%=CDF90%(TMTUMi)

-	CTMTUMa,10%=CDF10%TMTUMa

-	CTMTUMa,50%=CDF50%TMTUMa

-	CTMTUMa,90%=CDF90%(TMTUMa)

Whether and how to combine the CTMT metrics for different routes/channel models is FFS.

<<< END OF CHANGES >>>