3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2511645

# Bengaluru, India, 25th – 29th August, 2025

Title: 	Skyworks Solutions Inc inputs to RAN4 Release 20 5GA and 6G work

Source: 	Skyworks Solutions, Inc.

Agenda Item:	11	New or revised WID/SID

Document for:	Approval

# 1	Background

In RP#108, we shared our high-level views on the Release 20 HPUE work for 5GA and 6G [1,2]. In this contribution, we share further details with the RAN4 group to discuss their interest for the coming RAN4 Release 20 work/study item decisions in next plenary.

# Discussion

## 2.1 HPUE options for Release 20 5G Advanced

Within the current NR specification there are already a number of HPUE cases that are covered:

- Single band and single CC power classes:

  - PC5 1Tx (20dBm) for NRU

  - PC3 default power class (23dBm) for FDD/TDD TN and NTN

    - 1Tx and 2Tx for NR with optional power boosting

    - 2Tx for NRU

  - PC2 (26dBm) for FDD/TDD TN (+NTN in Rel-19)

    - 1Tx and 2Tx for NR with optional power boosting

  - PC1.5 (29dBm) for TDD TN

    - 2Tx for NR (+NR-NTN in Rel-19)

    - 4Tx for FWA

  - PC1 1Tx (31dBm) for NR (FWA+IoT NTN in Rel-19)

  - Rel-19 is also introducing means to reduce MPR

- Intra-band contiguous and non-contiguous CC power classes:

  - PC3 default power class (23dBm) for 1Tx and 2Tx FDD/TDD TN

  - PC2 1Tx and 2Tx (26dBm) for FDD/TDD TN

  - PC1.5 (29dBm) introduced in Rel-19

    - 1Tx and 2Tx for contiguous case

    - 1Tx for non-contiguous case

- Inter-band power classes:

  - PC3 default power class (23dBm) for 1/2/3Tx with PC3 per band (PC5 for NR-U bands)

  - PC2 (26dBm) for 1/2/3Tx with PC3/PC2 per band

  - PC1.5 (29dBm) 1/2/3Tx with PC3/PC2/PC1.5 per band

Release 20 candidate HPUE topics builds on the above and a number of new HPUE cases were proposed by companies. A non-exhaustive list of proposals is copied here:

- PC1 with 4Tx UL for FWA. PC1 with 2Tx and UL-MIMO.

- PC1.5 FDD with 2Tx. PC1.5 with 1Tx. PC1.5 TDD and FDD with 3Tx

- PC1 with inter-band and intra-band UL CA (4Tx and 8Tx enhancements)

  - 4x26 dBm. 8x23 dBm. 2x29 dBm (New request for Rel-20)

  - Combination of 23 and 26 (and 29) dB PC1.5 with 4Tx or 8Tx for UL intra-band and inter-band CA

- Reduce UE MPR with relaxed Tx EVM requirements and advanced BS receiver for 64QAM and 256QAM. Reduce UE MPR with relaxed Tx EVM requirements for QPSK and 16QAM

- Specify ACLR of 26dB and corresponding MPR requirements for U6GHz (n104). Specify RF requirements for 200MHz CBW for U6GHz (n104)

- High power class framework

- Power class handling for CA (allowing maximum output power per band for different CA combos)

- Specify the UE RF requirements of MPR reduction by defining different regions corresponding to asymmetric ACLR.

- PC1.5/PC1 for handheld NR-NTN and IoT-NTN devices

- Higher power than PC1 for handheld NR-NTN and IoT-NTN devices

- PC1/PC2 for RedCap NTN UE

- NTN power domain enhancements

- MPR reduction with emission relaxation under NTN deployment.

The proposals above can be classified in a few categories:

- New power class configuration based on existing PA smartphone implementation

- New power class configuration based on PA smartphone implementation yet to be developed

- Power boosting, MPR improvements.

In our view, RAN4 should prioritize new power classes based on existing PA implementations and enabling as much MPR reduction and/or power boosting options as possible.

Going into some level of details, there are PC1 options that can use existing PA implementations in principle, but it should be within a feasible linearity target. Current PC1 definition calls for 37dB ACLR while PC3 is 30dB ACLR and PC2/PC1.5 are 31dB ACLR. Although it seems feasible to achieve 32dBm with 8x23dBm or 4x26dBm, it cannot be the case if the ACLR target is 37dB. However, the 1dB margin to the PC1 31dBm level, can be traded to a 3dB improvement in ACLR. Thus, a PC1 ACLR target of less or equal to 34dB is preferred and correspond to a similar absolute power interference than PC1.5.

Also, there are request for 1Tx PC1.5 implementations (including PC1 with two Tx with PC1.5 PAs). If we understand that some scenarios like NTN, indoor CPE and Automotive may see little benefit from multiple Tx without specific work in RAN1, we believe that a new PC1.5 1Tx PA and/or filter implementation may suffer from the same issue as the existing 1Tx PC1 case in terms of both filter and PA low cost component availability. If PA and filter technologies exist to achieve 29dBm power handling, it is unlikely to correspond to the high-volume technology or products developed for smartphones.

In our view RAN4 has not yet exhausted all options for increased power out of a PC2 1Tx PA including:

- Power boosting options: we have showed that that inner 3rd allocations which do not have IMD3 issue OOB can be further boosted compared to the current inner half allocations which can address some of the coverage aspects.

- Reduced MPR options and/or power increase with relaxed in-band and/or out-of-band requirements.

Based on the above observations, we believe it is important to streamline the HPUE options for Release 20 to limit the amount of work (needed to allow time for 6G study and spectrum related work) and insure availability of high volume/low-cost components. Below are a few principles we propose to achieve this.

5GA Proposal 1: For Release 20 HPUE work should focus on combinations of existing PA implementations (23dBm/30dB ACLR and 26dBm/31dB ACLR) and enabling all power boosting options.

5GA Proposal 2: For PC1 based on 8x23dBm or 4x26dBm PAs, study the viability of up to 34dB ACLR to enable the reuse of current PC3 and PC2 PA implementations.

5GA Proposal 3: As an alternative to 1Tx PC1.5, study all the options for PC2 power boosting and apply to its 1/2/3/4/8Tx combinations.

5GA Proposal 4: Compared to R19 where TN and NTN HPUE work has been separated, given the little time allowable for 5GA in Release 20 it is important to enable synergies between TN and NTN work as the same experts are often involved. Where possible synergies between the 5GA and 6G HPUE work should be encouraged.

## 2.2 HPUE options for 6G

The list of Release 19 HPUE features in the chapter above can be taken as the starting point for 6G and some of the Release 20 5GA work could also benefit to the 6G study.

As noted above, there are already a number of cases that are specified in 5GA. However, the specification framework is very complex with:

- 5 power classes

- MPR and power boosting and reduced MPR options applying offsets to both the upper and lower bound of the power class

- A large number of inter-band cases with permutations of PC3/PC2/PC1.5 bands for 1UL or 2UL 2/3Tx and up to 3CC.

- PC3 as the default power class for two UL inter-band cases where each band is already PC3 capable.

- MPR and A-MPR studies require to explore a large set of RB allocation.

For 6G where UL performance and HPUE capability will drive a number of important UL and DL performance metrics, it is essential that the UEs are always operating at their maximum feasible RF output power across all UL configurations:

- Single band, intra-band and/or inter-band combinations

- RB allocations (possibly limiting allocation cases resulting in high power back-off)

We also should learn from our 5G experience, where Rel-20 will be the fifth release with HPUE content, and reduce the number of FR1 power classes, remove unnecessary power limitations and facilitate the definition of a power class from single UL band single CC up to two UL band with four CC.

Although we understand it is early to have a detailed discussion for power classes in 6G, it is important to propose a few principles to allow a straightforward HPUE specification framework in 6G.

6G Proposal 1: For 6G, define PC2 (26dBm) power class as the default per band power class for the 0.4-15GHz range. Number of supported Tx is FFS depending on the frequency. Power classes are jointly defined for TN and NTN including sensing. Power class definitions should target flat MPR up to 200MHz for frequencies > 2.5GHz and 400MHz above 5GHz.

6G Proposal 2: For 6G, study power class requirements to avoid unnecessary output power limitation. For example:

- Two UL inter-band power class always allows each band to reach its maximum power

- The UE is always allowed to boost some allocations (lower PAPR, In-band limitation only, improved linearity/impairments)

- Upper bound of the power class defined as the next power class up (overlapping power classes) to enable boosting, combining full power of available PAs in one band or two band.

6G Proposal 3: For 6G, study whether allocations resulting in large power back-off (MPR) should be avoided.

## 2.3 Further specification simplifications

For 5GA

In Release 19 we have put significant effort into providing the technical background for the HPUE MSD simplification for 1UL and 2UL HPUE. We know that some companies would also like to simplify the PC3 baseline MSD and, why not, REFSENS. We will support the effort, However, the scope and aspects to be simplified should be carefully chosen and possibly cover some R19 leftovers.

5GA Proposal 5: If decided, MSD/REFSENS simplification work in release 19 should be small and well scoped.

For 6G

In release 5GA we are struggling with the specification structure and size, which prevents users but also delegates from having a good picture of a given feature as requirements are spread across many clauses which impacts the quality and the completeness of the specification.

This aspect is also emphasized by the fact that there is no UE type defined while we have many requirements tailored to Vehicles, FWA, CPE, RedCap, etc

6G Proposal 4: 6G specification should consider features to be self-contained in a clause and introduce UE types to avoid ambiguity for the implementation.

# Conclusion

In this contribution, we provide our views on the Release 20 RAN4 work and although we have formal proposals, our goal is essentially to share them with the group to better prepare the upcoming RAN plenary discussions.

5GA Proposal 1: For Release 20 HPUE work should focus on combinations of existing PA implementations (23dBm/30dB ACLR and 26dBm/31dB ACLR) and enabling all power boosting options.

5GA Proposal 2: For PC1 based on 8x23dBm or 4x26dBm PAs, study the viability of up to 34dB ACLR to enable the reuse of current PC3 and PC2 PA implementations.

5GA Proposal 3: As an alternative to 1Tx PC1.5, study all the options for PC2 power boosting and apply to its 1/2/3/4/8Tx combinations.

5GA Proposal 4: Compared to R19 where TN and NTN HPUE work has been separated, given the little time allowable for 5GA in Release 20 it is important to enable synergies between TN and NTN work as the same experts are often involved. Where possible, synergies between the 5GA and 6G HPUE work should be encouraged.

5GA Proposal 5: If decided, MSD/REFSENS simplification work in release 19 should be small and well scoped.

6G Proposal 1: For 6G, define PC2 (26dBm) power class as the default per band power class for the 0.4-15GHz range. Number of supported Tx is FFS depending on the frequency. Power classes are jointly defined for TN and NTN including sensing. Power class definitions should target flat MPR up to 200MHz for frequencies > 2.5GHz and 400MHz above 5GHz.

6G Proposal 2: For 6G, study power class requirements to avoid unnecessary output power limitation. For example:

- Two UL inter-band power class always allows each band to reach its maximum power

- The UE is always allowed to boost some allocations (lower PAPR, In-band limitation only, improved linearity/impairments)

- Upper bound of the power class defined as the next power class up (overlapping power classes) to enable boosting, combining full power of available PAs in one band or two band.

6G Proposal 3: For 6G, study whether allocations resulting in large power back-off (MPR) should be avoided.

6G Proposal 4: 6G specification should consider features to be self-contained in a clause and introduce UE types to avoid ambiguity for the implementation.

# References

[1] RP-251323 On HPUE for 5G-Advanced in Rel-20, Skyworks Solutions Inc., RAN4#108

[2] RP-251324 On HPUE for 6G, Skyworks Solutions Inc., RP4#108